3GPP TSG-RAN WG2 Meeting #127bis		R2-2408384

Hefei, China, Oct 14th – 18th, 2024

Source:	Baicells

Title:		Discussion on LTM measurement reporting configuration

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

As is defined in the WID for R19 LTM mobility enhancements at RAN#102 meeting, the event triggered L1 measurement reporting is needed to be supported in LTM.

On the objective of LTM event triggered L1 measurement enhancement in R19 mobility WI, RAN2 made the following progress in the RAN2#125bis[1], RAN2#126[2] and RAN2#127[3]:

RAN2#125bis Agreements on measurements: 1. L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

RAN2#126 Agreements on measurements: 1. Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. 2. For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighboring cell for event LTM3 and event LTM5. 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

RAN2#127 Agreements on measurements: 1. Event LTM1 is not defined. 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation. 4. Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. 5. R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering. 7. Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this contribution, we will discuss the remaining open issues and further details of event triggered L1 measurement reporting configuration, including:

-	Whether to report when leaving condition is met;

-	How to configure the event triggered L1 measurement reporting for LTM.

- Discussion

  - Report on leave

In legacy L3 event-triggered measurement reporting, if the parameter reportOnLeave is set to true, then the measurement reporting procedure will be initiated by the UE when the leaving condition is met for a cell which was reported previously. In this way, the NW can timely know when a cell is not good enough anymore, then the NW can exclude this cell when determines a target cell for handover in legacy L3 event-triggered measurement reporting. Without report set by reportOnLeave, the network is unaware of the any change in cell quality, causing the NW to determine a wrong target cell. In LTM, any beam in candidate RS configuration can be used for LTM event evaluation. Therefore, it is also crucial for the network to get the updated beam quality when the previously reported beam has met the leaving condition in LTM.

Observation 1: Without reporting on leave, the NW may determine a wrong target cell.

Therefore, we propose:

Proposal 1: UE can be configured by the gNB to initiate a measurement report when the leaving condition is met.

  - Event triggered L1 measurement reporting configuration

According to the L3 measurement event, the report quantity can be RSRP/RSRQ/SINR of L3 measurement results. From our understanding, RAN2 can assume that L1-RSRP/L1-RSRQ/L1-SINR can also be used for the beam measurement result reporting for the LTM measurement event.

Observation 2: The measurement quantity (e.g., RSRP, RSRQ, SINR) can also be used to represent the link quality of the measured beams in LTM.

Proposal 2: The NW can configure UE to report the following quantities:

-	L1-RSRP;

-	L1-RSRQ;

-	L1-SINR;

We understand that one measurement resource may be configured with multiple events e.g. Event LTM2, Event LTM3 and Event LTM5 are configured to the same measurement resource. If we reuse LTM-CSI-ReportConfig for the event triggered measurement report configuration, there are two options to configure the measurement report configurations. The two options are as follow:

Option1: One report configuration (LTM-CSI-ReportConfig) includes one event configuration.

Option2: One report configuration (LTM-CSI-ReportConfig) includes multiple event configurations.

NW needs to configure the LTM-report-quantity, LTM-report-content and LMT-CSI-ResourceConfigID for each event. For the above two options, Option1 needs multiple report configurations to configure multiple events for one measurement resource. If the LTM-report-quantity, LTM-report-content and LMT-CSI-ResourceConfigID configurations are the same for each event, then these IEs are transmitted multiple times in multiple report configurations causing signaling overhead. Even if the configuration of LTM-report-quantity and LTM-report-content can be different for different events, at least the LMT-CSI-ResourceConfigID is retransmitted in different events for the same measurement resource. Option2 only needs one measurement report configuration, and the above IEs may need to be configured once in the report configuration for one measurement resource. The following table compares the two options.

Table I   The comparison between the two options

|  | Option1：Multiple report configurations for multiple events (each configuration for one event) | Option2：One report configuration containing multiple events |
| --- | --- | --- |
| Signaling overhead for Measurement report configuration | The following things are configured by gNB to UE in the LTM-CSI-ReportConfig: LMT-CSI-ReportConfigId1, LMT-CSI-ResourceConfigId, LTM-EVENT::={Event LTM2}, LTM-report-quantity ::= SEQUENCE { RSRP, RSRQ, SINR, }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R }, LMT-CSI-ReportConfigId2, LMT-CSI-ResourceConfigId, LTM-EVENT::={LTM3}, LTM-report-quantity ::= SEQUENCE { RSRP, RSRQ, SINR, }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R }, | The following things are configured by gNB to UE in the LTM-CSI-ReportConfig: LMT-CSI-ReportConfigId1, LMT-CSI-ResourceConfigId, LTM-EVENT::= SEQUENCE {LTM2, LTM3}, LTM-report-quantity ::= SEQUENCE { RSRP , RSRQ , SINR , }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R }, |
|  | The following things are configured by gNB to UE in the LTM-CSI-ReportConfig: LMT-CSI-ReportConfigId1, LMT-CSI-ResourceConfigId, LTM-EVENT::={LTM2}, LTM-report-quantity ::= SEQUENCE { RSRP, RSRQ, SINR, }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R }, LMT-CSI-ReportConfigId2, LMT-CSI-ResourceConfigId, LTM-EVENT::={LTM3}, LTM-report-quantity ::= SEQUENCE { RSRP, RSRQ, }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED { n1,n2,n3, }, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3, }, spCellInclusion ENUMERATED {false} OPTIONAL -- Need R }, | The following things are configured by gNB to UE in the LTM-CSI-ReportConfig: LMT-CSI-ReportConfigId, LMT-CSI-ResourceConfigId, LTM-EVENT::= SEQUENCE {LTM2, LTM-report-quantity ::= SEQUENCE { RSRP , RSRQ , SINR , }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R } }, LTM-EVENT ::= SEQUENCE {LTM3, LTM-report-quantity ::= SEQUENCE { RSRP , RSRQ , }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,}, spCellInclusion ENUMERATED {false} OPTIONAL -- Need R } }, |

Observation 3: When multiple events are configured for one measurement resource simultaneously, there are two options to indicate the triggered event, corresponding to two measurement report configurations.

Option1: One report configuration (LTM-CSI-ReportConfig) includes one event configuration.

Option2: One report configuration (LTM-CSI-ReportConfig) includes multiple event configurations.

Observation 4: The Option1 retransmits at least the LMT-CSI-ResourceConfigID for multiple events for one measurement resource. The LTM-report-quantity and LTM-report-content are retransmitted when they are configured same for the multiple events for the same measurement resource.

Observation 5: The Option2 uses one report configuration and at least the LMT-CSI-ResourceConfigID only need to be configured once in the report configuration for one measurement resource.

From the above observations, Option2 reduces the signaling overhead in the measurement reporting configuration from gNB to UE and we prefer Option2.

Therefore, we propose:

Proposal 3: One LTM CSI reporting configuration contains multiple events for the LTM CSI reporting.

- Conclusion

In this contribution, we discussed the issues related to LTM measurement event evaluation. We have the following Observations and Proposals:

Observation 1: Without reporting on leave, the NW may determine a wrong target cell.

Observation 2: The measurement quantity (e.g., RSRP, RSRQ, SINR) can also be used to represent the link quality of the measured beams in LTM.

Observation 3: When multiple events are configured for one measurement resource simultaneously, there are two options to indicate the triggered event, corresponding to two measurement report configurations.

Option1: One report configuration (LTM-CSI-ReportConfig) includes one event configuration.

Option2: One report configuration (LTM-CSI-ReportConfig) includes multiple event configurations.

Observation 4: The Option1 retransmits at least the LMT-CSI-ResourceConfigID for multiple events for one measurement resource. The LTM-report-quantity and LTM-report-content are retransmitted when they are configured same for the multiple events for the same measurement resource.

Observation 5: The Option2 uses one report configuration and at least the LMT-CSI-ResourceConfigID only need to be configured once in the report configuration for one measurement resource.

Proposal 1: UE can be configured by the gNB to initiate a measurement report when the leaving condition is met.

Proposal 2: The NW can configure UE to report the following quantities:

-	L1-RSRP;

-	L1-RSRQ;

-	L1-SINR;

Proposal 3: One LTM CSI reporting configuration contains multiple events for the LTM CSI reporting.

- References

- RAN2 #125bis chairman notes;

- RAN2 #126 chairman notes;

- RAN2 #127 chairman notes;