3GPP TSG-RAN WG2 #127bis 	R2-2408688

Hefei, China, 14th – 18th October 2024

Agenda Item:	8.2.4

Source:	Ericsson

Title:	UL multiple access for Ambient IoT

Document for:	Discussion/Decision

# 1	Introduction

Random access (RA) is one model of UL multiple access that needs to be studied for Ambient IoT (A-IoT). RAN2 has agreed to study 4-step like RA and 2-step like RA. In addition, contention-based access procedure and contention-free access procedure are also to be discussed. We further discuss details for different procedures in this paper.

# 2	Discussion

## 2.1	RAN2 agreements

In RAN2#127, the following agreements were made related to access procedure in random access AI 8.2.5.

Agreements

-	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

Agreements

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

In addition, we use following terms.

CR ID: Contention resolution ID

AS ID: Access Stratum ID

## 2.2	Random Access Types

In the last meeting, it is agreed to have random-access for an inventory or command procedure based on two steps or four steps initiated by a network trigger message for traffic DO-DTT. These message flows are tentatively described in below Figure 1.

Figure 1: An example on (a) 4-step contention-based RA (left side, agreed in last meeting), (b) 2-step contention-based RA (top right side, agreed in last meeting) and (b) 2-step contention-free RA (bottom right side).

One aspect of RA configuration is selection or the choice between 2-step and 4-step RA. In our view, the selection of RA type for the device to perform should be left to Reader. Otherwise, this can lead to extreme blind decoding at the Reader not knowing if the incoming initial RA message is Msg1 or MsgA. Another rationale is that 2-step and 4-step RA procedures’ collisions success depend on the inventory load size which Reader may be aware of (if CN divulges the load/group size information) but not the devices itself as their transmission transparent to each other. However, for the purpose of study we are fine with study both access mechanisms, i.e., device autonomously select the RA method, or the reader/gNB indicates the access method.

it is feasible to study the below different scenarios for random access:

Scenario 1: A-IoT 3-step contention-based random access

Scenario 2: A-IoT 2-step contention-based random access

Scenario 3: A-IoT 2-step contention-free random access

The 4-step contention-free random access is not needed since device doesn’t need to perform contention resolution before the transmission of the data. Device can always send its ID and data in the same message, which means only 2-step CFRA is needed.

Reader decides the type of random access that devices shall apply (i.e., 3 step CBRA, 2 step CBRA and 2 step CFRA).

As discussed above, the suitable behaviour is if gNB can decide RA type. Given, gNB will always determine the suitable RA type, and device will not autonomously select RA type. Therefore, device autonomously select RA type is not pursued.

Device autonomously to determine random access type is not pursued.

## 2.3	Feedback for D2R Message failure/success

In A-IoT, we have primarily two use cases: Inventory and command use case. Another hybrid use case is derived by combining both, i.e., inventory plus command use case. The fundamental difference in all these use case types would be if there is and upper response coming to CN to device (in command or inventory plus command use case). In such scenarios indicating separate AS feedback is unnecessary as device will expect upper layer feedback after transmitting Msg3 or device ID in uplink.

In 3-step contention-based RA, the use case with inventory followed by commands (read request, write command, security related negotiations), there will be an upper layer message to be delivered to the device after D2R message, i.e., after Msg3.

In 3-step contention-based RA, the R2D message after Msg3 can act as AS feedback implicitly indicating failure/success of Msg3 transmission.

On the other hand, with inventory use case where no upper layer response is expected from CN then gNB can consider providing AS feedback or not subject to local conditions and reliability or QoS constraints, which is in RAN1 domain. The gNB may not consider to feedback if the transmissions are already being reliable (if it is a requirement). RAN1 already has tools in hand by which transmission can be made reliable, for instance, configuring repetitions for transmissions, or in case passive device, network can transmit CW with increased power, etc.

Note, if a device expects security related authentication negotiations during inventory procedure, then in our view, this is equivalent to command use case as device will receive some requests from CN and the device may need to respond accordingly where such negotiations may be transparent to the reader.

In 3-step contention-based RA, the use case with inventory only, there may be no upper layer message to be delivered to the device after D2R message, i.e., no message after Msg3.

In 3-step contention-based RA, if the reliability of Msg3 transmission can be assumed similar to Msg1 transmission reliability. In that case, if Msg1 transmission is successful against channel conditions, then Msg3 transmission likely to be successful.

In 3-step contention-based RA, the RAN1 tools, such as repetition, increased CW transmission power can be utilized to support Msg3 transmission reliability.

Feedback may be needed if required collisions or channel errors are high and could fulfil required QoS or reliability targets. The main source of errors can come from collisions and changed or incoherent channel conditions, especially if there is large variation in Msg1 and Msg3 sizes and Msg1 transmission success cannot guarantee Msg3 transmission success.

In 3-step contention-based RA, if the message sizes (Msg1 size and Msg3 size) are noticeably different, then their transmission reliability against channel conditions may potentially be different.

Based on above discussion, following table summarises the AS feedback options indicating ACK and NACK.

Table 1

AS feedback options

| Feedback options for different use cases | Postive Acknowkedgement | Negative Acknowledgement |
| --- | --- | --- |
| No upper layer response expected (e.g., inventory) | Upto RAN1 - Transmit explicit AS feedback | Two options - Retransmit Msg2 - The behavior may change if feedback is ACK based or NACK based. - No Msg2 retransmission |
| Upper layer response expected (e.g., command, or inventory with authentication requests) - Read request - Write command - Security negotiation requests | AS feedback is implicit if device gets upper layer reponse |  |

Hence subject to the need of upper layer response, the feedback behaviour or options should be studied accordingly.

For 3-step CBRA, if there is no upper layer information in R2D message to be delivered after D2R message within the defined timing window, then the need of explicit AS feedback in R2D message is subject to reliability requirement which is up to RAN1 decision.

Note. In our proposal, we assume single low power receiver and no additional wake up receiver. Hence, in a only in a selected access occasion, the UE will monitor feedback. However, the UE may monitor other signalling in other access occasions pertaining to occasion count, other inventory related adjustment signalling, etc. subject random-access design.

Similar to above discussion on 3/4-step CBRA, we list proposals for 2-step CBRA.

For 2-step CBRA, if there is no upper layer information in Msg2 to be delivered after Msg1 transmission with data within the defined timing window, then the need of explicit AS feedback in Msg2 message is subject to reliability requirement which is up to RAN1 decision.

The transmissions in contention based random-access procedure may be prone to collisions when there is high number of devices contending the channel. In addition, in general, the channel error probability for A-IoT transmissions would be higher than of typical NR transmission due to device’s limited capabilities and most likely its inability to do channel measurements. As a consequence, A-IoT transmissions can be unreliable and thus, a suitable feedback mechanism can be useful to indicate device’s UL message success or failure. However, the feedback has a cost on device energy and processing resources which are already scarce and limited.

Subject to feedback design, reliability requirements, there can be multiple ways re-access for the failed devices can be done. For instance, re-access can be done in same or next inventory/access round, therefore, first, it is important if we agree on terminologies related to paging, access, inventory round and the interactions which RAN2 has not defined properly. See below figure which depicts an interaction between paging message, access round, occasions, etc.

Figure 2: A paging message triggers an access round composed of N occasions serving group of devices. Each occasion can be used for CBRA based access. If the paging targets a single device and only single occasion is allocated in the access round, then the allocation behaviour reduces to CFRA.

During the RACH email discussion i.e., [POST127][033][AIoT] Random Access (Huawei), there are on-going discussions on the terminologies, i.e., the need and definition of terminologies including access round, paging round. In our view, it is necessary for RAN2 to first discuss these terminologies before further study the below other issues, e.g., AS feedback and re-access.

RAN2 to discuss and agree on need and definition of terminologies on access round and paging round.

An access round can be defined as a time period over which devices can randomly select resources for its UL transmissions from a resource pool in a contention based manner.

The access round with more than one occasion is allocated to serve group of devices in a CBRA manner.

The access round with one occasion is allocated to serve single device in a CFRA manner.

The reason why a device may experience contention resolution failure or access failure, is due to high congestion in the channel, which may be partly due to that reader doesn’t allocate sufficient number of access occasions in the round. In other words, if the reader can allocate sufficient number access occasions in an access round, access or transmission failures from devices can be mitigated. However, it is not always possible for a reader to determine/know the exact number of target devices for a service request especially for the requesting targeting all devices in the area. In this case, the reader may allocate a number of occasions which is not suitable for devices in an initial round. Based on the accesses in recent rounds, the reader may estimate the congestion status based on that, the reader can adjust or reallocate a new number of access occasions, which is more fitting to the actual number of target devices. This would be helpful to mitigate or decrease congestion for subsequent access rounds. If it is necessary, the reader can determine to terminate an access round earlier and start a new round with more access occasions.

A paging message indicating number of access occasions triggers an access round divided into the indicated number of occasions.

The number of access occasions for an access round can be adjusted by the reader after the access round has been started.

From device perspective, it is beneficial for a device to re-access the channel to combat access failures. In our view, the reader can trigger re-accesses for the devices via occasions in the same or the next round. Both these options have pros n cons which are described in below table.

| Re-accessing in remaining occasions same round | Re-accessing in next round |
| --- | --- |
| Pros: The device gets opportunity for re-access in same round resulting in low latency. This may help in conserving energy as device able to finish its tasks, e.g., invenrory, compared to if device is permited to re-access in next round. Cons: Ths may increase in collision probability as more failed devices try to attempt re-access in remaining occassions leading relatively high collision probability in the last or remaining access occasions comapred to the first or initial occasions in a given round. Large collision rate can result in rsources wastage network side and energy wastage at device side | Pros: The collision probabaility is low compared to the case with re-accessing in same round. The failed device will pause its attempt and wait for new round. Hence, in the existing round, only one attempt will be allowed and thus, resulting low collision rate. Cons: The failed devices need to stay awake for relatively longer time period as their re-access opportunities postponed to new round, which may increase energy use cost. This may impact network energy cost in case devices are getting charged using network’s infrastructure, e.g., via CWE node. |

It is up to reader’s implementation to trigger D2R retransmissions or re-accesses, e.g., by retransmitting Msg2/Msg0 towards devices.

A device can re-access the channel via additional access occasions allocated by reader.

RAN2 to study the pros and cons of options for providing additional access occasions for re-access and capture in TR. The options include provide re-access occasions in the same access round, or subsequent new access round(s).

On a question whether the rest competing devices also need to be signalled with an explicit acknowledgement message, we think this is not needed, since the acknowledgement message to the winners have already indicated that other competing devices (with different temporary IDs) fail to their accesses. However, in the same message, additional information can be indicated for the failed devices.

As there can be more than one competing device, where one or more devices have won the contention (depending on resource usage policies) which can be indicated in Msg2, and additionally address the devices which have failed the contention without any explicit addressing. The reasons gNB cannot identify some devices can be numerous. For example, collisions or it can be as well due to channel failure, and gNB able correctly identify the device resulting in failing to positively acknowledge the device. Hence, in such scenarios, the gNB can respond with information in Msg2 that can help device re-access the inventory round. For instance, the Msg2 can target unidentified failed devices with backoff indicator (BI) e.g., information related to allowed access in same or different or adjusted round, see following proposals discussion, or exercise halting of immediate re-access for probable failed devices in order to decrease collision probability, etc.

For devices with unsuccessful random access, RAN2 to study the response message (Msg2) indicating additional information related to back-off and re-access.

## 2.4	AS ID for CFRA

In last meeting, it was agreed that in contention-free access, the A-IoT device directly sends the upper layer data (e.g., device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2). As long as the access procedure targets single device, the AS ID is not needed. The reason is that reader can allocate resources to this single device without the need to know the AS layer identities as there will be no collision on access of resources. However, it is important, that one only device access these resources. This can be left up to CN which expects only single device to respond. Hence, CN can send polling or paging command which targets a single device. The CN may or may not reveal to gNB that command is for single device. Revealing the information pertinent to group size constituting single device may benefit (without revealing the device ID or content of the command) so that reader does not allocate resources for multiple devices.

For CFRA with one device, the reader doesn’t need to know any form of the device ID for resource allocation as there is no ambiguity between the reader and the device regarding the allocated resources.

For CFRA with one device, no AS ID is needed for scheduling resources for subsequent transmissions.

If there are large number of devices requiring access which are known to the gNB/intermediate UE, then RAN2 can further study if it may be feasible for the gNB/intermediate UE to provide dedicated resources to multiple known devices in a scheduling round.

However, for the case with CFRA with multiple devices, the need of AS IDs should be discussed after on agreeing to study such use case. Currently, CFRA with single device has been agreed.

RAN2 to first discuss on CFRA with multiple devices before discussing AS IDs need for multiple devices’ CFRA based resource allocation.

RAN2 to include CFRA with multiple devices in the study and TR.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In 3-step contention-based RA, the use case with inventory followed by commands (read request, write command, security related negotiations), there will be an upper layer message to be delivered to the device after D2R message, i.e., after Msg3.

Observation 2	In 3-step contention-based RA, the R2D message after Msg3 can act as AS feedback implicitly indicating failure/success of Msg3 transmission.

Observation 3	In 3-step contention-based RA, the use case with inventory only, there may be no upper layer message to be delivered to the device after D2R message, i.e., no message after Msg3.

Observation 4	In 3-step contention-based RA, if the reliability of Msg3 transmission can be assumed similar to Msg1 transmission reliability. In that case, if Msg1 transmission is successful against channel conditions, then Msg3 transmission likely to be successful.

Observation 5	In 3-step contention-based RA, the RAN1 tools, such as repetition, increased CW transmission power can be utilized to support Msg3 transmission reliability.

Observation 6	In 3-step contention-based RA, if the message sizes (Msg1 size and Msg3 size) are noticeably different, then their transmission reliability against channel conditions may potentially be different.

Observation 7	An access round can be defined as a time period over which devices can randomly select resources for its UL transmissions from a resource pool in a contention based manner.

Observation 8	The access round with more than one occasion is allocated to serve group of devices in a CBRA manner.

Observation 9	The access round with one occasion is allocated to serve single device in a CFRA manner.

Observation 10	A paging message indicating number of access occasions triggers an access round divided into the indicated number of occasions.

Observation 11	For CFRA with one device, the reader doesn’t need to know any form of the device ID for resource allocation as there is no ambiguity between the reader and the device regarding the allocated resources.

Observation 12	RAN2 to first discuss on CFRA with multiple devices before discussing AS IDs need for multiple devices’ CFRA based resource allocation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reader decides the type of random access that devices shall apply (i.e., 3 step CBRA, 2 step CBRA and 2 step CFRA).

Proposal 2	Device autonomously to determine random access type is not pursued.

Proposal 3	For 3-step CBRA, if there is no upper layer information in R2D message to be delivered after D2R message within the defined timing window, then the need of explicit AS feedback in R2D message is subject to reliability requirement which is up to RAN1 decision.

Proposal 4	For 2-step CBRA, if there is no upper layer information in Msg2 to be delivered after Msg1 transmission with data within the defined timing window, then the need of explicit AS feedback in Msg2 message is subject to reliability requirement which is up to RAN1 decision.

Proposal 5	RAN2 to discuss and agree on need and definition of terminologies on access round and paging round.

Proposal 6	The number of access occasions for an access round can be adjusted by the reader after the access round has been started.

Proposal 7	It is up to reader’s implementation to trigger D2R retransmissions or re-accesses, e.g., by retransmitting Msg2/Msg0 towards devices.

Proposal 8	A device can re-access the channel via additional access occasions allocated by reader.

Proposal 9	RAN2 to study the pros and cons of options for providing additional access occasions for re-access and capture in TR. The options include provide re-access occasions in the same access round, or subsequent new access round(s).

Proposal 10	For devices with unsuccessful random access, RAN2 to study the response message (Msg2) indicating additional information related to back-off and re-access.

Proposal 11	For CFRA with one device, no AS ID is needed for scheduling resources for subsequent transmissions.

Proposal 12	RAN2 to include CFRA with multiple devices in the study and TR.

# 4	References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, August 8-19, 2024