3GPP TSG-RAN WG2 Meeting #95	R2-164974

Goteborg, Sweden, August 22 – August 26, 2016

# Agenda Item: 7.5

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 0899 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Logical Channel Prioritization in Multiple SL Transmissions: Option2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eD2D_Prox-Core |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - Multiple SL grants in one SC period to one ProSe Destination should not be allowed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - When performing logical channel prioritization procedure for each SCI transmitted in an SC period, the UE shall select a ProSe Destination not previously selected for this SC period. Impact analysis Impacted functionality: Sidelink Logical Channel Prioritisation for multiple SL transmissions Inter-operability: If the Network supports this change and the UE does not there are no interoperability problems. If the UE supports this change and the Network does not there are no interoperability problems. If the transmitting UE supports this change and the receiving UE does not there are no interoperability problems. If the receiving UE supports this change and the transmitting UE does not, the receiving UE may not receive data belonging to a ProSe Destination having logical channels with lower priority. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data in ProSe Destination having logical channels with lower priority may not get a chance to be transmitted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.14.1.3.1	Logical channel prioritization

The Logical Channel Prioritization procedure is applied when a new transmission is performed. Each sidelink logical channel has an associated priority which is the PPPP. Multiple sidelink logical channels may have the same associated priority. The mapping between priority and LCID is left for UE implementation.

The MAC entity shall perform the following Logical Channel Prioritization procedure for each SCI transmitted in an SC period:

The MAC entity shall perform the following Logical Channel Prioritization procedure for each SCI transmitted in an SC period:

-	The MAC entity shall allocate resources to the sidelink logical channels in the following steps:

-	Only consider sidelink logical channels not previously selected for this SC period and the SC periods (if any) which are overlapping with this SC period, to have data available for transmission.

-	Step 0: Select a ProSe Destination, not previously selected for this SC period, having the sidelink logical channel with the highest priority, among the sidelink logical channels having data available for transmission;