3GPP TSG-RAN WG2 #96	Tdoc R2-168016

# Reno, US, 14th – 18th Nov. 2016

Agenda Item:	9.3.1.1.1

Source:	ASTRI, TCL Communication Ltd.

Title:	Inter-Virtual Cell Handover in Two-Layer NR Networks

Document for:	Discussion, Decision

# Introduction

One of the objectives of the new study item [SI: FS_NR_newRAT] approved in RAN #71 is to study the mobility in NR [1]. In RAN2 #95bis meeting, the following agreements for DL-based mobility in RRC_CONNECTED mode have been reached [2]:

Agreements for DL-based mobility in RRC_CONNECTED mode (optimized for data transmission, at least for network-controlled mobility) - UE at least measures one or more individual beams and gNB should have mechanisms to consider those beams to perform HO. Note: This is necessary at least to trigger inter-gNB handovers and to optimize HO ping-pongs / HO failures. - FFS: whether UE report individual and/or combined quality of multiple beams. - UE should be able to distinguish between the beams from its serving cell and beams from non-serving cells for RRM measurements in active mobility. UE should be able to determine if a beam is from its serving cell. - FFS whether serving/non-serving cell may be termed 'serving/non-serving set of beam' - FFS: whether the UE is informed via dedicated signalling or implicitly detected by the UE based on some broadcast signals. - FFS how the cell in connected relates to the cell in idle - Study how to derive a cell quality based on measurements from individual beams. - In connected mode, intra-cell mobility can be handled by mobility without RRC involvement. - FFS whether there may be cases that do require RRC involvement. - UE should be able to identify a beam. FFS how beams are identified (to be defined by RAN1)

In this contribution, key principles for handover during network controlled mobility in NR are discussed. For simplicity, we assume each TRP has one beam in the following discussion, where the obtained proposals can also be easily extended to the case of each TRP with multiple beams.

# Discussion

## Two-Layer NR Architecture

According to the objectives of the SI [1], the RAN architecture may be split into ‘central units’ (CUs) and ‘distributed units’ (DUs). As such, the network operates as two layers, namely the CU layer and the DU layer. The CU layer consists of all the CUs, which corresponds to virtual cells (VCs), while the DU layer is composed by all the DUs, which are also known as transmission points (TPs). One of the benefits to have this two-layer architecture is to provide high data throughput as well as minimize the handover (HO) overhead.

Fig. 1. Two-Layer NR Architecture and Intra-NR Mobility Scenarios

One way to achieve such two-layer network is to let all the TPs within a VC transmit the common control channels, e.g., PSS, SSS, BCH and CRS in LTE standard, at the same time/frequency resources to provide a wide coverage. Fig. 1 illustrates the CU/DU splitting two-layer NR RAN architecture and the possible intra-NR mobility scenarios within such networks.

- Scenario 1: Inter-DU mobility within the same CU

- Scenario 2: Inter-CU mobility

As a result, when a UE moves around within a VC (Scenario 1), there is no need to conduct HO, while the data services are provided by one or more nearest TPs so a higher data throughput can be achieved. Although HO within a VC is thus avoided, HO still occurs when the UE moves cross VC coverages.

There are some open issues for inter-VC HO:

1)	As a UE moves between VCs, the UE needs to be HO to the new VC, and it also needs to physically attach to a new TP in the target VC. A 2-layer measurement and decision needs to be done.

2)	Given a high density of TPs, it will be very challenging for a UE to measure all of the neighbouring VCs as well as TPs, and thus different mechanisms of measurement are needed.

3)	TPs may be turned ON or OFF according to UE attachment conditions, traffic tidal effects to save energy as well as to minimize interference. Measurement for the OFF TPs during HO is an open issue.

Therefore, NR needs to standardize HO procedure for inter-VC that covers the above issues.

NR RAN network is a two-layer architecture. Two intra-NR mobility scenarios exist in such network, as shown above.

- HO is avoided in Scenario 1, whereas HO is inevitable in Scenario 2.

In Scenario 2, a two-layer measurement and decision needs to be investigated and standardized, including HO to VC, and physically attach to a new TP.

A UE needs to measure all of the neighbouring VCs as well as TPs in Scenario 2.

TPs may be turned ON or OFF according to UE attachment, to save energy and minimize interference. Measurement for the OFF TPs should be done during HO.

## Inter-Virtual Cell Handover in Two-Layer NR Network

As stated above, HO is avoided for mobility within a VC. All the TPs in a VC share a unique cell identity and a unique VC control channel. When a UE in connected status moves across the VCs, inter-VC HO is needed, during which not only the VC needs to be chosen but also a specific TP in the target VC needs to be identified to provide the data service to the UE, as shown in Fig. 2. In order to make decision on target VC and TP, measurement configurations and related signalling are needed. Specifically, possible technical method for HO should include the following components:

- Measuring. The UE needs to obtain measurement results by measuring candidate VCs and candidate TPs wherein each VC includes multiple TPs that share a common cell identity.

- Selecting. The UE needs to select a target VC and a target TP from the candidate VCs and candidate TPs according to the measurement results.

- Accessing. The UE accesses the target VC as well as the target TP.

In order to determine a technical method for inter-VC HO, three components, which are measuring, selecting, and accessing, should be studied.

- Fig. 2. Inter-VC HO Scenario

Since a VC is composed of densely deployed TPs, depending on which edge-TP is currently serving the UE at the HO triggering moment, the set of neighbouring VCs to be measured can be different. For efficient operations, mechanisms are needed such that the measurement object configuration for neighbouring VCs is sent to the UE only when 1) the serving TP of the UE is changed to an edge-TP or 2) the UE is moving between edge-TPs.

For inter-VC HO, the VC-level measurement should be triggered by the connection to edge TPs.

A TP must be chosen as the HO target. An extra layer of measurement other than the VC layer (e.g., cell identity based) is beneficial because only the TPs in the chosen VC needs to be measured so as to avoid unnecessary measurements.

Separated TP-level measurement configuration should be studied for inter-VC HO.

A UE only needs to measure the TPs in the target VC for TP selection, so an edge-TP list in the target VC needs to be known by the source VC and UE. Moreover, TPs could be turned ON/OFF from time to time. For an OFF TP, it is necessary to inform the source VC and UE about when to measure the OFF TPs, e.g., discovery reference signals’ timing information. It is also needed to study the suitability of LTE-like measurement and HO procedures for new NR scenarios including high order beam-forming and high frequency bands.

Inter-VC information exchange on TP list and status should be exploited for inter-VC HO.

# Conclusion

The following has been observed:

- NR RAN network is a two-layer architecture. Two intra-NR mobility scenarios exist in such network, as shown above.

- HO is avoided in Scenario 1, whereas HO is inevitable in Scenario 2.

In Scenario 2, a two-layer measurement and decision needs to be investigated and standardized, including HO to VC, and physically attach to a new TP.

A UE needs to measure all of the neighbouring VCs as well as TPs in Scenario 2.

TPs may be turned ON or OFF according to UE attachment, to save energy and minimize interference. Measurement for the OFF TPs should be done during HO.

The following has been proposed:

Proposal 1	In order to determine a technical method for inter-VC HO, three components, which are measuring, selecting, and accessing, should be studied.

Proposal 2	For inter-VC HO, the VC-level measurement should be triggered by the edge TP.

Proposal 3	Separated TP-level measurement configuration should be studied for inter-VC HO.

Proposal 4	Inter-VC information exchange on TP list and status should be exploited for inter-VC HO.

# References

RP-160671, New SID Proposal: Study on New Radio Access Technology, RAN #71, 7-10. March, 2016.

R2-167285, “Summary of RAN2#95bis offline discussions on NR Cell definition and relation to beams in Connected mode”, Ericsson, RAN2 #95bis, 10-14 Oct. 2016.