3GPP TSG-RAN WG2 Meeting #98	R2-1706207

Hangzhou, China, 15th – 19th May 2017

Agenda item:	9.4.4

Source: 	Qualcomm Incorporated

Title: 	Email discussion report [98#49]: Common RAN2 parameters for RLF and HOF simulations

Document for:     Information

# Introduction

RAN#75 approved a study item on Enhanced Support for Aerial Vehicles [1], also known as UAVs (Unmanned Aerial Vehicles) or Drones.

One of the objectives of the study is [1]:

- Handover: Identify if enhancements in terms of cell selection and handover efficiency as well as robustness in handover signalling can be achieved. [RAN2, RAN1]

In RAN2#98, early results for HOF and RLF performance evaluations were presented by some companies [2] [3]. RAN2 understood that common parameters are required for fair comparison and conclusion of the performance results. Therefore, the following email discussion was setup with the objective of finalizing common RAN2 parameters for RLF and HOC simulations, which will be used for further performance evaluations.

[98#49][LTE/UAV] Common RAN2 parameters for RLF and HOF simulation (Qualcomm)

-	Based on the simulation assumption in R2-1704155 and RAN1 agreements

-	Figure out the additional parameters needed by RAN2

Intended outcome: Agreeable Report

Deadline:  Thursday 2017-06-01

Companies are encouraged to provide their inputs in the following sections before the deadline indicated above.

# RLF and HOF simulation parameters

Editor’s Note: The discussion in this section may be a baseline for TP to Section 4.1 in TR 36.777.

As stated in the scope of email discussion above, PHY parameters will be based on RAN1 agreements. Therefore, we focus here only on RAN2 parameters.

## Channel models and LOS/NLOS probability

RAN1 is currently having an email discussion to finalize Drone channel models and LOS/NLOS probability. RAN2 will reuse the channel models and LOS/NLOS probability defined by RAN1.

In RAN1 simulations, once a UE is “dropped”, the channel characteristics of the UE (path loss, shadowing, height, LOS/NLOS etc.) does not change throughout the simulation. Following additional modeling may be relevant for RAN2 in addition to channel models defined by RAN1.

### LOS/NLOS switching

RAN1 will define channel models for LOS and NLOS, as well as probability of LOS/NLOS. However, RAN1 does not consider “switching” a single UE between LOS-NLOS. In RAN1 simulations, once a UE performs a coin-flipping to decide LOS/NLOS based on the defined probability formula, it stays that way throughout the simulation.

Companies are invited to provided their comments on modelling of LOS/NLOS switching below:

Table 1 LOS/NLOS switching

| Company | Comment |
| --- | --- |
| Qualcomm | To keep the simulation complexity low and to make comparison of simulation results across multiple companies reasonable, our opinion is that such switching of LOS/NLOS for the same UE is not needed (i.e., follow RAN1: use the LOS/NLOS probability to determine whether the UE is LOS or NLOS UE. This means, for the same height, there will be some UEs at LOS and some others at NLOS). However, if majority of the companies think such switching for the same UE is required, then RAN2 will need to decide on the switching model. (As already explained above, the channel models for LOS/NLOS and the probability of LOS/NLOS come from RAN1, but switching does not.) One example for such switching model would be: - Consider a LOS/NLOS correlation distance [X]. X can be taken from TR 36.839 Table 5.2.3.1. - Based on the speed S of the UE, the coin-flipping for LOS/NLOS is performed after interval t = X/S. |

Summary: As baseline, following the RAN1 modelling, UEs use fixed LOS or NLOS based on initial coin-flipping. FFS whether and how to model LOS/NLOS switching.

### Time varying shadowing for moving UE

In RAN1 simulations, UEs do not move, however for RAN2, time varying shadowing for moving UE should be modelled. It can be based on standard deviation of the shadowing (from RAN1) and correlation distance (TR 36.839 Table 5.2.3.1), i.e., shadowing value is sampled after interval ‘t’ based on correlation distance and speed.

Companies are encouraged to comment below if they have different opinion/proposal.

Table 2 Shadowing

| Company | Comment |
| --- | --- |

Summary:

Time varying shadowing for a moving UE is modeled by recalculating shadowing value based on shadowing standard deviation (as defined by RAN1) after UE has travelled distance of 25 m (based on its speed).

## RAN2 parameters

For the mobility study, the baseline assumption is based on R2-1704155.

Table 3 describes the parameters to be used to model the handover for these simulations. One column is prefilled with values from R2-1704155. Provide your company name and preferred values in additional columns if they are different than the proposed value.

Table 3 Proposed values of parameters to model the handover performance

| Parameter | Description | Unit | Value |  |  |  |  |  | Summary Notes: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Qualc omm | Intel | Ericsson | Huawei | Nokia | KDDI |  |
| TimeToTrigger | Time to trigger a measurement report | ms | 160 | 160 | 40, 80, 160 | 160 * SF (Considering TTT need to be multiplied with scaling factor depending on UE speed) | Typical value of 160 should be the starting point, but shorter (e.g. 80 ms can be also considered) | 40, 256 | 160 |
| A3Offset | Offset between signal strength of serving and neighboring cells | dB | 2.0 | 2 | 2, 3 | 2.0 | 2.0 | 1.5, 3 | 2 |
| MeasurementInterval | Physical layer measurement interval | ms | 40 | 10 | 40ms and 10ms after T310 started | 40 | 10 (more suitable for high speed UAVs) |  | 10 ms |
| SamplesToAverage | Number of physical layer measurements averaged |  | 5 | 20 |  | 5 | 20 |  | based on meas interval and meas period |
| L3RRMCoefficient | Filtering coefficient for layer 3 measurements |  | 1 | 1 | 1, 4 | 1 | 1 |  | 1 |
| RLMInterval | Radio link monitoring interval | s | 2 | L1 samples filtered linearly over a sliding window of 200ms (i.e. 20 samples) for Qout and 100 ms (i.e. 10 samples) for Qin (this is same as TS 36.839) |  | L1 samples filtered linearly over a sliding window of 200ms (i.e. 20 samples) for Qout and 100 ms (i.e. 10 samples) for Qin (this is same as TS 36.839) | As suggested by Intel. |  | L1 samples filtered linearly over a sliding window of 200ms for Qout and 100 ms for Qin (same as TR 36.839) |
| T310 | Timer to trigger radio link failure recovery | ms | 1000 | 1000 | 1000 N311 1 N310 1 | 1000 | 1000 is OK | 1000 | 1000 |
| HOPreparationDelay | Handover preparation delay | ms | 50 | 50 | 50 | 50 | 50 |  | 50 |
| HOExecutionDelay | Handover execution delay | ms | 40 | 40 | 40 | 40 | 40 |  | 40 |
| RSRPError | Standard deviation of RSRP measurement error | dB | 1.22 | 1.22 | 1.22 | 1.22 | 1.22 |  | 1.22 |

Companies may also add notes below if they want to justify their choice of values.

Table 4 Additional comments on parameters

| Company | Comment |
| --- | --- |
| Intel | Below parameters should also be added to the table - Qin threshold = -6dB - Qout threshold = -8dB - TMeasurement_Period, Intra, L1 filtering time in TS36.133 = 200ms |
| Ericsson | TTT, A3offset, L3RRMCoefficient: More than one value should be simulated. We propose few default values according to TS 36.839 while additional values not precluded After August meeting we can see if more further evaluations are needed. T310: N310 and N311 need to be set and given when presenting results. Other values than 1 can be used additionally. Additionally, we should fix one set of values for ground UEs. Proposal: speed 0km/h, TTT 160, A3offset 3dB, k=1 |
| Huawei | RLM interval Parameters should be added in the table: the Qout evaluation period (TEvaluate_Qout) is 200 ms; the Qin evaluation period (TEvaluate_Qin) is 100 ms; (abstracted from TS 36.133) |
| KDDI | TTT: Having simulations on some values is preferable. |

Summary:

Following values are agreed as baseline for mobility simulations. Other values are not precluded if companies are interested to study and submit more results.

| Parameter | Description | Agreed value(s) |
| --- | --- | --- |
| TimeToTrigger | Time to trigger a measurement report | 160 ms |
| A3Offset | Offset between signal strength of serving and neighboring cells | 2 dB |
| MeasurementInterval | Physical layer measurement interval | 10 ms |
| TMeasurement_Period, Intra | L1 filtering time in TS 36.133 | 200 ms |
| L3RRMCoefficient | Filtering coefficient for layer 3 measurements | 1 |
| Qin | Qin Threshold | -6 dB |
| Qout | Qout Threshold | -8 dB |
| TEvaluate_Qout | Qout evaluation period | L1 samples filtered linearly over a sliding window of 200 ms |
| TEvaluate_Qin | Qin evaluation period | L1 samples filtered linearly over a sliding window of 100 ms |
| T310 | Timer to trigger radio link failure | 1 s |
| N310 | Maximum number of consecutive "out-of-sync" indications from lower layers | 1 |
| T311 | Not used (RLF recover not simulated) | Not used (RLF recover not simulated) |
| N311 | Maximum number of consecutive "in-sync" indications from lower layers | 1 |
| HOPreparationDelay | Handover preparation delay | 50 ms |
| HOExecutionDelay | Handover execution delay | 40 ms |
| RSRPError | Standard deviation of RSRP measurement error | 1.22 dB |
| MTS | Minimum time to stay for ping-pong metric | 1 s |

## UE dropping

It is proposed that each modeled UE starts

- at a randomly selected location in the network,

- with an angular bearing angle selected randomly and uniformly, and

- moves at the assigned speed in a straight line for the duration of the simulation (with wrap-around).

Companies may also add further comments below on the proposed UE dropping model.

Table 5 UE dropping model

| Company | Comment |
| --- | --- |
| Intel | - Bouncing circle model as in section 5.4.5 in TS36.839 - Should also consider non-straight line and changing elevation. |
| Huawei | UEs are randomly dropped in each sector; moves in a non-straight line for the duration of the simulation (with wrap-around); |
| Nokia | This can be left up to RAN1. |
| Qualcomm | We think “wrap-around” is sufficient. RAN2 should try to limit the possible number of combinations of simulations scenarios. Regarding non-straight line movement of UE, we think the additional attribute makes it even harder to compare the simulation results from different companies. In our view, most use cases for drones (for example, package delivery, emergency response) include flights in straight lines for a reasonable distance before changing its course. We also note that TR 36.839 Section 5.4.4. also assumes UE movement in straight-line: “After initially dropped at a random location, the UE will randomly select a direction and move in straight line at a constant speed till hitting the simulation border.” If RAN2 wants to simulate non-straight line, then how such values change over time need to be defined. Note: UE height is discussed separately below. |

Summary:

Each modelled UE starts at a randomly selected location in the network. The UE then moves at the assigned constant speed at the constant height in a straight line for the entire duration of the simulation. The initial horizontal direction (bearing angle) is selected randomly and uniformly. When the UE hit the simulation border (the wrap-around contour), it will wrap around and enter the simulation area from a different point on the wrap-around contour.

## Drone height and speed

RAN2 has agreed that the study will target up to 300m AGL for the Drone UEs. However, it was also noted that the requirements are independent of simulation assumptions.

Companies are encouraged to provide values of UE heights (AGL) and UE speed for the simulations if they are different than proposed values.

Table 6 UE height and speed

| Parameter | Description | Unit | Value Qualcomm | Intel | Ericsson | Huawei | Nokia | KDDI | Summary/ Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE height | Height in meter above ground level | m (AGL) | 0 (i.e., ground), 50, 120 | 50, 300m | (50), 150, 300 | 50, 100, 300m | 30, 100, 300 | 0 50, 100 | 0, 50, 100, 300 |
| UE speed | Horizontal speed | Km/h | 0, 1.5, 3, 30, 60, 120 | 0, 30, 60, 160km/h | 30 , 160 km/h | 3, 30, 60, 160km/h | 30, 60, 160 | 1.5, 3, 60 | 3, 30, 60, 160 |

Companies may also add notes below if they want to justify their choice of values.

Table 7 Additional comments on UE height and speed

| Company | Comment |
| --- | --- |
| Intel | Arrange to study up to 300m and 160km/h, we should at least have simulation on the upper values. |
| Ericsson | Start with 150m and 300m with LOS assumption. When RAN1 finalizes channel models with NLOS, simulate lower hight e.g. 50m or 75m |
| Huawei | Replacing 0km/h with 3km/h as the minimal UE speed value , considering no mobility impact when the UE’s speed is 0km/h. |
| Nokia | The altitude values provided above depend a lot on the channel model/environment. Assuming this would be RMa, the height of 30 m allows to verify the scenario, wherein there is a considerable probability of NLOS, while 100 m and 300 m would most likely be LOS cases. Nevertheless, there could be still differences between 100 and 300 m. Thus, we recommend verifying all three values. Additionally, we would like to suggest another scenario to be checked: vertical take-off/landing. It could be something basic like take-off with e.g. a vertical speed of 30 km/h up to the 50 m altitude. We would be grateful if you could share your opinion on this aspect. |
| Qualcomm | Regarding change in altitude (UE moving up and down), we think the additional attribute makes it even harder to compare the simulation results from different companies. In our view, most use cases for drones (for example, package delivery, emergency response) include flights performed at relatively constant height, except for take-off and landing scenarios as noted by Nokia. We would like to note that RAN1 simulations do not assume change in height once the UE is “dropped” in the scenario. We would also note that TR 36.839 Section 5.4.4. also assumes UE movement in straight-line. If RAN2 wants to simulate change in UE height, then how such values change over time need to be defined. Therefore, we think, except for take-off/landing, modeling of change in altitude is not needed. For take-off/landing scenarios, the RAN1 channel models are “discontinuous” at different height brackets which creates more issues for RAN2 to properly model when UE moves from one altitude bracket to other. Therefore, this aspect may be further discussed in next meeting if companies have enough interest to bring in details of how to model. |

Summary:

Following values are agreed as baseline for mobility simulations. Other values are not precluded if companies are interested to study and submit more results.

| Parameter | Description | Agreed value(s) |
| --- | --- | --- |
| UE height | Height in meter above ground level | 0 m, 50 m, 100 m, 300 m (AGL) (NOTE 1, NOTE 2) |
| UE speed | Horizontal speed | 3 km/h, 30 km/h, 60 km/h, 160 km/h |

NOTE 1: 0 m AGL corresponds to ground UEs.

NOTE 2: As a baseline, UE height is constant throughout the simulation. FFS whether and how to model variable height, e.g. for take-off and landing scenarios.

## Co-existence of Drones and ground UEs

It was also discussed during the meeting that there the drones and ground UEs need to co-exist in the same network. This is particularly important for handover studies due to the anticipated higher interference from Drones which can cause change in performance of ground UEs.

Companies are encouraged to provide values for total number of UEs per cell (this value is on average as the actual value can change due to mobility), and percentage of drones among total UEs for the simulations if they are different than proposed values.

Table 8 Coexistence and distribution of Drone vs ground UEs

| Parameter | Value Qualcomm | Huawei | KDDI | Summary Note |
| --- | --- | --- | --- | --- |
| Total number of UEs per cell (on average) | 7 | 10 | 10 | 15 (Same as RAN1) |
| % of Drone UEs in the scenario | 0 (all ground UEs), 25 (to model initial deployments) 50 (uniformly mixed), 100 (all drone UEs for comparison) | 0 (all ground UEs), 50 (50% percent for ground UEs and 50% for the drone UEs in one cell), 100 (all drone UEs) | 0(all ground UEs), 10(10% percent for ground UEs and 90% for the drone UEs in one cell), 20(20% percent for ground UEs and 80% for the drone UEs in one cell), | Case 1: 0 (all ground UEs), Case 2: 1 UE per 10 sectors Case 3: 1 UE per sector Case 4: 3 UEs per sector Case 5: 5 UEs per sector (Same as RAN1 but all UEs to be outdoor) |

Companies may also add notes below if they want to justify their choice of values.

Table 9 Additional comments

| Company | Comment |
| --- | --- |
| Qualcomm | The reason for one small number (e.g. 25%) is to evaluate the condition of initial deployments where the ratio of drone UEs is expected to be small compared to when drones are more popular. |

Summary:

RAN2 assumes the same number of total and aerial UEs per sector as assumed by RAN1; however, all UEs will be considered as outdoor for RAN2 simulations.

# Performance metrics/KPIs

Editor’s Note: The discussion in this section may be a baseline for TP to Section 5.2 in the TR 36.777.

The results will be provided for the following metrics as described in Table 10. All values are averages. For the cases where both drone and ground UEs co-exist, the metrics will be provided separately for each type of UEs.

Table 10 Performance metrics for HO and RLF simulations

| KPI | Unit | Description |
| --- | --- | --- |
| Handover rate | HO/UE/sec | Number of HOs over time |
| Radio Link Failure (RLF) rate | RLF/UE/sec | Number of RLFs over time |
| Time in handoff | % | Fraction of time a UE is in a HO procedure |
| Time in Qout | % | Fraction of time a UE is in Qout state |

Companies are encouraged to provide further comments below if other KPIs are preferred or above KPIs are not preferred.

Table 11 Additional comments on performance metrics

| Company | Comment |
| --- | --- |
| Intel | Pingpong rate and HOF rate should also be studied. |
| Ericsson | We should follow TS 36.839 Section 5.2.1.2 for RLF logging and for presenting results for RLF. |
| Huawei | We agree with Intel. Pingpong rate and HOF rate should also be studied, following TS 36.839. |
| Nokia | We concur with Ericsson on the suggestion to follow the principles outlined in 36.839. We are also fine with Intel’s comment to take into account HOF and ping-pong rate, in addition to what has been suggested in Table 8. |
| KDDI | We agree with Intel. Pingpong rate and HOF rate should also be studied. |

Summary:

The results will be provided for the following metrics as described in Table 10. All values are averages unless stated otherwise. For the cases where both drone and ground UEs co-exist, the metrics will be provided separately for each type of UEs.

Table 10 Performance metrics for HO and RLF simulations

| KPI | Unit | Description |
| --- | --- | --- |
| Handover rate | HO/UE/sec | Number of HO attempts over time |
| HOF rate | HOF/UE/sec | Number of HO failures over time (including HOF) |
| Radio Link Failure (RLF) rate | RLF/UE/sec | Number of RLFs over time |
| Time in handoff | % | Fraction of time a UE is in a HO procedure |
| Time in Qout | % | Fraction of time a UE is in Qout state |
| Pingpong rate |  | CDF of (number of ping-pongs)/(total number of successful handovers excl. handover failures) |

The definition of Ping-pong and examples of counting method are given in TR 36.839 Section 5.2.2.

# Summary and Conclusion

Individual sections of the email discussion are summarized separately above. Based on the overall discussion above and in the email thread, as the outcome of this email discussion, agreed TP for TR 36.888 is provided in the Annex.

# References

[1] RP-170779, SID on Enhanced LTE Support for Aerial Vehicles, NTT DOCOMO INC, Ericsson

[2] R2-1704155, Handover Results for Aerial Vehicles, Qualcomm Incorporated

[3] R2-1705999, Proposal of potential LTE enhancements for Aerial Vehicles, KDDI

# Annex: Text Proposal for TR 36.777

### === TP start ==

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	RP-170779, "New SID on Enhanced Support for Aerial Vehicles"

[3]	3GPP TR 36.873: "Study on 3D channel model for LTE", V12.4.0

[4]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz", V14.0.0

[5]	3GPP TR 36.872: "Small cell enhancements for E-UTRA and E-UTRAN - Physical layer aspects", V12.1.0

[6]	3GPP TR 36.814: "Further advancements for E-UTRA physical layer aspects", V9.0.0

[7]	3GPP TR 36.839: "Mobility enhancements in heterogeneous networks", V11.1.0

<<unchanged sections skipped>>

## 5.2	Key performance indicator

Editor’s note:	To capture the level of performance in terms of latency, reliability, delay jitter, coverage, data rate, and UE density, positioning accuracy, etc.

For system level evaluation purposes, the following performance metrics are considered:

  - Packet throughput

    - UL and DL packet throughput statistics of aerial UEs Data traffic

    - UL and DL packet throughput statistics of all UEs Data traffic

    - UL and DL packet throughput statistics of terrestrial UEs Data traffic

  - Interference

    - UL IoT (interference over thermal) and DL wideband SINR statistics for reference

    - NOTE: UL IoT above refers to effective IoT defined in clause A.2.1.8 of [6]

  - Other metrics are not precluded

For the mobility evaluation purposes, the following performance metrics are considered:

Table 5.2-1 Performance metrics for HO and RLF simulations

| KPI | Unit | Description |
| --- | --- | --- |
| Handover rate | HO/UE/sec | Number of HO attempts over time |
| HOF rate | HOF/UE/sec | Number of HO failures over time (including HOF) |
| Radio Link Failure (RLF) rate | RLF/UE/sec | Number of RLFs over time |
| Time in handoff | % | Fraction of time a UE is in a HO procedure |
| Time in Qout | % | Fraction of time a UE is in Qout state |
| Pingpong rate (NOTE) |  | CDF of (number of ping-pongs)/(total number of successful handovers excl. handover failures) |

NOTE:	The definition of Ping-pong and examples of counting method are given in TR 36.839 [7, Section 5.2.2].

<<unchanged sections skipped>>

Annex A: Evaluation assumptions

Editor’s note: This section will capture details of evaluation assumptions.

## A.1 System level evaluation

<< Table from RAN1 unchanged/skipped>>

## A.2 Mobility evaluation

### A.2.1 Mobility parameters

For mobility evaluations, the assumptions listed in A.1 and Annex B are applicable unless stated otherwise in this section.

Following values are agreed as baseline for mobility evaluations. Other values are not precluded if companies are interested to study and submit more evaluation results.

| Parameter | Description | Agreed value(s) |
| --- | --- | --- |
| TimeToTrigger | Time to trigger a measurement report | 160 ms |
| A3Offset | Offset between signal strength of serving and neighboring cells | 2 dB |
| MeasurementInterval | Physical layer measurement interval | 10 ms |
| TMeasurement_Period, Intra | L1 filtering time in TS 36.133 | 200 ms |
| L3RRMCoefficient | Filtering coefficient for layer 3 measurements | 1 |
| Qin | Qin Threshold | -6 dB |
| Qout | Qout Threshold | -8 dB |
| TEvaluate_Qout | Qout evaluation period | L1 samples filtered linearly over a sliding window of 200 ms |
| TEvaluate_Qin | Qin evaluation period | L1 samples filtered linearly over a sliding window of 100 ms |
| T310 | Timer to trigger radio link failure | 1 s |
| N310 | Maximum number of consecutive "out-of-sync" indications from lower layers | 1 |
| T311 | Not used (RLF recover not simulated) | Not used (RLF recover not simulated) |
| N311 | Maximum number of consecutive "in-sync" indications from lower layers | 1 |
| HOPreparationDelay | Handover preparation delay | 50 ms |
| HOExecutionDelay | Handover execution delay | 40 ms |
| RSRPError | Standard deviation of RSRP measurement error | 1.22 dB |
| MTS | Minimum time to stay for ping-pong metric | 1 s |
| UE height (NOTE 1, NOTE 2) | Height in meter above ground level | 0 m, 50 m, 100 m, 300 m (AGL) |
| UE speed | Horizontal speed | 3 km/h, 30 km/h, 60 km/h, 160 km/h |
| Outdoor terrestrial UE ratio |  | 100% (NOTE 3) |

NOTE 1:	0 m AGL corresponds to ground UEs.

NOTE 2:	Aerial UE height is constant throughout the simulation.

Editor’s Note: FFS whether and how to model variable height, e.g. for take-off and landing scenarios.

NOTE 3:	Total number of UEs and ratio of Aerial UEs are same as in A.1. However, for mobility evaluations, all UEs are assumed to be outdoor.

### A.2.2 UE Placement and Trajectories

For mobility evaluations, each modelled UE starts at a randomly selected location in the network. The UE then moves at the assigned constant speed at the constant height in a straight line for the entire duration of the simulation. The initial horizontal direction (bearing angle) is selected randomly and uniformly. When the UE hits the simulation border (the wrap-around contour), it wraps around and enters the simulation area from a different point on the wrap-around contour.

### A.2.3 LOS/NLOS modelling

LOS or NLOS for an Aerial UE is fixed throughout the simulation based on initial determination of LOS/NLOS.

Editor’s Note: FFS whether and how to model LOS/NLOS switching.

### A.2.4 Time varying shadow fading

Time varying shadow fading for a moving UE is modelled by recalculating shadow fading value based on standard deviation given in Table B-3 after the UE has travelled distance of 25 m (based on its speed).

### ==TP End==