3GPP TSG-RAN WG2 #99	Tdoc R2-1708348

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.1.6

Source:	Ericsson

Title:	Further aspects on BSR transmission and cancellation

Document for:	Discussion, Decision

# Introduction

Uplink scheduling is a key functionality to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC, and additional requirements defined during the RAN requirements study. Details to how this functionality can be designed, compared to the baseline framework of LTE is discussed in contributions.

In RAN2#98 some agreements related to BSR were reached.

Agreements

- The number of LCGs will be increased up to 8.

- The concept of periodicBSR-timer and retx-BSRtimers are reused and are configured per MAC entity

- As a baseline, the concept of logicalChannelSR-ProhibitTimer is reused in NR. It is allowed to configured infinite value for this timer.

- The logicalChannelSR-Mask is supported

In RAN2#AdHoc held on 27th – 29th June 2017, some agreements related to the BSR triggers were made

Agreements

=>	No new BSR triggers in addition to LTE are needed

Being based on the LTE triggers as a baseline for NR, this contribution discusses the further potential enhancements for BSR triggers to better support multiple numerologies. BSR formats are discussed in [1].

# Discussion

## BSR transmission and cancellation in non-CA scenarios

In the LTE baseline, all triggered BSRs can be cancelled in either of two cases

- the UL grant(s) in this TTI can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader.

- a BSR is included in a MAC PDU for transmission.

An NR UE may be configured with multiple numerologies. The BSR is triggered per LCG as like LTE. It is naturally assumed that LCHs grouped in the same LCG have similar QoS requirements and correspondingly are mapped with similar numerologies/TTI types.

It can be assumed that LCHs in the same LCG have similar QoS requirements, and are mapped with similar numerologies/TTI types.

Several BSR events may be triggered at the same time, for different LCGs, configured with different numerologies. From the technology potential aspect, the UE MAC entity may have several options to transmit BSRs upon reception of an uplink grant, which is associated with a given numerology. These options are described as below

Option 1:  transmit a BSR comprising of all BSs for all LCGs with LCHs with data available regardless of which numerology is associated with the grant. The BSRs are cancelled after transmission of the MAC PDU including the BSR.

Option 2: transmit a BSR comprising of BSs for LCGs with LCHs with data available, and mapped with the numerology associated with the grant. For rest of LCGs which are not mapped with the numerology of the grant, the BSRs can be transmitted at later TTIs, when appropriate grants are received.

Option 3:  transmit a BSR comprising of all BSs for all LCGs with LCHs with data available regardless of which numerology is associated with the grant. The BSRs for LCGs, not mapped with numerology of the current grant, are not cancelled, and can be also transmitted at subsequent TTIs with other grants.

Option 1 is similar to LTE. We think it is simple and easy to control BSR transmissions. The other two options are mainly aiming to achieve the goal, i.e., BSR for LCHs are transmitted on the grant with suitable numerologies. They behave as if LCP procedure would be performed for not only LCHs, but also the BSR MAC CEs. They are complex and may bring unpredictable UE behaviours, since it is difficult for the UE to know which grants are going to be received in advance. Therefore, the gain is unclear compared to Option 1.

As like LTE, a BSR comprises BSs for all LCGs with LCHs with data available regardless of which numerology is associated with the received grant

Similarly, the same cancellation rules as LTE should be applied for BSR in NR.

As like LTE, all triggered BSRs can be cancelled in either of two cases

- A UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader.

- a BSR is included in a MAC PDU for transmission.

At the same time, a Padding BSR is not allowed to cancel a triggered Regular/Periodic BSR. A Padding BSR is triggered for a specific MAC PDU only and that trigger is cancelled when the MAC PDU with the Padding BSR has been built.

As like LTE, a Padding BSR does not cancel a triggered Regular/Periodic BSR.

## BSR transmission in CA scenario

In LTE, the MAC entity shall transmit at most one Regular/Periodic BSR in a TTI. If the MAC entity is requested to transmit multiple MAC PDUs in a TTI, the UE can select any one of the MAC PDU to carry the Regular/Periodic BSR, further it may include a padding BSR in any of the MAC PDUs which does not contain a Regular/Periodic BSR.

In NR, the UE MAC entity may receive multiple grants at the same time, and each grant may be associated with a separate numerology/TTI length. The gain by allowing more than one Regular/Periodic BSRs to transmit is not clear. It may also lead to additional complexity for BSR transmission and cancellation. Therefore, we suggest to reuse the LTE rules for NR.

As like LTE, the MAC entity shall transmit at most one Regular/Periodic BSR when receiving multiple grants at the same time and it is up to UE implementation on the selection of which serving cell to transmit Regular/Periodic BSR.

## BSR timers

There are two timers controlling BSR in LTE. The periodicBSR-Timer which triggers BSR periodically, and the retxBSR-Timer which triggers BSR if the UE has data and has not been scheduled. These timers are configured in subframes in LTE. As one subframe has the duration of one millisecond in both LTE and NR, we think they can be configured in either subframes or milliseconds. Buffer status reporting is a procedure in which the UE informs the gNB about the amount of data in the UE such that the gNB can make a scheduling decision. There seems to be no impact from varying numerologies or TTI durations in this case. However, as the general timing of everything (e.g. processing times) in NR is shorter than in LTE, shorter values may be considered.

The periodicBSR-Timer and retxBSR-Timer are defined in either subframes or milliseconds. Adding shorter values than in LTE can be considered.

# Conclusion

In section 2 we made the following observations:

Observation 1	It can be assumed that LCHs in the same LCG have similar QoS requirements, and are mapped with similar numerologies/TTI types.

Based on the discussion in Section 2 we propose the following:

Proposal 1	As like LTE, a BSR comprises BSs for all LCGs with LCHs with data available regardless of which numerology is associated with the received grant

Proposal 2	As like LTE, all triggered BSRs can be cancelled in either of two cases

1)	A UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader.

2)	a BSR is included in a MAC PDU for transmission.

Proposal 3	As like LTE, a Padding BSR does not cancel a triggered Regular/Periodic BSR.

Proposal 4	As like LTE, the MAC entity shall transmit at most one Regular/Periodic BSR when receiving multiple grants at the same time and it is up to UE implementation on the selection of which serving cell to transmit Regular/Periodic BSR.

Proposal 5	The periodicBSR-Timer and retxBSR-Timer are defined in either subframes or milliseconds. Adding shorter values than in LTE can be considered.

# References

- R2-1708349, BSR format in NR, Ericsson