3GPP TSG RAN WG2 Meeting #99 		R2-1708572

Berlin, Germany, 21st – 25th August 2017

Source:	SoftBank

Title:	Further analysis on APDC solution

Agenda Item:	9.3

Document for:	Discussion and Decision

Introduction

In the RAN#76, Uplink data compression (UDC) SI was extended for a further analysis on APDC solution as follows [1].

conclusion: additional information on APDC solution is provided to RAN2 by 26.06.2017, RAN2 will do corresponding analysis on APDC solution and RAN #77 in Sep.17 will decide between DEFLATE-based solution and APDC solution

In this contribution, we share our evaluation results on APDC solution.

Analysis on APDC solution

Compression algorithm

At the request of RAN#76, the sample source code of APDC compressor was provided to RAN2 by e-mail. In the last RAN2 meeting, it was pointed out that APDC compression algorithm is unclear, so we analyzed whether or not it is disclosed the compression algorithm in the sample source code.

The flowchart of APDC compression algorithm is captured in TR [2] (also copied in Annex). It consists of three steps (Steps 1, 2 and 3). In Step 1, the compressor searches the same data blocks between the stored packet and the new packet and counts the number of matched data blocks and mismatched data blocks. In Step 2, the compressor creates APDC common header and sets an appropriate packet action value to APDC common header according to the number of matched/mismatched data packets in Step 1. In addition, the compressor selects to use PMCR header or CPCR header or both. In Step 3, the compressor checks whether the header in Section 2 is the same as the previous one. If it is the completely same one, the compressor replaces it to APDC common header with a special value to reduce the packet size.

A result of the sample source code analysis is shown in Table.1, the functions corresponding to the APDC compression algorithm can be found in the sample source code.

Observation 1:	APDC compression algorithm is well disclosed.

Table.1 - Corresponding function of APDC compressor

| Step # | Corresponding function | Description |
| --- | --- | --- |
| 1 | apdc_pmcr_compress() apdc_cpcr_compress() | Search the number of matching or mismatching data blocks by comparing packets |
| 2 | apdc_build_packet() | Create the appropriate APDC headers (Common, PMCR, CPCR) |
| 3 | apdc_metadata_match_prev_pkt() | Check whether it is the same header and replace it to a short one if it is the same |

Compression efficiency

Tables. 2 and 3 show the evaluation results of APDC compression efficiency based on the provided sample source code. It was evaluated with 8Kbyte and 32Kbyte buffer memory, respectively. The overhead of APDC headers is included in the compressed file size. Given the results, we obtained a very similar performance between our results and Qualcomm’s [2].

Observation 2:	For APDC solution, multiple companies can achieve consistent compression performance with the same configuration.

Table.2 - Evaluation results of APDC (8Kbyte buffer)

| PCAP file | Original file size | Compressed file size (8K) | Compression Efficiency (8K) | Qualcomm [2] |
| --- | --- | --- | --- | --- |
| FTP data-Client (CMCC) | 1211 | 533 | 54.34% | 54.74% |
| FTP data-Server (CMCC) | 1782 | 885 | 50.34% | 50.39% |
| Video data (CMCC) | 13450 | 5246 | 61.00% | 62.04% |
| Long Video data (CMCC) | 1371861 | 319995 | 76.67% | 78.44% |
| SIP UE01 (CMCC) | 51020 | 8208 | 83.91% | 85.61% |
| SIP UE02 (CMCC) | 32680 | 6332 | 80.62% | 82.16% |
| SIP UE03 (CMCC) | 46688 | 7377 | 84.20% | 85.94% |
| Web surfing (CMCC) | 2381720 | 851605 | 64.24% | 67.75% |
| Video data (MediaTek) | 2453749 | 651029 | 73.47% | 73.98% |
| Long FTP data (MediaTek) | 879630 | 216959 | 75.34% | 75.34% |
| Multiple IP-flow (Qualcomm) | 5319100 | 1417499 | 73.35% | 75.32% |

Table.3 - Evaluation results of APDC (32Kbyte buffer)

| PCAP file | Original file size | Compressed file size (32K) | Compression Efficiency (32K) | Qualcomm [2] |
| --- | --- | --- | --- | --- |
| FTP data-Client (CMCC) | 1211 | 533 | 54.34% | 54.74% |
| FTP data-Server (CMCC) | 1782 | 885 | 50.34% | 50.39% |
| Video data (CMCC) | 13450 | 5240 | 61.04% | 62.04% |
| Long Video data (CMCC) | 1371861 | 299971 | 78.13% | 78.44% |
| SIP UE01 (CMCC) | 51020 | 7557 | 85.19% | 85.61% |
| SIP UE02 (CMCC) | 32680 | 5954 | 81.78% | 82.16% |
| SIP UE03 (CMCC) | 46688 | 6687 | 85.68% | 85.94% |
| Web surfing (CMCC) | 2381720 | 788237 | 66.90% | 67.75% |
| Video data (MediaTek) | 2453749 | 640067 | 73.91% | 73.98% |
| Long FTP data (MediaTek) | 879630 | 217047 | 75.33% | 75.34% |
| Multiple IP-flow (Qualcomm) | 5319100 | 1344941 | 74.71% | 75.32% |

Conclusion

In this contribution, we show the results of further analysis on APDC solution.

Observation 1:	APDC compression algorithm is well disclosed.

Observation 2:	For APDC solution, multiple companies can achieve consistent compression performance with the same configuration.

According to the observations, we think APDC is implementable and consistent solution.

Proposal:	Update the conclusion in TR to include the above observations.

References

[1] RAN#76 Draft Meeting Report

[2] 3GPP TR 36.754 V15.0.0 (2017-06)

- Annex A.	Flowchart of the APDC compressor

Figure 1. Example on Compressor Illustration [2]