3GPP TSG-RAN WG2 #99	R2-1708666

Berlin, Germany, 21 - 25 August 2017

Agenda item:	9.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	LOS/NLOS switching in mobility simulations

WID/SID:	FS_LTE_Aerial - Release 15

Document for:	Discussion and Decision

# 1	Introduction

RAN2 discussed mobility simulation assumptions during the course of an e-mail discussions summarized in [1]. Based on the discussion it is proposed in [1] to capture, among others, such simulation assumptions in TR 36.777:

### A.2.3 LOS/NLOS modelling LOS or NLOS for an Aerial UE is fixed throughout the simulation based on initial determination of LOS/NLOS. Editor’s Note: FFS whether and how to model LOS/NLOS switching. ### A.2.4 Time varying shadow fading Time varying shadow fading for a moving UE is modelled by recalculating shadow fading value based on standard deviation given in Table B-3 after the UE has travelled distance of 25 m (based on its speed).

In this contribution, we discuss the FFS related to LOS/NLOS switching captured by rapporteur of the e-mail discussion.

# 2	Discussion

The proposal to determine LOS or NLOS for a UE only once, at the beginning of the simulation, was made to reduce the complexity of the simulations. On the other hand, in consequence the credibility of simulation results based on such an assumption is decreased and it is misaligned with the decision to recalculate shadow fading for a UE after it covered a distance of 25 m [1]. We think that for the sake of simulation results’ accuracy it would be beneficial to apply time varying LOS/NLOS determination, similarly to shadow fading recalculation.

Proposal 1: Time-varying LOS/NLOS determination should be applied in mobility simulations.

The following section proposes a simple implementation of such approach.

# 3	Time-varying LOS/NLOS determination

As commented in [1] (Table 1) there are at least two simple models to implement a time-varying LOS/NLOS, based on typical channel modelling consideration:

- LOS/NLOS correlation distance, where the correlation distance is taken e.g. from TR36.839 ([2]) Table 5.2.3.1 or TR36.814 ([3]) Table B.1.2.2.1-4

- Time-correlation, based on the UE speed and a correlation distance from a)

We support to use the model option b) which is aligned with the time-varying shadow fading model agreed in [1].

Proposal 2: The adopted time-varying LOS/NLOS model should be based on the UE speed and LOS/NLOS correlation distance.

A possible implementation of the proposed time-varying LOS/NLOS model is described in Annex as pseudo-code, where:

- The LOS probability models captured in [4] are used to generate in each simulated time step an initial “LOS” or “NLOS” label, for each UE.

- LOS_CorrelationDistance and NLOS_CorrelationDistance parameter are the 2-D correlation distances [meter] for LOS and NLOS channel conditions, respectively. For simplicity, these two values can be considered equal e.g. as in TR36.839 Table 5.2.3.1.

- d_LOS or d_NLOS is the travelled distance [meter] while in the last LOS or NLOS channel condition, based on the UE speed

# 4	Summary

Proposal 1: Time-varying LOS/NLOS determination should be applied in mobility simulations.

Proposal 2: The adopted time-varying LOS/NLOS model should be based on UE speed and LOS/NLOS 2-D correlation distance.

# References

- R2-1706207 Email discussion report [98#49]: Common RAN2 parameters for RLF and HOF simulations - Qualcomm Incorporated

- 3GPP TR 36.839, Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks

- 3GPP TR 36.814, Evolved Universal Terrestrial Radio Access (E-UTRA); Further advancements for E-UTRA physical layer aspects

- 3GPP TR 36.777 V0.1.0, Study on Enhanced LTE Support for Aerial Vehicles

# Annex – possible implementation of time-varying LOS/NLOS model

---------------

FOR each UE

FOR each simulated time step

Step1: Draw a random number from a uniform distribution in the interval [0 1] and compare it with LOS probability threshold given by the scenario specific LOS link probability model from [4]

Step 2:

IF Step 1 yields “LOS”

IF UE was in LOS channel condition THEN

Keep UE in LOS channel condition

ELSE

Calculate travelled UE distance while in NLOS, d_NLOS

IF d_NLOS > NLOS_CorrelationDistance THEN

Change UE to LOS channel conditions

ELSE

Keep UE in NLOS channel condition

ELSE IF Step 1 yields “NLOS”

IF UE was in NLOS channel conditions THEN

Keep UE in NLOS channel condition

ELSE

Calculate travelled UE distance while in LOS, d_LOS

IF d_LOS > LOS_CorrelationDistance THEN

Change UE to NLOS channel condition

ELSE

Keep UE in LOS channel condition

---------------