3GPP TSG-RAN WG2 NR#2	Tdoc R2-1706580

Qingdao, China, 27th – 29th June 2017

Agenda Item:	10.4.1.5.1

Source:	Ericsson

Title:	System information handling in EN-DC

Document for:	Discussion, Decision

# Introduction

NR system information structure and content for standalone operation is presented in [2]. In this paper, we discuss which if this information is needed during non-standalone operation (EN-DC), and how this information is delivered to the UE. It is important to identify which IEs that are needed for EN-DC operation, since these need to be ready for the December drop.

# Discussion

A baseline for NR-SIB1 contents is proposed in [2], and repeated in table 1, including analysis for each field whether it is needed for EN-DC operation, or not.

Table 1: Baseline NR-SIB1 contents. Note that additional information elements are likely to be included based on further RAN2 decisions.

| Information element | Comment | Needed in EN-DC? |
| --- | --- | --- |
| List of PLMNs | Including indication of a cell reserved for operator use. May need to be extended to support new use cases, see [1] | No. UE gets this from the LTE SI. |
| Tracking Area Code | Need to be verified with SA2. May need to be extended to support new use cases, see [1] | No. Idle mode mobility is on LTE Cells. |
| Cell Identity | May need to be extended to support new use cases, see [1] | Possibly needed for ANR purpose, in case MeNB does not recognize CID reported by UE. |
| Cell camping parameters | Cell camping parameters | No. UE will not camp on NR cell in EN-DC. |
| p-Max | Need to be verified with RAN1 | Yes, this is probably needed. Need to be verified with RAN1 |
| Frequency Band Information | Including multi-band information Including UL band information from LTE SIB2. | Yes. This information is needed, for UE to know the NR band information. |
| Access Barring parameters | May aim for a unified approach instead of reusing LTE IEs. | No, access barring will be based on LTE. |
| Initial access configuration | Including RACH configuration, but also other physical channel and higher layer configurations needed at least to be able to receive messages 2 and 4 and to transmit msg3. | Yes, UE needs RACH configuration to access the NR cell. |
| Scheduling Info List | SI scheduling information parameters | No. UE does not have to read further SI from NR cell. |
| Additional parameters for requesting other SI | E.g. special RA preamble values, dedicated RACH opportunities, … | No. UE does not have to read further SI from NR cell. |
| Most significant bits of SFN | Corresponding to HFN in LTE | Yes, this is needed to complement the SFN read from MIB. |

On top of the information listed in Table 1, cell camping parameters and SI scheduling information parameters will also be included in SIB1, but none of these are expected to be needed for NSA operation, as UE will not camp on NR cells, and also not need request NR SI.

To ensure timely completion of NR SI parameters needed for EN-DC operation, it is proposed to focus completion on the parameters identified in table 1 as needed for EN-DC operation, namely p-Max, Frequency Band Information, Initial access configuration and Most significant bits of SFN. All these parameters can be signalled to the UE in RRCConnectionReconfiguration, when EN-DC is configured.

Work should focus on completing SIB1 IEs needed during EN-DC operation, namely p-Max, Frequency Band Information, Initial access configuration and Most significant bits of SFN. This information will be provided to the UE via LTE RRCConnectionReconfiguration, during SCG addition,

Cell identity may be needed for ANR purpose, in case the NR cell cannot be uniquely identified solely on CID. ANR operation for EN-DC is still under discussion and several alternatives proposed, see e.g. [3]. One alternative is to rely on similar procedure as was used in LTE, where a UE is requested to read a globally unique identifier (ECGI) broadcasted in SIB1 from the unknown cell and report to serving cell. However, this would require constant broadcasting of SIB1 in NSA NR cells, which would be wasteful, considering that the ANR feature is rarely used (only during neighbor relation establishment or change). Therefore, we propose to investigate further other options in [3], not depending on broadcasted SIB1 in NSA NR cell.

Await solution for ANR in EN-DC, before deciding whether global cell identity needs to be broadcasted from NSA NR cell.

The content of other NR SIBs has not yet been agreed, but looking at the LTE equivalents, they include information used for cell re-selection (inter- or intra-frequency, intra- or interRAT), HeNB, EWTS, MBMS, sidelink, etc. This kind of information will not be needed from the NR cell in EN-DC operation. Instead, UEs will use the corresponding LTE information.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Work should focus on completing SIB1 IEs needed during EN-DC operation, namely p-Max, Frequency Band Information, Initial access configuration and Most significant bits of SFN. This information will be provided to the UE via LTE RRCConnectionReconfiguration, during SCG addition,

Proposal 2	Await solution for ANR in EN-DC, before deciding whether global cell identity needs to be broadcasted from NSA NR cell.

# References

R2-1704390, System information content at network sharing, Ericsson

R2-1706495, System information structure and contents, Ericsson

R2-1707283, Neighbor relation establishment in NSA LTE-NR, Ericsson