3GPP TSG RAN WG2#NR_AdHoc#2	R2-1706650

Qingdao, China, 27 - 29 June, 2017                                 (Revision of R2-1704656)

Agenda item:	10.2.4

Source: 	ZTE Corporation

Title: 	Consideration on the LTE/NR tight interworking with NG-C

Document for:	Discussion and Approval

Introduction

The NR WI has been approved in RP-170847[1], and all the LTE/NR tight interworking options 3/4/7 are included in the working scope approved. During the discussion in RAN2, it can be found the procedure of option 3 is relatively clear. However, for the option 4/7, considering the use of new QoS architecture, there are still lots of open issues for study. So, the intention of this contribution is to share some views on the new QoS related open issues for option 4/7.

Discussion

ISSUE 1: Which node should determine how many DRB should be established in secondary node?

In LTE, there is a one to one mapping between E-RAB and RB, so in case one E-RAB is offloaded to SeNB, the SeNB will establish one DRB for this offloaded E-RAB accordingly.  However, in NR, the concept of PDU session and QoS flow are used instead of E-RAB, and the mapping between E-RAB and DRB is managed by gNB itself. Since it is possible to map multiple QoS flow of the same PDU session into one DRB, it should be clarified that which node should determine how many DRB should be established in the secondary node in the LTE/NR tight interworking.

Considering the mapping between DRB and QoS flow is determined by the RRM, and a separate RRM is located in secondary node, we propose that it is up to secondary node to determine how many DRB should be established and the mapping between the DRB and the offloaded QoS flow.

Proposal 1: For the option 4/7, it is up to MN/SN to determine how many DRB should be established in the MN and SN respectively.

Proposal 2: For the option 4/7, it is up to MN/SN to determine the mapping between DRB and QoS flow in the MN and SN respectively.

ISSUE 2: Which node should determine the DRB ID used in secondary node?

In LTE, since there is only one RRM located in MN, the DRB ID is allocated by MN. However, in NR, based on the proposal 1&2, it can be observed that the number of DRB in MN/SN is determined by MN/SN respectively, and it will be difficult for MN to predict how many DRB ID is needed in SN side and pre-allocate the DRB ID before the DRB has been established in SN. Based on the analysis above, in order to achieve a better independency/isolation between LTE and NR, we propose that the DRB ID for the DRB established in SN should be allocated by the SN itself.

Proposal 3: For the option 4/7, the DRB ID for the DRB established in SN should be allocated by the SN itself.

Based on the proposal 3, considering one common DRB ID space will be used in LTE/NR tight interworking, one more issue need to be addressed is how to coordinate the DRB ID allocated by MN and SN within the common DRB ID space, and some kind of coordination is required over Xn interface to insure that the DRB ID allocated by MN and SN will not collide with each other.

Proposal 4: For the option 4/7, some coordination over Xn interface is required to coordinate the DRB ID used in MN and SN within the common DRB ID space, the detail can be discussed in stage3 phase.

ISSUE 3: How to understand the configuration of bearer type and bearer type change in the NGC-based LTE/NR tight interworking?

In LTE, since there is one to one mapping between E-RAB and DRB, the configuration of bearer type indicate in which bearer type the E-RAB should be served. However, in NR, since the QoS flow is introduced as the basic granularity of QoS control instead of E-RAB and multiple QoS flow from one PDU Session can be mapped to one DRB, the bearer type should be configured for each QoS flow to indicate in which bearer type the QoS flow should be served.

Proposal 5: For the option 4/7, the bearer type (i.e. MCG split bearer, SCG bearer, SCG split bearer) should be configured to SN for each QoS flow offloaded to indicate in which bearer type the QoS flow should be served.

Since the number of DRB and the mapping between DRB and QoS flow in SN is determined by SN itself. It is not reasonable for the MN to change the bearer type of one specific DRB. In addition, considering the original intention of bearer type change is to change bearer type in which the E-RAB should be served and the QoS flow is used as the basic granularity of QoS control, the bearer type change procedure should be used to the change bearer type of one QoS flow (i.e.  in which bearer type one QoS flow should be served) instead of changing the bearer type of one specific DRB.

Proposal 6: For the option 4/7, the bearer type change procedure should be used to the change bearer type of QoS flow (i.e.  In which bearer type one QoS flow should be served) instead of the bearer type of DRB.

Conclusion

It is proposed that RAN2 discuss and adopt on following proposals:

ISSUE 1: Which node should determine how many DRB should be established in secondary node?

Proposal 1: For the option 4/7, it is up to MN/SN to determine how many DRB should be established in the MN and SN respectively.

Proposal 2: For the option 4/7, it is up to MN/SN to determine the mapping between DRB and QoS flow in the MN and SN respectively.

ISSUE 2: Which node should determine the DRB ID used in secondary node?

Proposal 3: For the option 4/7, the DRB ID for the DRB established in SN should be allocated by the SN itself.

Proposal 4: For the option 4/7, some coordination over Xn interface is required to coordinate the DRB ID used in MN and SN within the common DRB ID space, the detail can be discussed in stage3 phase.

ISSUE 3: How to understand the configuration of bearer type and bearer type change in the NGC-based LTE/NR tight interworking?

Proposal 5: For the option 4/7, the bearer type (i.e. MCG split bearer, SCG bearer, SCG split bearer) should be configured to SN for each QoS flow offloaded to indicate in which bearer type the QoS flow should be served.

Proposal 6: For the option 4/7, the bearer type change procedure should be used to the change bearer type of QoS flow (i.e. In which bearer type one QoS flow should be served) instead of the bearer type of DRB.

References

- RP-170847	New WID on New Radio Access Technology	NTT DOCOMO, INC