3GPP TSG RAN WG2#NR_AdHoc#2	R2-1706655

Qingdao, China, 27 - 29 June, 2017                                  (Resubmission of R2-1704650)

Agenda Item:	10.2.4

Source: 	ZTE Corporation

Title:  	Consideration on the intra-MN handover without SCG change involved

Document for:	Discussion and decision

Introduction

In LTE DC mobility scenarios, SCG change is always triggered whatever in the case of Intra-MeNB or inter-MeNB Handover due to the PDCP re-establishment happens in MeNB, and S-KeNB refresh is correspondingly  triggered by KeNB refresh.

For EN-DC (option 3/3a/3x), similar with LTE DC, PDCP re-establishment in MeNB also happens during the intra MeNB mobility, thus SCG change is still needed to be triggered in the case of Intra-MeNB Handover.

For NE-DC (option 4/4a), intra-gNB handover without PDCP reset/re-establishment is supported in case the PDCP anchor is not changed during the handover, see below agreement [1]:

RAN2#96： - RRC involved handover with and without PDCP entity re-establishment is supported.

A related agreement about security aspect was reached in NR adhoc meeting[2]:

RAN2#Ad-hoc 201701: 1: Security key refresh is not performed at every mobility procedure (i.e. handover), at least for the case of mobility where the PDCP anchor point is not changed (To be confirmed by SA3)

SA3 reply confirmation in RAN2#97 that handover without key change is acceptable, but they still need to consider one aspect related to the exposure of network topology. No further conclusion on above aspect was provided by SA3 yet.

For NGEN-DC (option 7/7a/7x), there's no agreement reached that whether the PDCP design of eLTE should follow LTE or NR, thus in case of intra-MN handover of option7 family, whether SCG change is triggered needs waiting for further progress of eLTE WI.

Introducing intra-MN HO without SCG change

Based above progress, at least for NE-DC (option4/4a), SCG doesn't have to be changed in case of intra-MN handover without PDCP re-establishment, i.e. KeNB of MN is not refreshed thus S-KeNB doesn't need to be changed which doesn't cause SCG change procedure.

Based this analysis, we propose to introduce a new procedure of intra-MN handover without SCG change to TS37.340 (stage2 for multi-connectivity) for NE-DC (option4/4a). The corresponding Text Proposal is provided in the below section.

For NGEN-DC (option7/7a/7x), whether this new procedure could be applied depends on the PDCP design of eLTE,  which needs to wait eLTE WI progress.

Conclusion

Proposal 1: Intra-MN handover without SCG change procedure should be supported for NE-DC(option4/4a).

Proposal 2: It's proposed to capture the Text Proposal in TS37.340.

Text proposal to TS37.340

10.X	Intra-Master Node handover without SCG change involved

[NE-DC]

This procedure is used to perform handover within the same MN while keeping the SCG unchanged.

Figure 10.X-1: Intra-MN handover without SCG change procedure

1.	The MN triggers the UE to apply the new configuration of target cell.

2. 	The UE synchronizes to the target cell under the same MN.

3.  The UE inform MN about the successful completion of intra-MN handover.

Noted that without SCG change involved, the above procedure is same with the intra-node handover of single connectivity scenario.

Reference

[1] R2-17xxxx_draft_report_RAN2_96_Reno_v0.1.doc

[2] RAN2-NR-AH-Spokane-Chairman-Notes-2017-01-19-eom.doc