3GPP TSG RAN WG2#NR_AdHoc#2	R2-1706659

Qingdao, China, 27 - 29 June, 2017                                 (Resubmission of R2-1704652)

Agenda item:	10.2.6.3

Source: 	ZTE Corporation

Title: 	Consideration on the capability restriction update procedure in LTE/NR tight interworking

Document for:	Discussion and Approval

Introduction

During RAN2 #97, UE capabilities for SA NR has been discussed,  the following agreements and one FFS have been achieved:

- Agreements - 1: Following issues should be considered in NR design (e.g. capabilities) with general solution: -  Issue 1: Hardware sharing between NR and other things, e.g. WLAN, BT, GPS, etc; -  Issue 2: Interference between NR and other things, e.g. WLAN, BT, GPS, etc; -  Issue 3: Exceptional UE issues (e.g. overheating problems). - 2: The UE radio access capabilities are static and the change is just to temporarily (e.g. under network control) limit the availability of some capabilities, e.g. due to hardware sharing, interference or overheating. - 3: The temporary capability restrict should be transparent to the NG core, i.e. only static capability is stored in the NG core. - 4: The UE signals the temporary capability restriction request to the gNB. - FFS: To which capabilities it may apply and how the limitation is expressed to the gNB.

Since the new procedure (i.e. the UE signals the temporary capability restriction request to the gNB) has been agreed to be introduced in NR, in this contribution, we intend to discuss on the procedure about how the temporary capability restriction should be performed for LTE/NR tight interworking.

Discussion

For LTE/NR tight interworking, in the scenario with NR as master (i.e. Option 4), based on the above agreements, the UE could naturally signals the temporary capability restriction request to the NR MgNB, and this restriction may lead to the re-negotiation of UE capability between NR MgNB and LTE SeNB. The procedure is illustrated in Fig 1 (the message name is just for example).

Fig 1 Temporary capability restriction procedure in Option 4

Proposal 1: The procedure of UE signalling the temporary capability restriction request to the gNB could be performed in LTE/NR tight interworking when NR as master.

In the scenario with LTE as master (i.e. Option 3/7), the RRC message of temporary capability restriction request to the eNB is not supported in legacy LTE, and considering the complexity of introducing this new RRC procedure into LTE specification, we think it is not appropriate.

However, since the SCG SRB could be established between NR SgNB and UE, which making NR RRC signalling transferred directly, then the temporary capability restriction request message could be signalled from UE via SCG SRB and be forwarded to LTE MeNB via X2 interface by NR SgNB. When receiving the message, LTE MeNB may decide to performe the UE Capability Re-negotiation procedure with NR SgNB, as Fig 2 shows.

Fig 2 Temporary capability restriction procedure in Option 3/7

Proposal 2: In LTE/NR tight interworking when LTE as master, the procedure of UE signalling the temporary capability restriction request to the gNB could be performed via SCG SRB.

Conclusion

It is proposed that RAN2 discuss and adopt on following observations and proposals:

Proposal 1: The procedure of UE signalling the temporary capability restriction request to the gNB could be performed in LTE/NR tight interworking when NR as master.

Proposal 2: In LTE/NR tight interworking when LTE as master, the procedure of UE signalling the temporary capability restriction request to the gNB could be performed via SCG SRB.

References

- R2-1701852	Consideration on NR UE capability	Huawei, HiSilicon