3GPP TSG-RAN WG2 NR Adhoc#2 Meeting	R2-1706725

Qingdao, China, Jun 27 – 29, 2017	revision of R2-1704885

Agenda item:	10.4.1.3.4

Source: 	Huawei, HiSilicon

Title: 	RRC state transition from INACTIVE to IDLE

Document for:	Discussion and Decision

# Introduction

According to the previous agreement on the state transition for NR, the RRC state transition from RRC_INACTIVE to IDLE is supported.

Agreements

7: The RRC state transition from INACTIVE to IDLE is supported

FFS For what cases this transition is supported (e.g. reject from network, other failure cases, other cases, etc).

In RAN2#98 meeting, it was agreed that network initiated RRC state transition from INACTIVE to IDLE follows INACTIVE to CONNECTED and then CONNECTED to IDLE as a baseline.

In this paper, we’d like give further analysis on the possible other use cases and solutions to execute the state transition from RRC_INACTIVE to IDLE.

# State transitions from RRC_INACTIVE to RRC_IDLE

For inactive UE, it will be known in the RAN level. In our viewpoint, the inactive UE will be configured to initially inform the network of the RNA and/or CN level location in case the change of RNA and/or CN level location or due to periodical RNAU timer expiration. The RNAU and/or CN level location update will be contained in a RRC signalling message, possibly RRC re-activation request [1]. Beside, the inactive UE can also trigger RRC re-activation request message due to other reasons, e.g., UL data to transmission. Generally, the UE sends RRC re-activation request message to the network and receives a confirmation message from the network.

However, if one of the following cases occur, it is desirable for the UE or network to perform state transition from RRC_INACTIVE to RRC_IDLE.

Case 1: Inactive UE fails to send the re-activation message

From the UE perspective, it is possible that the UE fails to send the RRC re-activation request message. This can be caused by out of coverage or service, or unsuccessful RRC re-activation request procedure, e.g., RRC re-activation request transmission failure or receiving no response from the network. When the failure condition is met, e.g., continuous failure numbers exceed the defined maximum times or the defined trying timer expired, it is reasonable that the inactive UE initially clears the stored UE context and enters idle. In the situation, the current quality is undesirable for the service. It is possible that similar failure will occur if the inactive UE tries to re-establish the connection to the network. Consequently, falling back into idle from inactive seems to be the best solution. Similar consideration and conclusion has also been discussed in light connection in LTE.

Fig. 1 Unsuccessful Re-activation procedure

Proposal 1: The inactive UE performs state transition to idle state when the inactive UE fails to send the reactivation message.

Case 2: The UE cannot find the suitable cell

For an inactive UE, it will perform cell reselection procedure if current cell quality cannot meet the defined criterion. Due to possibly unexpected network deployment, it is possible that in some locations the inactive UE cannot find the suitable cell to camp for a period of time. As a result, it is also desirable for the inactive UE to clears the stored UE context and enters idle upon the timer expires.

Proposal 2: The inactive UE performs state transition to idle state when the UE cannot find the suitable cell for a period of time.

Case 3: CN based paging is received

If the DL data comes, as an abnormal case, the network cannot find the expected UE by RAN based paging. It is reasonable for the network to consider the UE having moved out of the previously configured RNA. The network can release the S1 connections and consider the inactive UE as an idle UE. Therefore, the network will fall back to CN based paging to find the UE again. We assume that the UE can differentiate the CN based paging and RAN based paging via an indicator or the content of UE ID, e.g. S-TMSI or IMSI for CN based paging, RAN allocated UE ID for RAN based paging.  If the “inactive” UE receives the CN based paging, it implies that it has been an idle UE from network perspective. Consequently, it is desirable that the inactive UE should enter idle.

Proposal 3: The inactive UE performs state transition to idle state upon receiving CN based paging.

Case 4: Network responds a release message to configure inactive UE into idle

In this case, the RRC re-activation request message is successfully received by the network. However, due to overload case, unsuccessful context fetch procedure or other network strategy, the network finally decides to send a release message as a response to the inactive UE. In this way, the inactive UE will also enter idle according to the network configuration.

Fig. 2 Re-activation Reject procedure

Proposal 4: The network can configure the inactive UE into idle as a response to re-activation request.

# Conclusion

By analysing the detailed cases and potential solutions for UEs in inactive state to transit into idle state, we made the following proposals in NR:

Proposal 1: The inactive UE performs state transition to idle state when the inactive UE fails to send the reactivation message.

Proposal 2: The inactive UE performs state transition to idle state when the UE cannot find the suitable cell for a period of time.

Proposal 3: The inactive UE performs state transition to idle state upon receiving CN based paging.

Proposal 4: The network can configure the inactive UE into idle as a response to re-activation request.

# Reference

- R2-1706736 Discussion on CN location Update and RNA Update for inactive state, Huawei, HiSilicon