3GPP TSG-RAN WG2 Meeting NR AH#2		R2-1706760

Qingdao, China, 27–29 June 2017

Agenda Item:	10.4.2.2

Source: 	MediaTek Inc.

Title:  	Cell Selection and Reselection in NR

Document for:	Discussion and decision

# Introduction

In RAN2#97, we made the following agreements about idle/inactive mobility in NR.

1 For cell reselection, cell quality can be derived from N best beams where value of N can be configured to 1 or more than 1. FFS: Details of filtering to be applied (e.g. for the case N=1, the best beam is filtered by a single filter as the best beam changes) FFS: Whether to only consider beams above a threshold ('good' beams)

In this contribution, we discuss the further details, including the remaining FFS issues, of cell selection and reselection in standalone NR.

# Discussion

## Background

Cell selection and reselection in LTE can be outlined as follows. The UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".  The suitability of a cell is determined by the cell S-criterion:

Srxlev > 0 AND Squal > 0, where Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation Squal = Qqualmeas – (Qqualmin + Qqualminoffset)

If the UE finds a more suitable cell, it reselects to that cell and then camps on it. Cell reselection involve the measurement rules and reselection criteria.

Measurement rules:

- If Srxlev > SIntraSearchP and Squal > SIntraSearchQ, UE may choose not to perform intra-frequency measurements. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority. - UE shall always perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies.

Reselection criteria: Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection (Equal priorities between RATs are not supported)

- The cells is ranked according to the R criteria (s: serving, n: neighbour): - Rs = Qmeas,s + Qhysts - Rn = Qmeas,n + Qoffsets,n - If a cell is ranked as the best cell and the following conditions are met, the UE shall perform cell reselection to that cell. - More than 1 second has elapsed since the UE camped on the current serving cell. - The new cell is better ranked than the serving cell during a time interval TreselectionRAT.

## Cell selection

For cell selection, the UE should be able to determine the cell to camp on without the configuration from the network. Although a UE may observe multiple beams from a cell, it is hard for a UE to consider ‘N best beams’ since the ‘N’ value is unknown. There are some options, e.g.

- Fix the ‘N’ value in standard. The simplest way is to choose N=1.

- Leave it for UE implementation. The set of beam(s) to consider is determined by the UE, based on real-time channel conditions.

When performing cell selection, UE should camp on a suitable cell as fast as possible. That is, UE cares about both signal strength and latency. If the total number of beams is not too large, UE may search for the best beam for S criterion evaluation, Alternatively, UE may camp on a cell immediately when one or multiple beams in a cell meets the suitability criteria, and switches to a better beam or reselect to a better cell later, if needed. Therefore, we prefer leave the details for UE implementation.

Proposal 1:	The suitability criterion (S criterion) is evaluated based on NR cell quality.

Proposal 2:	Beam measurement to determine cell quality for suitability criterion evaluation is left for UE implementation.

## Cell reselection

For cell reselection in NR, we believe that measurement rules to skip measurement on neighbouring cells should be applied, and the reselection is based on NR cell quality and cell ranking, as in LTE.

Proposal 3:	For cell reselection in NR, measurement rules to skip measurement on neighbouring cells should be applied.

Proposal 4:	Cell reselection is based on NR cell quality and cell ranking, as in LTE.

### Measurement rule

After camping on a cell, UE determines whether to perform intra-frequency measurement and inter-frequencies or inter-RAT frequency cells of equal or lower priority, based on the measurement rules. Since UE is already camping on a cell, more sophisticated and precise cell quality derivation can be applied. We suggest that the cell quality used for measurement rule evaluation be derived by averaging ‘N best beams’, as in connected mode.

Proposal 5:	The cell quality used for measurement rule evaluation is derived by averaging ‘N best beams’, as in connected mode.

### Reselection criteria and cell ranking

For connected-mode mobility, a cell can be evaluated based on measurements on multiple beams, which mitigates the Ping-Pong effect. Unlike connected-mode, that an idle-mode UE switches back and forth between two cells does not bring signalling overhead and service interruption. However, considering multiple beams does help the UE reselect to a more preferable cell. Therefore, we suggest that the cell quality used for R criteria evaluation and cell ranking for cell reselection be derived by averaging ‘N best beams’, as in connected mode.

Proposal 6:	The cell quality used for R criteria evaluation and cell ranking is derived by averaging ‘N best beams’, as in connected mode.

Similar to the situation in connected mode, sometimes not all N beams are strong enough to be considered in cell ranking, or the UE simply cannot detect N beams. Therefore, a threshold may be set and only beams above such a threshold is considered in the evaluation of a cell for cell ranking.

Proposal 7:	The network may configure a threshold so that only beams above the threshold is considered in cell quality derivation for cell reselection.

# Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1:	The suitability criterion (S criterion) is evaluated based on NR cell quality.

Proposal 2:	Beam measurement to determine cell quality for suitability criterion evaluation is left for UE implementation.

Proposal 3:	For cell reselection in NR, measurement rules to skip measurement on neighbouring cells should be applied.

Proposal 4:	Cell reselection is based on NR cell quality and cell ranking, as in LTE.

Proposal 5:	The cell quality used for measurement rule evaluation is derived by averaging ‘N best beams’, as in connected mode.

Proposal 6:	The cell quality used for R criteria evaluation and cell ranking is derived by averaging ‘N best beams’, as in connected mode.

Proposal 7:	The network may configure a threshold so that only beams above the threshold is considered in cell quality derivation for cell reselection.

# Reference

- RP-160671, New SID Proposal: Study on New Radio Access Technology

- TR 38.913,  Study on Scenarios and Requirements for Next Generation Access Technologies