3GPP TSG-RAN WG2 NR Adhoc #2 															R2-1706835

Qingdao, China, 27 - 29 June 2017

Agenda item:		10.2.8

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Cell Identification in NR

WID:						NR_newRAT-Core - Release 15

Document for:		Discussion and Decision

# 1	Introduction

In LTE, the synchronization is based on Primary Synchronization Signal (PSS) and Secondary Synchronization Signal (SSS). For NR, RAN1 has considered that synchronization would also be based on PSS and SSS. The current assumption is that there would be three PSS sequences for initial downlink synchronization and number of SSS sequences would be increased from LTE so that total number of combinations would be 1000 compared to 504 combinations available in LTE. In addition to the synchronisation, these 1000 values could be used as Physical Cell ID (PCI) for measurement reporting and other procedures to identify cell in NR.

In this contribution, we discuss future needs of cell identification and whether LTE based approach with 1000 PCI values is sufficient for cell identification in NR.

# 2	Discussion

## 2.1	Cell IDs in LTE

In addition to the Physical Cell ID (PCI) in LTE with 504 values that are obtained from PSS/SSS with 5ms periodicity, following identities are used to identify different parts of the network in radio protocols. All these parameters are transmitted in SIB1 with 80ms periodicity with 20ms repetition:

-	PLMN Identity List, which is containing following elements with maximum size of 6 entries

-	PLMN identity (24bits)

-	Cell reserved for operator use (1 bit)

-	The global Cell ID, containing

-	PLMN Identity (24bits)

-	Cell ID (28bits).

-	Tracking area code, transmitted in SIB1 (16bits)

-	Used for NAS protocol for tracking area management. Not used by RRC or other radio protocols.

-	CSG Identity, transmitted in SIB1 (27bits)

Notably, RAN1 has agreed that PSS and SSS signals are transmitted together with PBCH containing MIB, in so called SS-block. In SS-block, the PSS and SSS would utilize one symbol each and PBCH would be time multiplexed with PSS and SSS occupying additional symbols. RAN1 has agreed that for initial cell selection UE can assume 20ms transmission period of SS-Block, the actually used SS burst set periodicity, ranging from 5ms to 160ms, will be configurable. Even though, final design of SS-Block is not completed, the number of information bits in PBCH is expected to be quite limited. Based on the LS sent by RAN1, RAN1 informed that based on the performance evaluation done so far, the range for the upper limit for the payload size is between 48 and 72 bits (including CRC).

Even though RAN2 has not discussed different network identities and corresponding sizes, above set of IDs could be assumed as good initial estimation. Possible RAN identifiers are further discussed in [7]. Thus, it becomes clear that PLMN identity list and Global Cell ID would not fit into PBCH being transmitted in SS-Block.

## 2.2	Scenario and requirements

In NR, the system should be able to provide significantly more capacity. Many times, this has been referred as 10 000x more capacity. This will require very small cells and typical ISD of 500m in dense urban deployments will shrink significantly especially in frequency layers providing high capacity. Additionally, majority of the new spectrum will become available at high frequencies that will drive towards small cells with high penetration loss from walls and other shadowing objects. However, supporting high speed scenarios for e.g. trains and cars is important and thus overlay coverage layer will introduce large cells potentially overlapping highly dense areas, resulting significant mixture of different cell sizes.

If we consider some very high dense areas such as Manhattan New York, we could calculate worst-case estimations. One worst case estimation, could indicate that we could have 20 macro cells/km2 and up to 40 000-80 000 small cells/km2 with average size of 100m2 or 50m2 respectively. The estimation is based, that there is 4km2 indoor space for 1km2 of outdoor land, with average of 8 floors in each building and 50% of land is used for buildings.

Even though, above calculation is very simple and not directly reflecting to any real network deployment, it gives us rough view of future networks that we might foresee. Deploying such high number of small cells would require extremely flexible deployment supporting at least:

-	Extensive automatic configuration and adaptive deployment.

-	Self backhauling – power cable or big battery is sufficient for NR eNB deployment

-	Network power efficiency resulting cells without or very limited periodic system information broadcast

-	Network topology changing and adapting to the capacity needs

Additionally, due to spectrum sharing and unlicensed spectrum, multiple operators may start sharing same spectrum resulting that:

-	Certain spectrum may have cells that UE is not able to access due to PLMN restrictions. This can be seen similar to the CSG-cells operations.

-	Certain cells in certain location are reserved for specific service slice.

Observation 1: In future NR networks, very high number of cells can be expected, especially on high carrier frequencies.

## 2.3	Network controlled mobility

In the network controlled mobility, the handover decision is done based on UE measurement reports. For correct handover decision, the NW needs to be able to unambiguously associate measurement report to the cell and possible beam. If this is not possible the network may prepare HO to wrong neighboring cell. This problem is known in LTE as “PCI confusion” where this association is compromised as multiple neighboring cell use same PCI value. Typically, this occurs when large macro cell operates with high number of small cells.

In LTE, to develop association between used PCI and Global Cell ID, Automatic Neighboring Relation feature was used, where the E-UTRAN is requesting set of UEs to read SIB1 and report global Cell ID with the PCI value to update PCI to Global Cell-ID relation.

This operation is not aimed to be constant operation but only if PCI values and/or deployment is modified there is need for new ANR process. The SIB1 reading reduces HO performance due to extra delay caused by SIB1 reading as SIB1 is transmitted less often and UE can decode it less reliable than PSS/SSS. As the solution only introduces relation between PCI value and Global Cell-ID, the solution does not help in case that multiple neighboring cells (inter frequency small cells) of the macro cell are using same PCI value. Thus, this solution does not solve future very dense deployments nor situation where multiple operators use same frequency without any PSS/SSS signal coordination.

In CoMP based mobility, the requirement is very similar as the NW should be able select correct beams from different cells or transmission points. Beams are hidden behind antenna ports as each beam has own antenna port making TRP invisible for the UE. Beams transmitted from different cells compared to beams provided from other TRPs from same cell do not differ as different cells are just using different synchronization signal sequences. Nevertheless, the network needs to have means to select appropriate limited number of antenna ports for UE reporting before actual CoMP transmission as transmitting CSI-RS from all possible ports for all the UEs is not feasible.

If NW controlled mobility is based on uplink SRS transmission and network’s capability of selecting best cell based on these UL transmission, we believe that NW should have good pre-understanding of which cells/TRPs the reception of uplink SRS transmission should be enabled. This would allow reusing time and frequency resources between UEs which is essential to make scheme scalable. Naturally, network can utilize the information from current serving cell but when considering deployments discussed above, UE reporting cells that it can detect would be beneficial.

Based above discussion we make following observation:

Observation 2: Large Cell ID space, which can support fast acquisition and provide unambiguous cell identification without SIB1 reading, would be beneficial for NW controlled mobility in NR.

## 2.4	UE based mobility

The UE based mobility is envisioned to be used in NR for two reasons. The first reason is UE cell re-selection in “power save” mode, i.e. RRC Connected Inactive mode and the second reason is fast cell change in high operating frequencies.

The UE cell reselection in RRC Connected Inactive state would require scheme with following characteristics:

-	Fast and simple cell search and measurement

-	Detecting all possible candidates fast.

-	Obtain reliable measurement with MIB and possible SIB reading

-	Read MIB and possible SIB only when needed and signal quality is strong enough.

- Robust evaluation reselection scheme

-	Avoiding re-selecting cell where access is denied (barred, wrong PLMN, not supporting UE BW or service)

-	Short interruption time as interruption will cause delay in U-plane latency.

If system information of the neighboring cell is provided in advance by dedicated signaling the UE should be able to identify cell unambiguously to decide which configuration to use and whether to update location to NW. If this can be done without system information reading after cell reselection both interruption time as well as UE power consumption would be optimized.

The UE based fast cell change on high frequencies is needed due to high shadowing loss at those frequencies. Due to high shadowing loss NW controlled handover will face problems due to delays of sending measurement report, HO preparation and sending HO command as serving cell can disappear completely at the point when neighbor is being detected by the UE. Thus, when all downlink channels and uplink RACH are beamformed the mobility becomes UE based beam management where UE indicates best beam out of pre-defined set of beams which can be either intra or inter cell beams.

To enable UE to perform RACH fast and perform fast cell change the network should provide system information of possible neighboring cells in advance and UE should unambiguously identify neighboring cell, to be able to utilize correct system information when detecting a beam. If unambiguous cell identification and thus access to the new cell would require system information reading beforehand, the cell change would not be fast and possible interruption time to the user service could be significant.

Based on above discussion we make following observation:

Observation 3: Large Cell ID space, which can support fast acquisition and provide unambiguous cell identification without SIB1 reading, would be beneficial for UE based mobility.

## 2.4	When UE needs to identify a Cell and beam

As in LTE the physical cell ID is tight to the PSS and SSS sequences it is worth considering at which point the UE really needs to identify cell unambiguously. In our view the UE does not need identify cell in synchronization phase as the UE just finds synchronization signals without knowing which cell or network element is sending this signal. Synchronization signals can be transmitted in SFN manner, and UE sees these as strong multipath and if signal components become outside of CP, those components introduce ISI to the synchronization signals.

Similarly, in measurement phase it is not essential for the UE to identify the cell unambiguously. UE can perform RSRP/RSRQ measurements without identifying actual cell. The negative impact of the requirement that UE needs to acquire MIB from all detected cells is further discussed in [3]. Only high enough RSRP values could be considered further for identification. However, if there are multiple networks in same frequency and UE cannot access all of those (PLMN with no roaming; not supported system bandwidth or service slide, barred cell), it would be beneficial for the UE to exclude fast those cells from measurements.

Therefore, the large Cell ID does not necessarily have to be decoded from very weak cell that are not considered as candidate cell, additional efforts can be rather done only after cell is close to measurement reporting phase criteria in network controlled mobility or UE based cell reselection or fast re-establishment.

There seems to be no need to define criteria when UE may initiate this additional effort, it could be rather left up to UE implementation, as long as large cell ID is obtained in advance in specific cases. These specific cases can be configured in measurement control by defining whether extension needs to be obtained for measurement report or not. Similarly UE accessing to the cell by using pre-configuration would verify extended PCI from MIB.

Based on above discussion we propose following:

Proposal 1: NR specification shall support extended physical layer cell ID.

Proposal 2: Physical layer cell ID shall be able to be acquired fast and power efficient manner without SIB reading, for both network controlled and UE based mobility scenarios.

To meet fast acquisition requirement we believe that extended cell ID should be provided in MIB as part of SS-block. The MIB would include additional bits to extend PSS and SSS value further. The number of additional bits should be considered in conjunction with other information to be included in MIB, but we believe that including 6 bits for extension would be possible, without jeopardizing other IEs or possible MIB size [5]. This would allow extension of PCI to be 1000*2^6 = 64000, matching the requirement discussed in Section 2.2.

From RAN2 point of view, it would be important to recognize the benefits of large extended physical cell ID size and consider it in MAC and RRC procedures.

Proposal 3: Agree to include extension of PCI (extend SSS/PSS cell identities) in MIB by [6bits TBD].

Proposal 4: Send an LS to RAN1 to inform RAN2 agreements on proposal 1, 2 and 3.

# 3	Conclusions

In this contribution, we have discussed future needs of the physical layer cell ID for NR. Based on discussion we make following observations:

Observation 1: In future NR networks, very high number of cells can be expected especially on very high carrier frequencies.

Observation 2: Large Cell ID space, which can support fast acquisition and provide unambiguous cell identification without SIB1 reading, would be beneficial for NW controlled mobility in NR

Observation 3: Large Cell ID space, which can support fast acquisition and provide unambiguous cell identification without SIB1 reading, would be beneficial for UE based mobility.

Additionally, we make the following proposals:

Proposal 1: NR specification shall support extended physical layer cell ID.

Proposal 2: Physical layer cell ID shall be able to be acquired fast and power efficient manner without SIB reading, for both network controlled and UE based mobility scenarios.

Proposal 3: Agree to include extension of PCI (extend SSS/PSS cell identities) in MIB by [6bits TBD]

Proposal 4: Send an LS to RAN1 to inform RAN2 agreements on proposal 1, 2 and 3.

# References

- RP-160671, “Study on New Radio Access Technology”, NTT DoCoMo

- TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, 3GPP

- R1-1705840 “NR-PBCH Design”, Nokia, Alcatel-Lucent Shanghai Bell, RAN1 #88bis

- R1-1705708	“Discussion and evaluation on NR-PBCH design”	DOCOMO, RAN1 #88bis

- R1-1705321	“NR-PBCH design”	Samsung, RAN1 #88bis

- R2-1704287 “PBCH reading for measurement purposes”, Nokia, Alcatel-Lucent Shanghai Bell, RAN2 #98

- R3-17xxxx “NG RAN identifiers”, Nokia, Alcatel-Lucent Shanghai Bell, RAN3 #96