3GPP TSG-RAN WG2 NR Adhoc #2 															R2-1706840

Qingdao, China, 27 - 29 June 2017

Agenda item:	10.4.2.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Cell selection/reselection basics for NR IDLE – cell quality derivation

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

Durin the study phase of NR following agreements were reached regarding reselection handling in NR as capture din 38.804 v1.0.0:

#### 10.1.1.1	Cell reselection

The following cell reselection methods as specified in TS 36.304 [15] are applicable based on the corresponding parameters broadcast while the UE is camping on a cell in NR:

-	Intra-frequency reselection is based on ranking of cells.

-	Inter-frequency reselection is based on absolute priorities.

-	Inter-RAT reselection can be also based on absolute priorities.

-	Frequency specific cell reselection parameters common to all neighbouring cells on a frequency;

-	Service specific prioritisation;

NOTE 1:	For NR, it is FFS for which services the service specific prioritisation is applied and how it could be applied for the case of network slices.

-	A concept of neighbour cell lists and black cell lists;

-	Speed dependent cell reselection.

In multi-beam operations, measurement quantity of a cell is derived from N best beams corresponding to the same cell where the value of N can be configured to 1 or more than 1.

NOTE 2:	It is FFS on details of filtering to be applied (E.g. for the case N = 1, the best beam is filtered by a single filter as the best beam changes) and whether to only consider beams above a threshold (good beams).

Additionally for the need to support reselection behavior both in IDLE and INACTIVE states.

In this paper we give a view how to proceed to progress reselection handling in the RAN2 specifications  namely 38.304 and 38.331.

Additionally for cell selection following agreements were made:

## 9.1	Cell selection

Cell selection is performed by one of the following two procedures:

a)	Initial cell selection (no prior knowledge of which RF channels are NR carriers);

1.	The UE shall scan all RF channels in the NR bands according to its capabilities to find a suitable cell.

2.	On each carrier frequency, the UE need only search for the strongest cell.

3.	Once a suitable cell is found this cell shall be selected.

b)	Cell selection by leveraging stored information.

1.	This procedure requires stored information of carrier frequencies and optionally also information on cell parameters, from previously received measurement control information elements or from previously detected cells.

2.	Once the UE has found a suitable cell the UE shall select it.

3.	If no suitable cell is found the Initial Cell Selection procedure shall be started.

The following three levels of services are provided while a UE is in RRC_IDLE:

-	Limited service (emergency calls, ETWS and CMAS on an acceptable cell);

-	Normal service (for public use on a suitable cell);

-	Operator service (for operators only on a reserved cell).

The definition of an acceptable cell, a suitable cell, a barred cell and a reserved cell is also applicable for cell selection in NR. A cell is considered as suitable if the following conditions are fulfilled:

-	Measurement quality of a cell is above a threshold;

-	A cell is served by the selected/registered PLMN and not barred.

NOTE:	Other conditions are FFS if any.

In multi-beam operations, measurement quantity of a cell is derived amongst the beams corresponding to the same cell. It is FFS how to derive the cell level measurement quantity from multiple beams, which may or needs not be different for the one in RRC_CONNECTED.

# 2	Cell quality derivation

For multi-beam operation for reselection there was agreements in the TR:

In multi-beam operations, measurement quantity of a cell is derived from N best beams corresponding to the same cell where the value of N can be configured to 1 or more than 1.

NOTE 2:	It is FFS on details of filtering to be applied (E.g. for the case N = 1, the best beam is filtered by a single filter as the best beam changes) and whether to only consider beams above a threshold (good beams).

On the other hand for cell selection:

In multi-beam operations, measurement quantity of a cell is derived amongst the beams corresponding to the same cell. It is FFS how to derive the cell level measurement quantity from multiple beams, which may or needs not be different for the one in RRC_CONNECTED.

It would seem beneficial to simplify to use same rules for measurements for cell selection and reselection thus we propose to use the one agreed for reselection

Proposal: Measurements done for cell selection and reselection in case of multi-beam operation, measurement quantity of a cell is derived from N best beams corresponding to the same cell where the value of N can be configured to 1 or more than 1

To perform reliable quality measurements and prevent too frequent re-selections the key principles of LTE measurements could be reused such as filtering of L1 measurement samples, Treselection, threshold values with respective offsets/hysteresis.

Figure 1 proposes the measurement model to derive the IDLE/INACTIVE cell quality based on measurements on SS-blocks. The used measure may be e.g. RSRP/RSRQ. The SS-block-RSRP measurement is done on at least NR-SSS signal (RAN1 agreement)

SS-block Quality for Intra-cell Selection

- SS-block RSRP measurement is done on NR-SSS signal

- A cell may transmit multiple SS blocks and different SS blocks can be identified based on a time index (RAN1 agreement)

- In case of intra-cell measurement:

  - UE selects the e.g. the strongest SS-block based on the filtered measurements and performs SS- block selection according to configured criteria.

  - Same filtered samples are provided to selection function which determines the current cell quality UE is camping from N strongest SS-blocks.

- Layer 2 filtering (e.g. moving average) is  applied for  SS-block L1 measurements to determine individual SS-block quality. L2 filtering is performed at lower layer. The purpose of L2 filter is to stabilize the measurements so that the SS-block selection and cell quality derivation is done on stable measurement results. Filter parameters are configured by network.

- In case of inter-cell measurement:

  - L2 filtering is applied for the SS-block measurements but in case of inter cell measurements only cell quality is evaluated.

Cell Quality for Re-selection

- Selection function is run on provided SS-block measurements to determine the cell quality value

- Select the N highest filtered SS-blocks quality values to represent the cell quality, apply further network configured filtering to derive cell level quality for L3 evaluation of a re-selection criteria

Let’s then assume that L2 filtering is invisible to the IDLE mode specification. As have been agreed one would use N-best beams from lower layer filtered results to determine cell quality. It seems that straightforward it is to assume averaging of N-best beams. Of course some other method can be used as well but even those should enable basic averaging of N-best beams.

Proposal: Use averaging of N-best beams to determine cell quality. If other methods are defined is FFS

So from L3 measurements point of view we have following functions:

- Selecting N-best beams from lower layer filtered results

- Averaging measurements

- Filtering results

Now one needs to then consider when L3 filtering done – averaging lower layer filtered results before L3 filtering or doing L3 filtering before averaging? Let’s assume simple scenario:

- UE hears beams 1, 2 and 3

- Due to radio condition change UE hears beams 3, 4 and 5

How does averaging work in this scenario? At least there is two basic ways to consider:

- Whenever any of N-best beams change L3 used measurements is started from scratch

- Whenever any of N-best beams change it is not seen by L3 filtering but only the result of beam selection/averaging is seen by L3 filter

It would seem unfortunate to start L3 filtering from scratch – beams are probably changing quite frequently and then if we always start the filtering we would basically lose all the benefits of filtering.

Proposal: For each measured cell N-best beams are selected and averaged. L3 filtering (if any) is run on this averaged result.

In LTE UE performs at least two L1 measurements spaced by at least DRX cycle/2 and the filtering defined in 36.133 would be just sort of an average of these two measurements. There is not really any higher layer filtering on top of this in L3 for reselections.

In NR one would need to consider if there is need to additional L3 filtering as well – Especially in higher frequencies it might be needed due to extremely fast fading characteristics of the channel. Two stages of filtering provides network means to configure UE behaviour in IDLE/INACTIVE mode when selecting SS-block / re-selecting a cell. But if L3 filtering is really needed requires careful studies thus we propose:

Proposal:  Leave it FFS if L3 filtering is required for cell selection/reselection measurements.

Figure 1 Measurement Model for IDLE/INACTIVE mode in NR

# 3	Conclusion

In this paper we provided various views on IDLE/INACTIVE mode cell selection/reselection modeling as well as how actual cell quality results are derived from lower layer samples and possible need for L3 filtering of those lower layer results:

Regarding discrepancy between cell selection/reselection cell quality quantity:

Proposal: Measurements done for cell selection and reselection in case of multi-beam operation, measurement quantity of a cell is derived from N best beams corresponding to the same cell where the value of N can be configured to 1 or more than 1

Regarding dervatino of measurement results for calculating cell selection/reselection quantities:

Proposal: Use averaging of N-best beams to determine cell quality. If other methods are defined is FFS

Proposal: For each measured cell N-best beams are selected and averaged. L3 filtering (if any) is run on this averaged result.

Proposal:  Leave it FFS if L3 filtering is required for cell selection/reselection measurements.

# References

- 3GPP TS 21.801 Drafting Rules