3GPP TSG-RAN WG2 NR Ad Hoc			     R2-1706852

Qingdao, China, 27th – 29th June, 2017

Agenda item:		10.3.4.2 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	SDAP PDU format

Document for:	Discussion and Decision

1.	Introduction

According to TR 38.804, the main services and functions of the SDAP layer, which is newly defined for the NR QoS framework, include:

- Mapping between a QoS flow and a DRB;

- Marking QoS flow ID in both DL and UL packets.

This document discusses SDAP PDU format used for NR QoS framework.

2.	Discussion

QoS flow is the finest granularity of QoS differentiation for the NR QoS framework, and is identified by QoS Flow ID. Traffic with the different QoS Flow ID value within a PDU session requires the different traffic forwarding treatment. For such different traffic forwarding treatment, UL QoS flow ID is sent in SDAP header with UL packet so that gNB can determine the QoS marking which is carried in an N3 encapsulation header of the UL packet. The gNB and UPF are going to treat the received UL packet based on the QoS marking (QoS Flow ID) in the N3 encapsulation header. Therefore, UL SDAP PDU should be used to convey UL packet and QoS flow ID associated with the UL packet.

Contrary to the uplink, DL QoS flow ID does NOT need to be sent for per-QoS flow differentiation. It’s because the traffic forwarding treatment on the radio interface is done on a per DRB basis. However, DL QoS flow ID can be included in the SDAP header when AS and/or NAS-level reflective QoS is triggered. Therefore, DL SDAP PDU should be used to convey DL packet, QoS flow ID and indicator indicating whether or not each of NAS and AS level reflective QoS is triggered.

Observation 1: UL SDAP PDU can be used to convey UL packet and UL QoS flow ID for per-QoS flow differentiation, whereas DL SDAP PDU can be used to convey DL packet, QoS flow ID and Indicator for AS and NAS level reflective QoS.

Proposal 1: Define SDAP PDU for UL and DL, respectively.

3.	Proposal

In this document, we present our view on SDAP PDU format. We have following proposal:

Proposal 1: Define SDAP PDU for UL and DL, respectively.