3GPP TSG-RAN WG2 NR Ad hoc#2                                                     			 R2-1706939

Qingdao, China, 27th – 29th June 2017										update of R2-1704694

Agenda item:	10.2.8

Source:	Samsung

Title:	Operational aspects of conditional handover mechanism

Document for:	Discussion and Decision

- Introduction

From RAN2#97bis, it has been agreed to study the conditional handover in NR as follows:

Agreements

=>	We will progress the basic HO mechanism for NR and when stable we can discuss whether to support conditional handover and discuss other potential optimisations.

And in RAN2#97, there was also a discussion on this and got the agreement for the continuation of the discussion as following:

Agreement 'Conditional' handover can continue to be discussed within the WI phase

Even though in the design perspective there would be no concrete agreement soon since baseline HO mechanism for NR is not completed, still discussion can be possible to get common sense on the conditional handover. For obtaining this, we show different aspects of the signalling overhead and handover failure rate (HFR) in comparison with LTE case.

- Basic call flow and events used therein

The motivation of the conditional HO is mainly for overcoming the fast link drop which consequently incurs unsuccessful transmission of Measurement Report from UE or Handover command from sourcegNB, respectively. The result of the unsuccessful transmission of MR and HO CMD is that UE should go through the RLF recovery procedure. The possible solution can be thought of direct handover to the possible target cell without any delay and for this, there must be a procedure to get those target cells a priori.

The following figure 1 is the basic call flow. The Phase 1 is running continuously until Phase 2 is triggered. Once Phase 2 is triggered, UE might go to the target cell and get configured newly again.

Figure 1Basic call flow of conditional HO

We need two kinds of events for each phase. Phase 1 is for maintaining candidate target cell list while Phase 2 is for actual HO execution. For this experiment, we adopt A4 and A5 for each event respectively.

For Phase 1, the measurement report triggering event for adding candidate target cells is A4 of which entering condition is neighbour cell is greater than a given signal quality (absolute value). The measurement report triggering event for removing a cell from candidate target cells is the operation based on A4 reportOnLeave configuration, i.e., once those triggered target cell is leaving after TTT, UE triggers to send MR.

The basic idea behind adopting absolute value based event for collecting candidate target cells is to reduce MR overhead by controlling the target cell link quality level without the consideration of the serving cell’s link quality, since if adopting relative value based event i.e., A3 we cannot control the number of MRs. In detail, once the serving cell link quality drops the almost every neighbour cell will be triggered for A3 (even there might be some delays among triggering) and there is no room to control the MR overhead. By configuring high threshold for A4, MR overhead must be decreased while low threshold would make increasing MR overhead.

Observation1. Absolute value based measurement event has more room to control the MR and network signalling overhead than the relative value based event.

For Phase 2, the HO execution event is A5 in our simulation setup. A5 event is triggered when serving cell link quality is less than given threshold and neighbour cell link quality is greater than another given threshold. And we also consider the situation where A3 and A5 are applied for Phase 2 event simultaneously for comparing the performance. Note that the target cell is one of the candidate target cells, i.e., one of the candidate target cells meets A3 or A5, then UE would execute conditional handover.

Regarding above observation 1, we showed the simulation results last RAN2#98 [1]. And the following table summarizes this.

Table I. A5 (serving cell threshold: -87 dBm, and target cell threshold: -87 dBm) for Phase 2 event

|  | A4 entering condition threshold |  |
| --- | --- | --- |
|  | -87 dBm | -80 dBm |
| Handover failure rate | 1.31 % | 7.98 % |
| # of addition MR per sec | 3.47 | 1.11 |
| # of average candidate target cells | 4.58 | 3.19 |

Table II. A3 (offset value is 3 dB) and A5(serving cell threshold: -87 dBm, and target cell threshold: -87 dBm dB) for Phase 2 event

|  | A4 entering condition threshold |  |
| --- | --- | --- |
|  | -87 dBm | -80 dBm |
| Handover failure rate | 0.58 % | 3.69 % |
| # of addition MR per sec | 4.5 | 1.78 |
| # of average candidate target cells | 5.45 | 3.67 |

It gives the information that HFR  on using conditional HO in HF could be lower than that on using LTE algorithm in HF. However the number of additional MR per sec is increased because of candidate cells management at the same time. Increasing number of MR message is obviously a system overhead. Therefore  it is necessary to control this.

On analysing the performance factor of conditional HO in HF itself shown in [1], the lower A4 threshold is, the lower HFR is and the higher the MR overhead is. Since lower A4 threshold makes more candidate cells prepared, the MR would be needed more whereas the hit ratio of the actual target cell within candidate target cells is increasing.

Observation2. There is a trade-off relationship between HFR and Number of MR per sec, which can be jointly controlled by A4 threshold value.

To solve this trade-off problem, RAN2 is requested to study the conditional HO design with considering number of MR and HFR performance.

Proposal 1. RAN2 is requested to design the conditional HO mechanism by considering the number of MR and HFR performance trade-off.

3. Conclusion

In this contribution, the conditional HO mechanism is studied in the overhead and HFR perspective. The following observations and proposal could be derived:

Observation1. Absolute value based measurement event has more room to control the MR and network signalling overhead than the relative value based event.

Observation2. There is a trade-off relationship between HFR and Number of MR per sec, which can be jointly controlled by A4 threshold value.

Proposal 1. RAN2 is requested to design the conditional HO mechanism by considering the number of MR and HFR performance trade-off.

4. References

[1] R2-1704694, Performance of UE autonomous handover RAN2 #98, Samsung

[2] 3GPP TR38.900 Study on channel model for frequency spectrum above 6 GHz , V14.2.0 (2016-12)

[3] 3GPP TR 38.802 Study on New Radio Access Technology Physical Layer Aspects V14.0.0 (2017-03)

Annex A: Simulation parameters

The following table summarizes the simulation parameter shown in [1].

| Parameter | HF-NR |  |
| --- | --- | --- |
|  | Value | Ref. |
| Carrier frequency | 28 GHz | - |
| Bandwidth | 1 GHz | [3] |
| ISD | 200m | [3] |
| Number of sectors | 3 | [3] |
| Path Loss | 53.23 + 35.3 log10(R) | [2] |
| Shadowing Standard Deviation | 7.82 dB | [2] |
| Penetration Loss | 27.88 + N(0, σ) dB | [2] |
| BS TX Power | 33 dBm | [3] |
| UE TX Power | 23 dBm | [3] |
| BS Antenna Gain | 24 dBi | [3] |
| BS Antenna Element Gain | 8 dBi | [3] |
| UE Antenna Gain | 15 dBi | [3] |
| BS Antenna Height | 10m | [3] |
| Noise Figure | UL: 8 dB, DL : 11 dB | - |