3GPP TSG-RAN WG2 NR Ad Hoc		R2-1706974

Qingdao, China, 27th - 29th June 2017		(revised R2-1704585)

Source:	vivo

Title:	Discussion on other SI request

Agenda Item:	10.4.1.5.6

Document for:	Discussion and Decision

- Introduction

At the RAN2#98 meeting, the on-demand other SI request was discussed and the following agreements have been made [1]:

Agreements 1 For MSG1 based SI request, the minimum granularity of requested SI is one SI message (a set of SIBs as in LTE). 2 For MSG1 based SI request, one RACH preamble can be used to request for multiple SI messages. Agreements for On demand request for broadcast delivery 1 On demand SI request will maximise commonality with the RACH procedure 2 Network sends an acknowledgement in MSG2 to the UE’s SI request sent in Msg1 FFS Network sends an acknowledgement in MSG4 to the UE’s SI request sent in Msg3

According to the above agreement, it is unclear how to handle the case when UE wanted other SI have both requested by MSG1 and MSG3. Therefore, in this contribution, we further discuss the other SI request issues and provide some suggestions.

- Discussion

The UE can use MSG1 or MSG3 to request the other SI according to network configuration. However, it is unclear how to handle the case if the UE requires other SIs requested by both MSG1 and MSG3.  For instance, according to network configuration, the other SI block one (OSIB1) should be requested by MSG1 and other SI block two (OSIB2) should be requested by MSG3. The UE requires both OSIB1 and OSIB2.  In the following, the procedure for some solutions is provided to handle this case.

Solution 1:  UE requests other SI by MSG1 and MSG3 respectively

UE requests other SI via MSG1 and receives the other SI sent by gNB and then requests other SI via MSG3 or UE firstly requests the other SI via MSG3 and then requests other SI via MSG1. The procedure that UE firstly requests the other SI via MSG1 and then requests the other SI via MSG3 is shown in Figure 1.

Figure 1 UE request other SI using MSG1 and MSG3 respectively

Solution 2: UE request all other SI by MSG3

UE requests all other SI by MSG3 at one time. The procedure can found in Figure 2.

Figure 2 UE request all other SI using MSG3

Solution 3: UE request other SI by MSG1 and MSG3 in the same procedure

UE requests all other SI by MSG3. The procedure is shown in Figure 3.

Figure 3 UE request other SI using MSG1 and MSG3 in the same procedure

The above solutions are compared in the Table 1.

Table 1: The comparison of above solutions

| Solutions | Pros. | Cons. |
| --- | --- | --- |
| Solution 1 | Obey the minimum configuration strictly | Increase other SI acquisition delay |
| Solution 2 | UE can request the other SI at one time and other SI acquisition delay is low | UE need to ignore the minimum configuration for other SI request |
| Solution 3 | Obey the minimum configuration and other SI acquisition delay is low Saving preamble resource. |  |

According to the above discussion, we have the following proposal:

Proposal 1: When UE requests multiple other SIs which need to be requested in MSG1 and MSG3, the UE can request all SIs in one-time request procedure via MSG3.

- Conclusion

In this contribution, we mainly discussed the issues for other SI request. Base on the discussion, we have the following proposal:

Proposal 1: When UE requests multiple other SIs which need to be requested in MSG1 and MSG3, the UE can request all SIs in one-time request procedure via MSG3.

- References

- 3GPP RAN2 Meeting #98 Chairman Notes.