3GPP TSG-RAN WG2 NR Ad Hoc #2	R2-1706987

Qingdao, China, 27th – 29th June, 2017

Agenda Item	: 10.3.1.4.2 (NR_newRAT-Core)

Source	: LG Electronics Inc.

Title	: Potential impact of multiple/repetated preambles on RA

Document for	: Discussion and Decision

1.	Introduction

According to the TR 38.802, the descriptions regarding random access procedure have been addressed as follows:

8.2.1 Random access procedure

8.2.1.2 Procedure

RACH resource is also defined as a time-frequency resource to send RACH preamble. Whether UE needs to transmit one or multiple/repeated preamble within a subset of RACH resources can be informed by broadcast system information, e.g., to cover gNB RX beam sweeping in case of NO Tx/Rx beam correspondence at the gNB.

In this contribution, we’d like to discuss the potential impact of multiple/repeated preambles on RA.

2.	Discussion

In NR, during initial access, UE can detect multiple beams by monitoring SS blocks, i.e., perform DL beam sweeping, and UE selects a suitable/best gNB Tx beam based on the DL measurement.

If the gNB Tx/Rx beam correspondence is available, during random access procedure, UE can use both the gNB Tx and Rx beams based on the DL measurement and corresponding association. In this case, one preamble transmission for a single gNB Rx beam may be sufficient to successfully perform the RA procedure.

Proposal 1. In MAC layer, UE transmits only one RACH preamble within a single RA procedure if the gNB Tx/Rx beam correspondence is available.

However, if UE transmits one preamble for a specific gNB Rx beam within a single RA procedure in case gNB Tx/Rx reciprocity is available, it may increase the latency to initial access because UE does not know its suitable gNB Rx beams. For the performance of RA procedure, UE needs to transmit the multiple/repeated preambles for the full/partial gNB Rx beams within a single RA procedure in case gNB Tx/Rx correspondence is not available.

Observation. In MAC layer, UE can transmit multiple/repeated RACH preambles for the full/partial gNB RX beams within a single RA procedure if the gNB Tx/Rx beam correspondence is not available.

Based on the above observation, we’d like to discuss the MAC issues that may arise from the multiple/repeated preambles for the full/partial gNB Rx beams within a single RA procedure.

Unlike LTE, NR supports multiple numerologies and the region for PRACH transmission could be aligned to the boundary of uplink symbol/slot/subframe. Depending on the RAN1 decision, if necessary, RAN2 may need to discuss how to compute the RA-RNTI for the transmitted RACH preambles because the time unit of a PRACH resource could be different from LTE. However, regardless of the RA-RNTI, we should firstly discuss whether the MAC layer needs to transmit the multiple/repeated preambles for the full/partial gNB Rx beams. In this contribution, we assume that region for PRACH transmission is aligned to a subframe like as LTE.

We think that preamble repetition for gNB RX beam sweeping can be supported by PHY layer, so it is not necessary to consider it in MAC layer. Although the Tx/Rx beam correspondence is not available at gNB, the preamble repetition for the full/partial Rx beams also can be designed to follow the decision of PHY layer. Based on the design, the gNB can select the best gNB Rx beam of a UE by considering that the repeated preambles are transmitted from one UE. In this case, UE can receive a single RAR including a UL grant for the best gNB Rx beam.

However, if UE transmits multiple/different preambles for multiple gNB Rx beams within a single RA procedure, it can arise the RAR collision between UEs as shown in figure 1.

UE cannot exactly detect its RAR message although gNB successfully receives the preamble for a specific Rx beam without preamble collision between different UEs. As described in the figure 1, during initial access, two UEs can select the same gNB Tx beam based on DL measurement, and they transmit randomly selected multiple/different preambles on a subset of PRACH resources informed by the gNB. Here, UEs don’t need to know detailed information of gNB Rx beams. The gNB can successfully detect each preamble for the Rx beams, but according to the current RA procedure, it leads to RAR collision because it is not possible to distinguish RAR message between different beams.

In order to support multiple/different preambles within a single RA procedure, a new scheme should be defined in MAC layer, for example RA-RNTI allocation per Rx beam or indication of beam index in RAR.

Proposal 2. RAN2 should discuss whether UE needs to transmit the multiple/different RACH preambles within a single RA procedure if the Tx/Rx beam correspondence is not available at gNB.

Figure 1. An example of RAR collision between different UEs

3.	Conclusion

In this contribution, we discuss the multiple/repeated RA preamble transmission, and our observation and proposals are as follows:

Proposal 1. UE transmits only one RACH preamble within a single RA procedure if the Tx/Rx beam correspondence is available at gNB.

Observation. UE transmits multiple/repeated RACH preambles for the full/partial gNB RX beams within a single RA procedure if the Tx/Rx beam correspondence is not available at gNB.

Proposal 2. RAN2 should discuss whether UE needs to transmit the multiple/different RACH preambles within a single RA procedure if the Tx/Rx beam correspondence is not available at gNB.