3GPP TSG-RAN WG2 NR Ad Hoc #2		R2-1707064

Qingdao, China, 27th - 29th June 2017		(revised R2-1704575)

Source:	vivo

Title:	BSR trigger due to QoS flow in NR

Agenda Item:	10.3.1.6

Document for:	Discussion and Decision

- Introduction

After the RAN 98 meeting, one email discussion was left for the BSR trigger. There are there cases for BSR trigger due to QoS flow in NR. In this contribution, we further discuss the rational of BSR trigger for QoS flow in NR.

- Discussion

  - The BSR trigger due to Qos flow

There are three cases for BSR trigger due to Qos flow in NR

Case 1: The new BSR trigger for new QoS flow on default bearer [2].

Case 2: The new BSR trigger for new QoS flow on all bearers [1]

Case 3:  The new BSR trigger for QoS flow remapping [3]

For remapping case, we consider that the network can generally know this buffer size without any new trigger because the remapping is implemented by the gNB. Anyway the gNB can schedule the UE if there are data still in the buffer.

For default DRB or all DRBs, We prefer that the new trigger can apply to all DRBs because generally the network can allocate the service to the suitable DRB. It is not logical that the arrival of the new QoS flow data in default bearer is more urgent than the arrival of the new QoS flow data in other bearers. For normal case, the important services should be allocated with the dedicated DRB.

Some companies also argue that as long as the flows are mapped on the same DRB, RAN only has to provide the same treatment to them. However, LCG number of BSR is only extended to 8, it is hard to map all diversity QoS flows to limit LCG.

Proposal 1: The new BSR trigger can apply to all the bearers. The network should configure which new QoS flow can trigger the BSR, even if the logical channel on which the new QoS flow is carried has the same priority with the existing logical channel.

- Conclusion

In this contribution, we further discuss the rational of BSR trigger for QoS flow in NR. And provide below proposals.

Proposal 1: The new BSR trigger can apply to all the bearers. The network should configure which new QoS flow can trigger the BSR, even if the logical channel on which the new QoS flow is carried has the same priority with the existing logical channel.

- References

- R2-1704575 BSR trigger for Qos flow in NR	vivo

- R2-1702638	 BSR and QoS Flows	Nokia, Alcatel-Lucent Shanghai Bell	discussion

- R2-1704664 Consideration on the triggering of BSR 	ZTE Corporation