3GPP TSG-RAN WG2 #98 AH	Tdoc R2-1707151

Qingdao, P.R. of China, 27th – 29th June 2017

Agenda Item:	10.2.2.5

Source:	Ericsson, Vodafone

Title:	On the addition of enhanced ECN support in NR

Document for:	Discussion, Decision

# Introduction

ECN has been specified in TS36.300 since a few years back, the specification text in section 11.6 is suggested to be included also in TS38.300.

This paper suggests to enhance ECN support in NR with the addition of ECN bits in the headers of protocols that encapsulate IP packets or similar and that are accessible on the RLC layer. This makes it possible to ECN mark RLC-PDUs when congestion is detected on the RLC layer. The paper explains why such a modification is beneficial for the performance of NR.

# Discussion

## Motivation

ECN (Explicit Congestion Notification) is a technology that has been available for quite a while, its present semantics is specified in RFC3168 [2]. The main feature with ECN is that IP packets are marked instead of discarded by congested nodes, with congested nodes is meant for instance wireline switches or wireless access points that begin to queue up packets because the incoming data rate is higher than the outgoing ditto. The obvious benefit with ECN is a lower (almost zero) packet loss, which will significantly reduce the amount of retransmissions, which is highly beneficial e.g. for interactive applications and real-time video.

ECN is transport protocol agnostic. While the ECN marking is done on the ECN bits in the IP header, the actual feedback of the ECN marking in performed by the transport protocols, thus no dedicated reverse in-band or out-of-band signaling channel on the IP layer or below is needed to manage the feedback for the ECN information to the transport protocol sender. ECN support is specified for TCP, RTP/UDP and SCTP, ECN support for QUIC is also work in progress [11].

Transport protocol reaction to ECN is to reduce the sending rate, the default action specified in [2] is to reduce the sending rate in a manner like the action if packet loss occurs. Absence of ECN marked packets is taken as an indication that it is possible to increase the sending rate. This means that ECN is a mechanism that operates throughout the connection lifetime, not just only at the beginning of the connection.

L4S (Low Loss Low Latency Scalable throughput) is a novel technology, with its origin in data center transport technology, that can give very low latency even at high bitrates and network load levels, the concept is briefly presented in [4]. Services that can exploit L4S include for instance 360 Video and Virtual Reality, one key property with L4S is that the amount of data in the network (i.e. in flight) is kept at a minimum level, this gives additional benefits during e.g. handover because less data needs to be re-routed or discarded. L4S exploits ECN marking in a different way than specified in [2]. This to achieve high link utilization with low queue latency, the details of this is currently standardized in IETF.

Because the ECN bits are defined in the IP header, no deep packet inspection is needed, therefore ECN can also work over IPsec tunnels as copying of the ECN bits between inner and outer IP headers is defined in [1].

ECN has for a long while had a relatively slow pickup, mainly because it has been difficult to claim large benefits with ECN, for instance large file transfers do benefit very little from ECN support. The trend is however pointing at a larger use of internet services that are highly interactive. The user experience of interactive services is highly depending on a timely delivery of objects. In many cases, a lost packet in the transport layer leads to extra delay on the application layer due to retransmission. Today Apple enables ECN as default for their services and 50% of their devices connected by Ethernet or WiFi [5] as well as devices connected to the mobile data service of T-Mobile, Three and Telstra [8]. Studies of ECN-readiness of servers shown continual increase, with the latest reaching 70% [9] (2014 data).

Novel technology like L4S requires efficient and prompt ECN marking of packets for its function. The reasons are twofold

- Prompt ECN marking is required to quickly react to growing queues.

- L4S relies on measuring the amount of ECN marked packets, the transport protocol sending rate is finely adjusted based on this information. This property is exploited to make it possible for the transport protocols to quickly increase the sending rate, to avoid problems with sending rate overshoot and increased delay, it is thus instrumental that the measured amount of ECN marked packet is as fresh as possible.

Current 3GPP technology ECN support is limited to ECN marking in the IP header, which in practice limits ECN marking to the PDCP layer, prior to ciphering and possible compression. If congestion due to e.g. queue buildup is detected on the RLC layer, then a signal needs to be sent back to the PDCP layer to ECN mark an IP packet before it is encapsulated in a PDCP-PDU. The problem is that this gives a tail marking property to the ECN marking, which makes it hard to fulfil the requirement from L4S of prompt ECN marking.

To summarize, the issues with existing ECN marking capability in 3GPP access are:

- If congestion is detected e.g. on the RLC layer:

  - It becomes necessary to signal up to the PDCP layer to ECN-CE mark the next available PDCP-SDU (IP packet). This gives a tail-marking behavior that can degrade ECN efficiency for services that require low latency.

  - Mark/discard decision difficult. It is not possible to know on the RLC layer if an RLC-SDU is ECN capable, this means that a not-ECN capable IP packet must be discarded on the PDCP layer, this leads to inefficient tail-drop AQM implementations.

  - Split architecture deployments may implement PDCP processing in one node and RLC and lower layer processing in another node, with a transport link in between. The additional delay between the PDCP and RLC layer makes it a challenge to achieve a low queue delay on the RLC layer, given the simultaneous requirement to have enough data available on the RLC layer to optimally use the transport blocks.

- Header compression efficiency: Changing ECN bits gives a reduced compression efficiency for instance for ROHC. This situation is further aggravated if PDCP-SDUs are ECN marked on the PDCP layer prior to header compression.

- Processing efficiency: For high speed data it can be beneficial to preprocess a few PDCP-SDUs for reasons of processing efficiency. This can give further tail-mark/drop problems as described in 1) above

An addition of ECN marking capability on layers below the IP layer in 3GPP access gives the following benefits:

- Enhanced ECN marking efficiency. ECN marking will always be in-front of queue, which is beneficial especially for services that require low latency. For instance, URLLC is one example but enhanced ECN marking capabilities is beneficial for a very wide set of use cases, ranging from OTT services to specialized industrial use cases.

- Efficient ECN marking is maintained even though PDCP-SDUs are preprocessed on PDCP layer.

- Increased header compression efficiency in case that the ECN bits are zeroed prior to header compression.

- PDCP-SDU discard is not recommended in NR, deployment of ECN reduces the likelihood of buffer overflow and packet discards on the RLC layer.

In addition, the suggested change addresses the remaining issue #1 in an LS on ECN from IETF [3].

Section 7 gives an example that shows the potential with enhanced ECN marking capabilities in NR.

ECN support only in PDCP layer becomes a limiting factor for services that require low latency and high throughput.

ECN support in a protocol header that is accessible on the RLC layer is required for the enhanced ECN capability in NR that is necessary for the support of low latency and low loss access that is beneficial for a large variety of services.

## Realization

For the optimal implementation of ECN and especially the implementation of L4S support in RAN it is recommended that a lower layer header (from here on denoted LLH) that is accessible from the RLC layer include the ECN bits. This makes it possible to set the ECN bits in the LLH when congestion is detected on the RLC layer. The semantics to copy the ECN bits between the IP header and the LLH should follow the guidelines in RFC6040 [1] in which case the outer header is represented by LLH. The benefit with this is that it is possible to mark packets that are in the head of the queue with the obvious result that congestion signals reach the destinations faster. 
The location of the ECN bits (PDCP header, RLC header or other header) is left for discussion and decision in the RAN2 working group.

The steps involved in the ECN bits handling are as follows, for simplicity the term lower layer header (LLH) is used here, this depicts the protocol header where the ECN bits modification is effected when congestion is detected on the RLC layer:

- ECN bits in IP header (PDCP-SDU) are first copied to lower layer header

  - ECN bits in IP header are optionally zeroed for increased compression efficiency.

- PDCP-PDU is forwarded down to RLC layer.

- When congestion related mark/drop decision is made in RLC layer:

  - LLH has ECT(0) or ECT(1) code point set:  set CE code point in PDCP-PDU

  - LLH has Not-ECT code point set  discard packet, as per previous discussion, this case should be avoided

  - LLH has CE code point set  no action

- At PDCP de-capsulation of IP header: Copy ECN bits from LLH to IP header

  - If IPv4  update checksum in IP header

The suggested steps above are of low complexity. As regards to bullet Cb) above, PDCP discard in NR is regarded as a general problem and should be avoided, this is another motivation for the use of ECN as packet discards due to congestion can be avoided.

Affected nodes:

The suggested change requires a change in both gNB and UE. It is recommended that the ECN support is implemented both for UL and DL user plane processing.

ECN support in the form of ECN bits in a protocol header implies added functionality to copy the ECN bits between the IP header and the lower layer protocol header that is accessible from the RLC layer.

## Details

Recent work in IETF [6] will update RFC3168 to give room for different semantics for the ECN code point ECT(0) and ECT(1).

- The use of ECT(0) is intended for state of the art congestion control algorithms such as Reno and Cubic. This traffic class is named Classic.

- The use of ECT(1) is intended to enable very low latency communication as nodes should apply a very low marking threshold for traffic with this code point set. The senders (e.g. TCP servers) should employ congestion controls like DCTCP (Data Center TCP) to exploit the shallow marking threshold to its fullest extent. This traffic class is named L4S (Low Loss Low Latency Scalable throughput). The key benefit with L4S enabled traffic is that very low queue delay can be achieved even at high bitrates.

The interested reader is encouraged to read [6] and the referenced literature on L4S for more information.

A node that implements ECN marking must be able to separate classic traffic from L4S traffic. DualQ, described in [10] proposes to split the traffic in two different queues depending on how the ECN bits are set in the IP packets.

- Not-ECT, ECT(0) : Packet goes into a classic queue

- ECT(1), ECN-CE : Packet goes into an L4S queue

This scheme is proved to work well in a wireline switch implementation and may be useful in a 5G NR implementation. However in a 5G NR context an alternative is to directly classify flows as Classic or L4S and direct them to different bearers.

The exact implementation details can be vendor specific. The important properties that should be fulfilled are

- Classic traffic should experience a mark/drop behavior that is adapted for optimal performance with endpoint congestion control algorithm that are similar to Reno or Cubic in behavior. The mark/drop behavior should target roughly a BDP (Bandwidth Delay Product) queue depth.

- L4S traffic should experience marking at much lower queue delays than what is given by the 1*BDP rule. The exact setting is up to experimentation but for high bitrate bearers with throughput higher than ~20Mbps and reasonably short RTTs, marking thresholds below 5ms are possible

# Network assisted alternatives to ECN

A recent RAN2 contribution [7] proposes to remove support for ECN and instead rely on RAN-assisted codec adaptation. RAN-assisted codec adaptation has its merits for instance in that it can provide a direct rate to e.g. video coder and thereby, at least in theory achieve the highest possible bitrate without excessive delay or packet loss. Despite the merits with RAN-assisted codec adaptation there are a few things to consider.

- RAN-assisted codec adaptation is devised for real time media such as video and voice over RTP. New services may run over a large assortment of transport protocols such as TCP and QUIC. Adaptation of the RAN-assisted codec adaptation for this new services each require standardization around how said mechanisms is supposed to work with the new services.

- Scalability: RAN-assisted codec adaptation requires a robust and safe signalling mechanism to minimize the risk of man-in-the middle attacks. A limited use of RAN-assisted codec adaptation such as in the context of MMTEL voice and video can be manageable. Problem however arise if such a mechanism is applied at large for all kinds of internet services, a large part of which constitute very short lived connections. The signalling load can thus become unmanageable. ECN in comparison has a complexity that is orders of magnitude lower than the RAN-assisted codec adaptation mechanism.

- Bottleneck in other nodes(backhaul): With the introduction of new bearers in 5G with very high peak bitrates it is likely that the bottleneck moves higher upstream, for instance to the 5G backhaul infrastructure. RAN-assisted codec adaptation has no means to indicate congestion or a recommended bitrate when congestion occurs in these nodes. ECN on the other hand is today implemented in major wireline switches.

- Fixed wireless access or self-backhaul: 5G access technology can be used in scenarios where for instance tenants in a building share a fixed wireless 5G access bearer via e.g. a home router. Another scenario is where a 5G access bearer is used for self-backhaul. In both cases, many flows and users with a potentially wide range of characteristics, may share a common bearer and there is no clear solution how a RAN-assisted codec adaptation can share the capacity between the flows. ECN on the other hand works also in this case.

Given the arguments above the conclusion must be that RAN-assisted codec adaptation is not a viable alternative to ECN.

RAN-assisted codec adaptation is not a viable alternative to ECN

Furthermore, [7] makes statements around ECN that warrants clarification:

Quote 1: “If any node on the routing path does not support ECN, or it does not understand the meaning of ECN-Capable Transport (ECT) codepoint, ECN mechanism may not work correctly”: ECN connections are designed to work correctly even if none of the nodes on the path understand ECN - they just don’t get the benefit of ECN. ECN only needs to be deployed in buffers that might become a bottleneck to get the benefits. A congested node that does not implement ECN capability will discard packets and ECN connections are designed react to packet loss also. While the slow pickup of ECN may lead to the conclusion that general use of ECN is years away, fact is that wireline switches already today support ECN and players like Apple have announced that they enable ECN. The main pain point with ECN today is lack of implementations for it in 3GPP access technology. Thus it is the lack of ECN support in 3GPP RAN nodes that hamper the adoption of ECN, that is a strong argument to implement and deploy ECN support in 3GPP access, not to replace it with another solution.

Quote 2: "In practice, ECN has seen very limited deployment in the Internet [2].". Reference [9] (based on 2014 data) measured 56% of servers supporting ECN. Regarding clients, since 2015 Apple has enabled ECN (currently on a random selection of 50% of its devices when connected by Ethernet or WiFi, with plans to move to 100% given the trial has been very successful). Microsoft plans to follow suit.

Statements on the status of ECN in [7] are wrong or misleading

# Text proposal

This is the text proposal to TS 38.300.

## 2	References

[xx]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

## X.Y      Explicit Congestion Notification

The eNB and the UE support of the Explicit Congestion Notification (ECN) is specified in Section 5 of [xx] (i.e., the normative part of [xx] that applies to the end-to-end flow of IP packets), and below.  ECN is beneficial especially for latency sensitive interactive applications such as chat and gaming as well as for real-time voice and video, this because loss as a congestion signal is avoided, losses that would otherwise necessitate retransmission of packets with additional application delay as a result.

ECN is supported by the user-plane protocols through carrying the ECN codepoints.

Editor’s note: It is for further study which user-plane protocol carries the ECN codepoints.

# Conclusion

In section 2 we made the following observations:

Observation 1	ECN support in PDCP layer only becomes a limiting factor for services that require low latency and high throughput.

Observation 2	ECN support in a protocol header that is accessible on the RLC layer is required for the enhanced ECN capability in NR that is necessary for the support of low latency and low loss access that is beneficial for a large variety of services.

Observation 3	ECN support in the form of ECN bits in a protocol header implies added functionality to copy the ECN bits between the IP header and the lower layer protocol header that is accessible from the RLC layer.

Based on the discussion in section 2 we propose the following:

Add support for enhanced ECN for Uplink and Downlink user plane traffic in NR.

Study which layer (PDCP, RLC or other) should carry the ECN bits.

# References

https://tools.ietf.org/html/rfc6040

https://tools.ietf.org/html/rfc3168

R2-1700677, “Response to Reply LS in response to (SP-150579) on 3GPP Work on Explicit Congestion Notification for Lower Layer Protocols”, IETF, RAN2#97, Athens, Greece, 13 - 17 February, 2017

http://www.gsma.com/newsroom/wp-content/uploads/WWG.16-v1.0.pdf

https://www.ietf.org/proceedings/98/slides/slides-98-maprg-tcp-ecn-experience-with-enabling-ecn-on-the-internet-padma-bhooma-00.pdf

https://tools.ietf.org/wg/tsvwg/draft-ietf-tsvwg-ecn-experimentation/

http://www.3gpp.org/ftp/TSG_RAN/WG2_RL2/TSGR2_98/Docs/R2-1705164.zip

“Networking for the Modern Internet” Apple Worldwide Developer Conference https://developer.apple.com/videos/play/wwdc2016/714/ (video requires Apple's Safari browser)

Trammell, B., Kuhlewind, M., Boppart, D., Learmonth, I., Fairhurst, G. & Scheffenegger, R., "Enabling Internet-Wide Deployment of Explicit Congestion Notification," In: In Proc Passive & Active Measurement (PAM'15) Conference (2015)

https://tools.ietf.org/wg/tsvwg/draft-ietf-tsvwg-aqm-dualq-coupled/

https://datatracker.ietf.org/doc/draft-johansson-quic-ecn/

# Example simulation

The potential with efficient ECN marking in the context of L4S is shown in this section. The example shows a comparison between three congestion control algorithms when a large file is transmitted over a 20ms RTT bottleneck where the link rate changes between 20 and 100Mbps in 5 second steps. The three congestion control algorithms are run in one iteration each.

The congestion controls are:

- Cubic: The most widely used congestion control algorithm today. Cubic is of the type AIMD (Additive Increase Multiplicative Decrease), which means that rate increase when link throughput is increased can become slow.

- BBR: A novel congestion control algorithm devised by Google that estimates the link throughput and RTT and determines the optimal sending rate based on this information. For an efficient operation BBR must estimate the min RTT at regular intervals (every 10s), in effect this means that the transmission rate is reduced for a short period. The success of such operations depend on the traffic mix over the bottleneck, other traffic over the bottleneck can give faulty min RTT estimates, which will degrade the performance of BBR

- DCTCP-bis: A modified data center TCP (DCTCP) congestion control algorithm that exploits the potential with L4S, the development of congestion control algorithm goes under the working name ‘Prague’. The DCTCP-bis algorithm is a very early version that points out the potential with Prague given the enhanced congestion information given by L4S.

Figure 1 shows the throughput, DCTCP-bis manage to fill the bottleneck most efficiently with BBR on second place. Cubic has problems to ramp up the speed when the link bitrate increases due to the AIMD nature of Cubic. The graph also shows that DCTCP-bis is slightly faster than BBR to reach full link utilization.

Figure 2 shows the queue delay in the bottleneck: Cubic has the largest standing queue. This is because the AQM must be tuned with a higher mark/discard threshold to allow for full link utilization. The graph illustrates that both Cubic and BBR gives a slow start delay spike, this can have a negative effect on other already ongoing flows as they will too experience this delay spike. DCTCP-bis does not give any noticeable delay spike in slow start at all, the latter indicates that the L4S concept, with enhanced ECN support in RAN and congestion control adapted for this, has potential to give low queue delay even in complex scenarios with multiple parallel flows over the same bottleneck.

BBR manages to reach roughly the same queue delay as DCTCP-bis. BBR however takes longer to restore a low queue delay when the bottleneck bandwidth drops from 100 to 20Mbps than what is the case with DCTCP-bis.

Figure 1 Throughput

Figure 2 Queue delay