3GPP TSG-RAN WG2 #98-AH	Tdoc R2-1707155

Qingdao, P.R. of China, 27th – 29th June 2017

Agenda Item:	10.3.3.1

Source:	Ericsson

Title:	PDCP modelling issues

Document for:	Discussion, Decision

# Introduction

In this contribution, we address remaining open modelling issues and FFSs of the current running draft PDCP TS 38.323.

# Discussion

It had been agreed in RAN2#97 that “All NR PDCP state variables are based on COUNT”. From the COUNT value, HFN as well as SN can be easily obtained considering MSB or LSB part of the bit-based integer. There is no need to define additional HFN-related state variables. HFN based assistant “local function” variables could however be considered within an algorithm, e.g. the reception algorithm, to improve readability, but are not needed to be defined as state variables in PDCP. Addressing the open issue on TX_HFN, be believe therefore that it should be removed from the PDCP specification.

TX_HFN and RX_HFN are not defined as a state variables. HFN is derived from the COUNT-based state variable.

A further FFS was listed on whether duplication is also applicable to PDCP control PDUs. In NR the control PDU can be either a PDCP status report, or an interspersed ROHC feedback. We see duplication in PDCP mostly as a way to improve the reliability to transmit an application layer data unit or RRC signalling message within a certain latency bound. The PDCP status report does not improve reliability of a transmission. There is also no need to convey it within a certain latency bound, as the same information would be included already in a timely RLC AM status report. The PDCP status report provides merely a way to optimize retransmissions at handover/bearer type change. Therefore, PDCP status report should not be duplication. ROHC feedback is used to improve the compressor confidence, but not strictly required for ROHC operation. We believe that thus higher reliability of ROHC feedback is not required. Therefore we propose:

Duplication is not applicable to PDCP control PDUs.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	TX_HFN and RX_HFN are not defined as a state variables. HFN is derived from the COUNT-based state variable.

Proposal 2	Duplication is not applicable to PDCP control PDUs.

# References

“draft 38232-010_clean_v2”, [98#37][NR/UP] - Running TS 38.323 - LG