3GPP TSG-RAN WG2 #98 AH	Tdoc R2-1707158

Hangzhou, P.R. of China, 27th – 29th May 2017

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	SPS Enhancements in NR

Document for:	Discussion, Decision

# Introduction

The relevant SPS agreements in RAN2-97bis are highlighted as below

Agreements on SPS:

-	Like in legacy LTE, at least SPS period is configured by RRC.  FFS how frequency resources, MCS, etc., for SPS are provided to the UE depends on RAN1 discussion.

-	UL skipping for dynamic grant should be configurable.  FFS if UL skipping for SPS is configurable

-	Working assumption:  Like in LTE, DRX behaviour with SPS UL should be to restart inactivity timer when UL data is transmitted, and not to restart when SPS UL grant is not used.  This behaviour depends on outcome of DRX design.

This paper further discusses how to configure SPS in case a UE supports multiple SPS suitable services in NR.

# Discussion

In LTE, for either UL or DL, there is at maximum one SPS configuration active per UE, which is signalled via RRC. The SPS configuration can be activated/deactivated via PDCCH addressed with Semi-Persistent Scheduling C-RNTI.

For either UL or DL in LTE, up to one respective SPS configuration can be configured for one UE per cell.

For LTE V2X Rel-14, a UE can be configured with up to 8 active SPS configurations per cell. However, the V2X use case is not included in NR phase 1. It means there is no similar use case or motivation in NR for the moment to support multiple SPS configurations for a UE per cell.

There is no use case/motivation similar as LTE V2X to support multiple SPS configurations for a UE per cell in NR.

For LTE CA, there is also only 1 SPS configuration per UE in the primary cell. This is sufficient since it is seldom for a UE in CA to have 2 SPS services at the same time. In addition, cross-carrier scheduling provides better flexibility and resource utilization than that with multiple SPS configurations. However, in NR, the situation is different in that a UE may be configured with multiple TTI lengths/numerologies. Each carrier/cell may have different TTI length/numerology characteristic. It is thus more likely than in LTE that the UE benefits from having more than one SPS-suitable service in parallel, e.g. URLLC and VoIP. The eMBB service may have diverse QoS requirements in terms of latency and transmission reliability. In some cases, it would be beneficial to apply SPS for eMBB in case critical latency is required.

For carrier aggregation in NR, it is beneficial to support a SPS configuration per cell (both primary and secondary cells).

For carrier aggregation, NR supports one SPS configuration per cell.

In LTE UL SPS configuarion, the N-th grant occuring in the subframe can be calculated via the following formula:

- (10 * SFN + subframe) = [(10 * SFNstart time + subframestart time) + N * semiPersistSchedIntervalUL + Subframe_Offset * (N modulo 2)] modulo 10240.

- Where SFNstart time and subframestart time are the SFN and subframe, respectively, at the time the configured uplink grant were (re-)initialised.

Similar SPS configuration is also applied to DL. The SPS transmission periodicity is configured in terms of subframe.

In LTE, SPS transmission periodicity is configured in the unit of subframes (i.e. 1 ms).

However, multiple numerology/slot duration will be supported in NR. In CA, each cell may be configured with a very different TTI length/slot duration. Considering the means provided by RAN1, such as flexible sub-frame and slot structure, with the smallest possible transmission/scheduling unit is mini-slot, comprises a few OFDM symbols (e.g., 2 OFDM symbols). It may thus be beneficial to support SPS configuration in a granularity shorter than 1 subframe, e.g. on a mini-slot level.

NR supports a SPS transmission periodicity shorter than 1 subframe, e.g. on a mini-slot basis.

# Conclusion

In section 2 we made the following observations:

Observation 1	For either UL or DL in LTE, up to one respective SPS configuration can be configured for one UE per cell.

Observation 2	There is no use case/motivation similar as LTE V2X to support multiple SPS configurations for a UE per cell in NR.

Observation 3	For carrier aggregation in NR, it is beneficial to support each SPS configuration per cell (both primary and secondary cells).

Observation 4	In LTE, SPS transmission periodicity is configured in the unit of subframes (i.e. 1 ms).

Based on the discussion in Section 2 we propose the following:

Proposal 1	For carrier aggregation, NR supports one SPS configuration per cell.

Proposal 2	NR supports a SPS transmission periodicity shorter than 1 subframe, e.g. on a mini-slot basis.