3GPP TSG-RAN WG2 #98-AH	Tdoc R2-1704379

Qingdao, P.R. of China, 27th – 29th June 2017

Agenda Item:	10.3.4.3

Source:	Ericsson

Title:	Reflective QoS and Presence of Flow-ID

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss several unresolved issues related to reflective QoS.

# Discussion

## Presence of SDAP header and QoS Flow ID

To enable reflective QoS, the RAN marks downlink packets over Uu with a QoS flow ID. The UE marks uplink packets over Uu with the QoS flow ID for the purposes of marking forwarded packets to the CN.

RAN2-97bis agreed that …

DL packets over Uu are not marked with “Flow ID” at least for cases where UL AS reflective mapping and NAS reflective QoS is not configured for DRB.

AS layer header include the UL “Flow ID” depending on network configuration

RAN2-98 discussed the topic again and concluded the following:

1. The QoS flow ID is presence once the AS reflective QoS is active.  FFS whether it is always present.

2. gNB should be informed when NAS layer reflective QoS is activated (e.g. can be used).  It is FFS how we handle NAS reflective QoS and dependent on how/when it will be provided.

3. RAN2 will support a mode in which SDAP header is not present and is configured per DRB.  If configured, FFS how the different fields are handled.

### Dynamic presence of QoS Flow ID

Bullet 3 above implies that the eNB configures by RRC for each DRB whether or not the UE shall include the SDAP header in uplink PDCP SDUs and whether the SDAP header is present in DL PDCP SDUs. According to bullet 1, it should be discussed further whether the “QoS flow ID”, once SDAP is configured, “is always present” or whether it may be present only dynamically. To achieve the latter the SDAP header would need to indicate with one bit the presence of the “QoS Flow ID” . Since such an indication would consume one bit itself it would not allow reducing the size of the SDAP header below one octet. Hence, we consider it more efficient to aim for an SDAP header that has a fixed size of one byte (if configured to be present by RRC).

If the SDAP is configured for a DRB by means of RRC, the “QoS Flow ID” is present in UL and DL packets on that DRB (not dynamically enabled/disabled).

### Re-configuring presence of SDAP header

Since UE and network must be aware at any point in time which PDCP SDUs contain an SDAP header, the presence of this header should only be changed by a synchronized reconfiguration, i.e., RRCConnectionReconfiguration including mobilityControlInfo. One should note that this still requires that the PDCP receiver entity informs the SDAP entity for each delivered PDCP PDU whether the SDAP header is present. If one would like to avoid that, RAN2 should restrict the configuration of SDAP header to full configuration (fullConfig). However, we consider it acceptable to allow enabling/disabling the SDAP header during a handover.

The eNB may change the presence of the SDAP header only by means of a handover, i.e., a synchronized reconfiguration.

Due to the decision to make SDAP a separate protocol above PDCP, the RoHC compressor and decompressor (which are specified to be part of PDCP) must now peek into the SDAP PDU and work with the SDAP SDU (the IP packet). While this is not a nice design, we believe that with the two proposals above both the UE and the network have all information that is necessary to perform RoHC.

Based on the RRC configuration RoHC compressor and decompressor entities in UE and network side can determine the position of the IP packet inside each PDCP PDU, i.e., whether an SDAP header is present or not.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	If the SDAP is configured for a DRB by means of RRC, the “QoS Flow ID” is present in UL and DL packets on that DRB (not dynamically enabled/disabled).

Proposal 2	The eNB may change the presence of the SDAP header only by means of a handover, i.e., a synchronized reconfiguration.

# References

R2-1707159. SDAP Header Format. Ericsson, RAN2-98-AH, Qingdao, P.R. of China, 27th – 29th June 2017

R2-1707161. QoS Flow Remapping Within the Same Cell and in Handover. Ericsson, RAN2-98-AH, Qingdao, P.R. of China, 27th – 29th June 2017

# Annex: QoS-related agreements in previous meetings

RAN2-95 discussed the basic principles of the NR QoS framework and reached the following agreements:

Agreements

1	For DL for a non-GBR flow, the eNB sees an indication over NG-u and based on the indication the eNB maps the packet to a DRB of an appropriate QoS. RAN2 understanding of SA2 agreements is that eNB has a QoS profile associated with the indication.

FFS whether there is a requirement for every different QoS indication to be mapped to a different radio bearer.

2	Functionality is required to differentiate flows from different PDN-connections over the radio interface (e.g. by using separate DRBs or by an explicit indication in a header)

3	For DL, the eNB establishes DRBs for the UE taking the QoS profiles in to account.

FFS how the DRB is established in the first packet is an UL packet.

At RAN2-95bis some further agreements were achieved and the first of the FFSs above was resolved:

Agreements

1:	RAN determines the mapping relationship between QoS flow (as determine by the UE in UL or marked by the CN in DL) and DRB for UL and DL.

1a	RAN can map multiple QoS flows to a DRB.

2	Specification will not forbid a GBR flow and non-GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

3	Specification will not forbid more than one GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

FFS: Whether traffic from different PDU sessions can be mapped to one DRB or not.

Agreements

1	Default DRB is established by eNB at PDU session establishment (or an existing DRB may be used if mapping of more than one session to a DRB is allowed)

2. 	If the first packet of the flow is UL packet, if no mapping rule is configured in the UE, the packet is sent through default DRB to the network.

FFS How and when the network can remap the flow to more appropriate DRB.

FFS the first packet is handled in the case that pre-authorised QoS is configured

FFS whether the pre-authorised QoS applies to RAN or only to the UE.

FFS whether there is a single level of mapping from UL TFT (5 tuple) to DRB, or whether there is a 2 level mapping from UL TFT to QoS flow and then from QoS flow to DRB.

RAN2#96:

Agreement

1:	Traffic from different PDU sessions are mapped to different DRBs

2:	In DL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow->QOSflow mapping, and AS is responsible for the QOSflow->DRB mapping (confirmation of SA2 agreement status).

3:	In UL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow->QOSflow mapping, and AS is responsible for the QOSflow->DRB mapping.

4	DL packets over Uu are marked in band with QOS-flow-id for the purposes of reflective QoS

5	UL packets over Uu are marked in band with QOS-flow-id for the purposes of marking forwarded packets to the CN.

FFS for bullets 4 and 5 whether it can be semi-statically configured to not include the QOS flow ID in some cases.

FFS for bullets 4 and 5 whether it might be possible to use a shorter id over the radio compared to that received from the CN. This is a stage 3 issue.

Agreements

1	For reflective QoS, the UE determines QoS Flow ID to DRB mapping in the uplink based on the downlink packets received within a DRB and applies those filters for mapping uplink Flows to DRBs.

2	The UE "continuously" monitors the QoS Flow ID in downlink PDCP packets and updates the reflective QoS Flow ID to DRB mapping in the uplink accordingly.

3	RRC can configure an uplink mapping

FFS The precedence of the RRC configured mapping and reflective QoS (e.g. can reflective QoS update an RRC configured mapping)

Working assumption:

If an incoming UL packet does not match a QoS Flow ID to DRB mapping (neither a configured nor a determined via reflective QoS), the UE shall map that packet to the default DRB of the PDU session.

=>	FFS whether the QoS field is added by PDCP or a new protocol layer above PDCP.

RAN2 Ad-Hoc January 2017:

Agreements

1: A new user plane AS protocol layer (e.g. PDAP) above PDCP should be introduced to accommodate all the functions introduced in AS for the new QoS framework, including:

-	QOS flow->DRB routing;

-	QoS-flow-id marking in DL packets;

-	QoS-flow-id marking in UL packets;

2	The new protocol layer is applicable for all cases connecting to the 5G-CN

3:	Single protocol entity is configured for each individual PDU session.

RAN2-97 Athens:

1 RAN2 to confirm that the timing of non-default DRB establishment (RAN to UE) for QoS Flow configured during PDU Session Establishment could be done NOT at the same time as PDU Session Establishment. (up to eNB implementation) 2 Working assumption from RAN2#96 is confirmed. i.e. First UL packet that doesn't have a mapping to a DRB, is mapped to a default DRB.

1 “Lossless HO”, that is, lossless, in sequence without duplication to upper layers, should be supported in specification for intra-NR. FFS whether we support QoS flow remapping at handover and, if supported, whether the handover is lossless for this case.

RAN2-97bis Spokane (April 2017)

Agreements 1 NR/NR DC should support that different QoS flows of the same PDU session can be mapped to MgNB and SgNB. 2 In the case of NR/NR DC where different QoS flows of the same PDU session are mapped to MgNB and SgNB then there is one SDAP entity in the MgNB and one in SgNB for that PDU session. RAN2 understand that support of this behaviour is still under discussion on SA2

Agreements: - New AS layer PDU is PDCP SDU - AS layer header is byte-aligned - DL packets over Uu are not marked with “Flow ID” at least for cases where UL AS reflective mapping and NAS reflective QoS is not configured for DRB. - AS layer header include the UL “Flow ID” depending on network configuration