3GPP TSG-RAN WG2 #98-AH	Tdoc R2-1707168

Qingdao, P.R. of China, 27th – 29th June 2017

Agenda Item:	10.3.1.13

Source:	Ericsson

Title:	UP timers in MAC

Document for:	Discussion, Decision

# Introduction

At the RAN1#86bis meeting it was agreed that the duration of a subframe in NR is 1 ms and the reference numerology is 15 kHz:

Agreements

•	Sub-frame duration is fixed to 1ms

•	Reference numerology for defining subframe duration is 15 kHz

In this contribution, we discuss some impacts on different user plane timers related to MAC given the above agreement and how these timers should be configured for different numerologies and TTI durations.

# Discussion

The timers as used in LTE are investigated in this paper. Whether or not all of them will be adopted into NR is not decided but they have been used as a baseline in this investigation on which timers will be impacted on having a fixed subframe duration of 1ms and if the configurable value range of each timer is also adequate for NR. In the following text, we have excluded timers which impact parts of the protocols not included in the first phase of NR, e.g. timers related to WLAN and sidelink operation.

The use of a subframe or a PDCCH-subframe as a time unit for configuring timers in NR will result in different time duration depending on the TTI duration used. Therefore, means to configure timers in NR regardless of numerology is needed to make sure the gNB and UE are aligned when a specific timer is active or not.

## MAC Timers

Most timers in the LTE MAC are configured by using the time unit of a subframe or a PDCCH-subframe. Therefore, the agreement on having a subframe fixed to 1ms would result in different behavior depending on the TTI duration configured for some timers while other timers are without impact. Especially the term PDCCH-subframe could give rise to some confusion in NR, as there is greater flexibility in PDCCH configuration. If the UE is monitoring several component carriers, using different numerologies, the number of PDCCH occasions and thereby PDCCH-subframes would be very different on different carriers and could be ambiguous in the framework of C-DRX, i.e. which carrier should be used for counting at what point in time? It is therefore here proposed to exclude the use of a PDCCH-subframe in the specs for NR. See also [1] where a framework for PDCCH monitoring is proposed.

Exclude the use of the term PDCCH-subframe in the specs for NR.

The MAC timers have been divided into two groups below; MAC timers that we believe are not impacted by the use of different numerologies and MAC timers that we believe are impacted.

### MAC timers not impacted by the use of different numerologies/TTI durations

The timers in this sub clause are the MAC Protocol timers in LTE that we believe could be adopted to NR without the need for any change in configuration or value range.

-	timeAlignmentTimer

-	At expiry the UE time alignment is considered lost, so that UE needs to re-synchronize to the network. Since the UE mobility and drifting of the UE clock is not expected to be different for NR, this timer does not need changes for NR operation.

-	sCellDeactivationTimer

-	Does not need changes, since SCell handling in CA is independent of the TTI used. Current values for this timer are in between 2 and 128 radio frames and should be sufficient also for NR.

-	periodicPHR-Timer, prohibitPHR-Timer

-	The power headroom might change with changes in path loss, but since dynamic path loss changes to be reported do not occur faster with a shorter TTI, also this timer does not require changes and current values should be sufficient.

-	periodicBSR-Timer, retxBSR-Timer, logicalChannelSR-ProhibitTimer

-	Faster periodic BSR reporting is not foreseen when UE is configured for numerologies using shorter TTI.

-	sr-ProhibitTimer

-	Refers to SR-periods as its basis, i.e. it can be considered a counter. No change of this timer’s behaviour is needed.

NR should adopt the timeAlignmentTimer, sCellDeactivationTimer, periodicPHR-Timer, prohibitPHR-Timer, periodicBSR-Timer, retxBSR-Timer, logicalChannelSR-ProhibitTimer and sr-ProhibitTimer and reuse the existing configurations. Additional values are FFS.

### MAC timers impacted by the use of different numerologies/TTI durations

The MAC timers below are configured by using either subframes or PDCCH-subframes. It should be noted that a PDCCH-subframe would not be applicable in NR since it could be several PDCCH occasions per subframe and therefore very ambiguous. The numerology used on different carriers have an impact on these timers and given that a UE can be configured with two or more carriers using different numerologies, which also can change dynamically, we feel that they need more analysis on how to handle and how to be configured to ensure the optimal operation with respect to UE energy savings, reachability/latency and complexity.

The MAC Protocol in LTE supports the following DRX related timers

-	onDurationTimer

-	This timer is controlling the reachability of the UE by configuring the number of PDCCH occasions to be monitored. Reusing the PDCCH-subframe configuration would result in different behaviour depending on the numerology. This timer should therefore be configured by an absolute time unit in ms. The monitoring during while the onDurationTimer is running should be configuring by using a monitoring pattern that could be reused across numerologies, see [1] for more details.

-	drx-InactivityTimer

-	The number of PDCCH-subframe(s) for which the UE should be Active after successfully decoding a PDCCH. This timer should therefore be configured by an absolute time unit in ms.

-	drx-RetransmissionTimer, drx-ULRetransmissionTimer

-	Indicates the maximum number of PDCCH-subframes for which UE should be monitoring PDCCH awaiting a retransmission. This timer also needs to be defined in an unambiguous way and should therefore be configured by an absolute time unit in ms.

-	longDRX-CycleStartOffset

-	Defines long DRX cycle length as well as the DRX offset. This timer should therefore be configured by an absolute time unit in ms.

-	shortDRX-Cycle

-	Indicates the length of the short cycle in subframes similar to the longDRX-CycleStartOffset. This timer should therefore be configured by an absolute time unit in ms.

-	mac-ContentionResolutionTimer

-	Specifies the number of consecutive subframe(s) during which the MAC entity shall monitor the PDCCH after Msg3 is transmitted. Needs to be unambiguously defined and adapted to NR. This timer should therefore be configured by an absolute time unit in ms.

-	ra-ResponseWindowSize

-	The RA Response window which starts at the subframe that contains the end of the preamble transmission plus three subframes and has length ra-ResponseWindowSize. Needs to be unambiguously defined and adapted to NR. This timer should therefore be configured by an absolute time unit in ms.

The above MAC timers: onDurationTimer, drx-InactivityTimer, drx-RetransmissionTimer, drx-ULRetransmissionTimer, longDRX-CycleStartOffset, shortDRX-Cycle, mac-ContentionResolutionTimer, and ra-ResponseWindowSize should all be adapted to NR and be configured by an absolute time unit in ms. Additional values are FFS.

# Conclusion

We have the following proposals:

Proposal 1	Exclude the use of the term PDCCH-subframe in the specs for NR.

Proposal 2	NR should adopt the timeAlignmentTimer, sCellDeactivationTimer, periodicPHR-Timer, prohibitPHR-Timer, periodicBSR-Timer, retxBSR-Timer, logicalChannelSR-ProhibitTimer and sr-ProhibitTimer and reuse the existing configurations. Additional values are FFS.

Proposal 3	The above MAC timers: onDurationTimer, drx-InactivityTimer, drx-RetransmissionTimer, drx-ULRetransmissionTimer, longDRX-CycleStartOffset, shortDRX-Cycle, mac-ContentionResolutionTimer, and ra-ResponseWindowSize should all be adapted to NR and be configured by an absolute time unit in ms. Additional values are FFS.

- References

R2-1707121, Impact on MAC from PDCCH monitoring occasions, Ericsson, Hangzhou, P.R. of China, May 2017