3GPP TSG-RAN WG2 #98-AH	Tdoc R2-1707171

Qingdao, P.R. of China, 27th – 29th June 2017

Agenda Item:	10.3.2.3

Source:	Ericsson

Title:	RLC UM open issues

Document for:	Discussion, Decision

# Introduction

This contribution discusses the open issues of NR RLC UM operation.

# Discussion

## Sequence numbering

One of the open issues, also discussed in the email discussion [1] is if SN is included in all PDUs, or only in the case the PDU corresponds to a segmented RLC SDU. The main argument for not including the SN for complete SDU is that it would be redundant design and there would be some overhead gain, as the header size could potentially be smaller.

However, in our view there are not many advantages in not using the SN in all PDUs.

First, a receiver window cannot be moved and the variables updated in similar fashion as in LTE if SN is included only for SDU segments. Majority of the companies seem to be in favour of using a receiver window. If the receiver window is only moved when new SN is detected, the window would not move at all unless segmented SDUs arrive.

If SN is used only for SDU segments, the receiver window and the receiver window movement need to be redefined.

In the discussions on RLC UM mode, there has not been a clear use case for UM, so it is not clear what are the requirements for the unacknowledged mode. In our opinion, we should target both VoIP-like use where the payload can be relatively small, and higher data rate cases, for example video streaming with larger payload and TB sizes. We do not agree that UM mode should be used only for small payload cases.

RLC UM mode should support both low data rate (VoIP-like) and high data rate (high TB size) use cases.

This affects the header design: The headers should be kept as small as possible when the header-to-data ratio is low and when the ratio is high, one additional header byte does not result in any practical gain. It can be further argued that even in VoIP case, optimization of one or two bytes does not bring much gain.

Header length optimization of one or two bytes does not result in significant gains.

Some additional arguments on the drawbacks if SN is not included in all PDUs:

- , A header is required in any case for all PDUs to distinguish between segmented and complete SDU, thus the potential reduction in overhead may not be more than one or two bytes, depending on the exact header format, e.g., the length of the SN field. See also Section 2.3 for more details.

- Not using SN will not result in processing gain as header for PDUs with a complete SDU needs to be processed by the receiver in any case.

- As identified e.g. in [3], removing SN would mean fast HARQ failure detection is not possible for UM.

- Removing SN for complete SDUs results in extra complexity and header formats (lengths) for RLC UM, which should be avoided at this phase.

- It is beneficial to align the UM solution with RLC AM solution to avoid implementation complexity.

Based on the above discussion, we do not see any benefit in bringing extra complexity and effort for RAN2 on designing new form of RLC UM, as the LTE RLC UM has been proven to work and there seems to be no clear benefit in changing the existing RLC UM design.

All RLC UM PDUs include a SN.

## SN field length

In the email discussion [1] some questions were hinting towards using a 6-bit SN field and companies provided their views on if this is sufficient. As stated above, a 6-bit SN field does not give large enough range, even in the case if the SN is included only for segmented SDUs. The SN field length should be long enough to support worst case scenario where the SN rate is proportional and close to SDU segment rate, that is, a scenario where most if not all SDUs are segmented (for a period of time). Even today, when RLC UM is used for VoIP it can be seen that packets need to be segmented e.g. on cell edge when the coverage is poor. For NR scenarios using high frequencies, temporary deep fades will happen, and RLC UM needs to be able to cope with such situations. The example use case for UM should not be only VoIP, but RAN2 should consider use cases such as video streaming and future services using RLC UM with not as stringent latency requirements.

For example, let us take LTE as a baseline and consider 8 HARQ processes and 8 ms HARQ RTT. In the worst case scenario, we would need to segment all SDUs for some time period, so let’s further assume every RLC SDU is segmented into two segments so that it would be carried by two HARQ processes. Each HARQ RTT would then carry 4 RLC SDUs. For 6-bit SN, we would have 64 segmented SDUs in total before SN wrap-around, corresponding to 64 / 4 * 8 = 128 ms in case of no retransmissions.

However, if one HARQ process is stuck for some time and fails to deliver a SDU segment, and only 3 SDUs could be delivered per HARQ RTT, the wrap-around would take 64 / 3 * 8 = 171 ms. This is a short gap, which e.g. a video streaming application can easily overcome by buffering, but where RLC UM would fail in SDU reassembly when using 6-bit SNs.

Moreover, in the above analysis we have used LTE numerology, while for the NR case with shorter HARQ RTT the situation would be worse. Additionally, if we use CA the SN range should take this into account, and 6-bit does not give much flexibility in increasing the number of component carriers.

Thus, we should adopt similar SN ranges we have agreed for PDCP operation, that is, 12- and 18-bit SNs. The configuration should follow SN configuration for PDCP.

12-bit or 18-bit SN field is used for RLC UM and is configured together with PDCP SN range.

## Header length

In Table 1 we have listed the minimum header size for different SN lengths. We have taken into account the FI-like field (2 bits), possible SN (12 or 18 bits) and SO field (16 bits - using max LTE SO field size as baseline). The listed numbers are absolute minimums for byte aligned header using these three fields, if additional fields or reserved bits are included the sizes may be larger.

Table 1. Minimum byte aligned RLC UM header sizes based on 2 bits FI field, 16 bits SO field and SN field.

|  | Complete SDU | First SDU segment | Middle/last SDU segment |
| --- | --- | --- | --- |
| 12-bit SN in all PDUs | 2 bytes | 2 bytes | 4 bytes |
| 18-bit SN in all PDUs | 3 bytes | 3 bytes | 5 bytes |
| 12-bit SN only for segmented PDUs | 1 byte | 2 bytes | 4 bytes |
| 18-bit only for segmented PDUs | 1 byte | 3 bytes | 5 bytes |

One observation based on Table 1 is that using SN for only the segmented SDUs would lead to gain of most 2 bytes (when using 18-bit SN). We do not think this is significant enough to make this change compared to LTE RLC UM. Moreover, if the header size would be optimized for all the different SDU options, there could be up to three different header sizes.

The implementation and processing complexity increase with the number of different header formats or lengths.

Our preference is to use SN in all PDUs, and in that case the sizes of headers for complete SDU and first part of segmented SDU would be the same regardless of which SN range is used. As the SO-field needs to be included for middle and last segmented SDU, increasing the header size by two bytes if 16-bit SO-field is used, it is viable to use longer header middle/last part of segmented SDUs.

PDU format for unsegmented SDU and first part of segmented SDU have the same header size.

Note that if SN is not included for complete SDU, there is no header overhead gain if the header size is the same as for first part of segmented SDU. In any case we want to avoid defining multiple different header formats for RLC UM.

## Receiver window operation

Figure 1. LTE RLC UM receiver window.

The receiver window operation for RLC UM can used the LTE baseline as explained in our previous contribution [2]. It has been agreed that duplication detection is not necessary, but a window can be still used for reassembly of the SDU segments, thus we think the LTE baseline should still be followed. This applies equally well regardless of if RAN2 agrees to include SN in all PDUs or only in the PDUs containing SDU segments.

RLC UM entity shall keep a receiver window and receiver window state variables VR(UR), VR(UH), VR(UX) and UM_Window_Size (or corresponding variables using NR naming)

A receiver window alone does not protect against cases where part of the transmitted SDUs are lost, and the transmitter window wraps around. Majority of companies agree [1] that the window-based operation is pull-based, i.e., SNs received out-of-window move the receiver window. However, if there is a situation where number of consecutive PDUs are lost, the window can get stuck temporarily. For example, in the case SN is used only for segments, and a segment is lost (i.e. not recovered by HARQ) followed by transmission of complete SDUs, the window would remain in its position until a segmented SDU would arrive again. In such case the receiver would unnecessarily keep a segment, considered for reassembly, in its buffer. Alternatively, if RAN2 agrees to include SN in all PDUs, then we don’t see any advantage in altering the baseline LTE design. It should be noted a timer is also kept in NR RLC AM, and there are benefits in aligning UM and AM design.

Furthermore, especially if RAN2 agrees to use a short SN of 6-bits, as discussed in Section 2.2, it is not unlikely the SN will wrap-around (see the example in previous section) causing the receiver to mix up between old and new SDU segments based on SN. This would cause erroneous reassembly, possibly within a short time frame, depending on the used SN range.

In [3] fast HARQ failure detection is mentioned as an additional motivation for using a (discard) timer together with a window.

Based on the above we propose that similar timer a t-Reordering in LTE is used for NR RLC UM. Regarding the timer operation, we further prefer to keep the LTE design, see the Annex below for details.

RLC UM entity shall keep a receiver timer similar to t-Reordering in LTE.

# Conclusion

In section 2 we made the following observations:

Observation 1	If SN is used only for SDU segments, the receiver window and the receiver window movement need to be redefined.

Observation 2	Header length optimization of one or two bytes does not result in significant gains.

Observation 3	The implementation and processing complexity increase with the number of different header formats or lengths.

Based on the discussion in section 2 we propose the following:

Proposal 1	RLC UM mode should support both low data rate (VoIP-like) and high data rate (high TB size) use cases.

Proposal 2	All RLC UM PDUs include a SN.

Proposal 3	12-bit or 18-bit SN field is used for RLC UM and is configured together with PDCP SN range.

Proposal 4	PDU format for unsegmented SDU and first part of segmented SDU have the same header size.

Proposal 5	RLC UM entity shall keep a receiver window and receiver window state variables VR(UR), VR(UH), VR(UX) and UM_Window_Size (or corresponding variables using NR naming)

Proposal 6	RLC UM entity shall keep a receiver timer similar to t-Reordering in LTE.

# References

R2-1706794, “E-mail discussion summary of RLC UM”, Qualcomm Inc., RAN2#98-AH, Qingdao, China

R2-1704365, “RLC UM details for NR”, Ericsson, RAN2#98, Hangzhou, China

R2-1705778, “RLC UM operation for NR”, Sequans Communications, RAN2#98, Hangzhou, China

# Annex for information (based on 36.322)

### 5.1.2	UM data transfer

#### 5.1.2.2	Receive operations

##### 5.1.2.2.1	General

The receiving UM RLC entity shall maintain a receiver window according to state variable VR(UH) as follows:

-	a SN falls within the receiver window if (VR(UH) – UM_Window_Size) <= SN < VR(UH);

-	a SN falls outside of the receiver window otherwise.

When receiving an UMD PDU from lower layer, the receiving UM RLC entity shall:

-	place it in the reception buffer (see sub clause 5.1.2.2.2);

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Receiver as needed (see sub clause 5.1.2.2.3);

When t-Receiver expires, the receiving UM RLC entity shall:

-	update state variables, reassemble and deliver RLC SDUs to upper layer, discard segments which cannot be reassembled to complete SDU and start t-Receiver as needed (see sub clause 5.1.2.2.4).

##### 5.1.2.2.2	Actions when an UMD PDU is received from lower layer

When an UMD PDU with SN = x is received from lower layer, the receiving UM RLC entity shall:

-	place the received UMD PDU in the reception buffer.

##### 5.1.2.2.3	Actions when an UMD PDU is placed in the reception buffer

When an UMD PDU with SN = x is placed in the reception buffer, the receiving UM RLC entity shall:

-	if all segments with SN = x are received:

-	reassemble RLC SDU from all segments with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x falls outside of the receiver window:

-	update VR(UH) to x + 1;

-	if VR(UR) falls outside of the receiver window:

-	set VR(UR) to (VR(UH) – UM_Window_Size);

-	if  all segments with SN = VR(UR) are received:

-	update VR(UR) to the SN of the first UMD PDU with SN > current VR(URfor which not all segments have been received;

-	if t-Receiver is running:

-	if VR(UX) <= VR(UR); or

-	if VR(UX) falls outside of the reordering window and VR(UX) is not equal to VR(UH):

-	stop and reset t-Receiver;

-	if t-Receiver is not running (includes the case when t-Receiver is stopped due to actions above):

-	if VR(UH) > VR(UR):

-	start t-Receiver;

-	set VR(UX) to VR(UH).

##### 5.1.2.2.4	Actions when t-Receiver expires

When t-Receiver expires, the receiving UM RLC entity shall:

-	update VR(UR) to the SN of the first UMD PDU with SN >= VR(UX)  for which not all segments have been received;

-	reassemble RLC SDUs from any UMD PDUs with SN < updated VR(UR), remove RLC headers when doing so and deliver the reassembled RLC SDUs to upper layer if not delivered before;

-	discard all RLC SDU segments with SN < VR(UR)

-	if VR(UH) > VR(UR):

-	start t-Receiver;

-	set VR(UX) to VR(UH).

===============

# 7	Variables, constants and timers

## 7.1	State variables

Each receiving UM RLC entity shall maintain the following state variables:

a) VR(UR) – UM receive state variable

This state variable holds the value of the SN of the earliest UMD PDU that is still considered for segment reassembly. It is initially set to 0.

b) VR(UX) – UM t-Receiver state variable

This state variable holds the value of the SN following the SN of the UMD PDU which triggered t-Receiver.

c) VR(UH) – UM highest received state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs, and it serves as the higher edge of the reordering window. It is initially set to 0.

## 7.3	Timers

The following timers are configured by RRC:

x) t-Receiver

This timer is used by the receiving side of an UM RLC entity to detect loss of RLC PDUs at lower layer (see sub clauses 5.1.2.2). If t-Receiver is running, t-Receiver shall not be started additionally, i.e. only one t-Receiver per RLC entity is running at a given time.