3GPP TSG-RAN WG2-NR#2	R2-1707203

Qingdao, China, 27th – 29th June 2017

Agenda Item:	10.4.1.5.2

Source:	Panasonic

Title:	The format of the SI index/identifier

Document for:	Discussion, Decision

# Introduction

In RAN2#98, the following agreements have been made regarding to the content of the SI index/identifier and validity of the value tag across multiple cells [1]:

Agreement

There will be at least a value tag and area ID

-	value tag is associated to each SIB

-	value tag can be valid in only one cell or when combined with an area ID to be valid in more than one cell.

FFS whether the area ID and value tag is separately signalled or as a single identifier

FFS whether the area ID is associated to each SIB/ SI message or associated to a group of SIBs/ SI messages or all SIBs/ SI messages.

In this paper we would like to discuss the above two FFS items, and propose an efficient way to signal the SI index/identifier (value tag + area ID) while still making the configuration flexible.

# Discussion

It is difficult to know what the SI index (area ID + value tag) should look like in the minimum SI (or in other messages) before we can resolve the following two FFS items:

- Whether the area ID is associated to each SIB/ SI message or associated to a group of SIBs/ SI messages or all SIBs/ SI messages.

- Whether the area ID and value tag are separately signalled or as a single identifier.

Regarding to the first FFS, we should consider first whether the association between the area ID and the SI messages are configurable or fixed. If these are fixed, for example, there is only one area ID and it is associated with SI message 1 ~ 3, then only one area ID and several value tags will be signalled in the minimum SI, as shown in Figure 1(a). Since the overheads are small and fixed, using minimum SI to carry the area ID and value tag is affordable. On the other hand, if association are configurable, then we probably need to signal an area ID and a value tag for each SI message, as shown in Figure 1(b). This will of course create more signalling overheads but also allow the network to configure the association more flexibly and enable more deployment scenarios. For example, the network can make Area_ID_1, Area_ID_2, and Area_ID_3 equal to one value, and make Area_ID_4 and Area_ID_5 equal to another value, which means SI1, SI2 and SI3 are associated to one area, and SI4 and SI5 are associated to another area.

Observation 1: The dynamic association between the area ID and the SI messages creates more signalling overheads, but also provides more configuration flexibility and enables more deployment scenarios.

| SI Index |  | SI message |
| --- | --- | --- |
| Area_ID | value_tag_1 | SI1 |
|  | value_tag_2 | SI2 |
|  | value_tag_3 | SI3 |
|  | value_tag_4 | SI4 |
|  | value_tag_5 | SI5 |

| SI Index |  | SI message |
| --- | --- | --- |
| Area_ID_1 | value_tag_1 | SI1 |
| SI_Index _1 |  |  |
| Area_ID_2 | value_tag_2 | SI2 |
| SI_Index _2 |  |  |
| Area_ID_3 | value_tag_3 | SI3 |
| SI_Index _3 |  |  |
| Area_ID_4 | value_tag_4 | SI4 |
| SI_Index _4 |  |  |
| Area_ID_5 | value_tag_5 | SI5 |
| SI_Index _5 |  |  |

Figure 1(a) Fixed association.	              Figure 1(b) Dynamic association.

Figure 1. The signalling overheads comparison between the two options regarding to how to associate the area ID with the SI messages: a) fixed association; b) dynamic association.

Here we prefer to allow the network to freely configure the association between the area ID and SI messages, even if doing so could bring more signalling overheads. The overheads can be alleviated by using other signalling optimization methods as introduced later in this paper. If the network is allowed to configure the association between the area ID and the SI messages, there is no need to consider the first FFS question since all options in the first FFS question are possible.

Proposal 1: The association between the area ID and the SI messages are configurable, and more than one area can be configured.

To alleviate the overheads caused by the dynamic association between the area ID and the SI messages, as well as to avoid the hassle to create new network area, we prefer to reuse the existing area such as the NAS tracking area, the RAN-based notification area, CSG, or even the cell as the SI area. Therefore, instead of signalling the real area ID as the part of SI index/identifier, the area ID is actually used to indicate a certain area scope. For example, area ID ‘00’ means in the same scope as in the NAS tracking area, area ID ‘01’ means in the same scope as in the RAN-based notification area, and so on and so forth. The real area ID can be signalled somewhere in the minimum SI.

Proposal 2: Area ID and value tag are signalled as a single identifier (SI index/identifier), where the area ID is used to indicate the area scope.

# Conclusion

In this paper, we discuss the two FFS items in the stored SI section, have the following observation.

- Observation 1: The dynamic association between the area ID and the SI messages creates more signalling overheads, but also provides more configuration flexibility and enables more deployment scenarios.

Based on the observation, RAN2 is kindly asked to approve the following proposals.

Proposal 1: The association between the area ID and the SI messages are configurable, and more than one area can be configured.

Proposal 2: Area ID and value tag are signalled as a single identifier (SI index/identifier), where the area ID is used to indicate the area scope.

# References

Draft Report of 3GPP TSG RAN WG2 meeting #98.