3GPP TSG-RAN WG2 NR#2	Tdoc R2-1707236

Qingdao, China, 27th – 29th June 2017

Agenda Item:	10.4.1.5.1

Source:	Ericsson

Title:	SCG bearer establishment

Document for:	Discussion, Decision

# Introduction

In RAN#98, the following was agreed:

Agreement

1	For MR DC, it is allowed that all DRBs are configured in SCG (i.e. no DRBs configured in MCG) (assuming that a DRB was previously established in MCG.

The intention of the last part of the agreement is somewhat unclear, and we would like to clarify it in this contribution.

# Discussion

As the agreement is written, there can be two interpretations of the last sentence:

- Prior to MR-DC with all DRBs in SCG, there must be MR-DC with a DRB established in MCG

- Prior to MR-DC with all DRBs in SCG, there must be a DRB established in the node, which once MR-DC is established will control the MCG

With the first interpretation, it would mean that two reconfigurations are needed to reach the situation where all DRBs are configured in SCG. The first reconfiguration to setup the MR-DC, with one DRB in MCG. Then a second reconfiguration to move all DRBs to the SCG.

We don’t see a need for such limitation, and would prefer the second interpretation, in which all DRBs can be configured directly under the SCG, in the reconfiguration setting up the MR-DC. This was also the intention in [1], which was the contribution discussed when taking agreement 1. Then, the last sentence becomes in fact superfluous, since there is always at least one DRB configured, also in single connectivity.

Therefore, we propose the following:

For MR DC, it is allowed that all DRBs are configured in SCG (i.e. no DRBs configured in MCG)

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	For MR DC, it is allowed that all DRBs are configured in SCG (i.e. no DRBs configured in MCG)

# References

R2-1705142, Direct SCG (Split) Bearer Setup in EN DC	Huawei, HiSilicon