3GPP TSG-RAN WG2 NR Adhoc#2								R2-1707416

Qingdao, China, 27 – 29 June, 2017

Title: 	DL Flow control for EN-DC

Source: 	Huawei, HiSilicon

Agenda item:	10.2.5

Document for:	Discussion and decision

Revision of:	R2-1705143

Introduction

For LTE-NR dual connectivity, different kinds of bearer types will be supported in variant deployments, take E-UTRA is the master as an example, the protocol stack of these bearer types could be illustrated as below:

In this contribution, we analyse the DL flow control in case of EN-DC.

Discussion

In LTE, the purpose of the Downlink Data Delivery Status (DDDS) procedure is to provide feedback from the SeNB to the MeNB to allow the MeNB to control the downlink user data flow via the SeNB for the respective E-RAB.

In NR, the main difference comparing with LTE includes SCG split bearer, higher data rate, high reliability, and PDCP duplication. In this section, we analyses the flow control function related to these aspects.

- Support of SCG split bearer

Due to the introduction of SCG split bearer, the secondary node will split packets received from core network of the bearer, in order to adjust the data amount of this bearer to be transmitted in master node and secondary node, it will also be needed for the master node to provide DDDS report to the secondary node.

Proposal 1: support bi-direction Downlink Data Delivery Status procedure in X2/Xn interface between master node and secondary node, to support MCG split bearer DL flow control and SCG split bearer DL flow control.

- Support of higher data rate

In LTE, it was up to SeNB decision on when to trigger the Downlink Data Delivery Status procedure. As the MeNB will remove the buffered PDCP PDUs according to the feedback of successfully delivered PDCP PDUs, if the SeNB report the status report infrequently, the MeNB has to buffer very large amount of PDCP PDUs.

In 5G, the data rate will significantly increase comparing with Rel-12/13 LTE, i.e. target peak data rate 20Gbps for downlink and 10Gbps for uplink. In order to provide the DDDS in a more efficient way, it is better to introduce these procedures in a periodic way, or to be triggered after successfully delivered an indicated number of PDCP PDUs.

As shown in the figure below, the anchor Node1of a bearer which receiving DL data from Core network, will indicate the periodic reporting timer or the packet amount to report to the splitting Node2 which will help to transmit the data towards to the UE for the split bearer, e.g. during the split bearer setup. Then the splitting Node2 will trigger the DL data delivery status procedure towards the anchor Node1 periodically or after the delivery of the indicated amount of packets.

In case of pool radio condition, the secondary node may not able to provide the status report in a timely manner in case the “packet amount to report” mechanism is used, hence it is better to select the periodic reporting.

Proposal 2: enable the anchor node to request the splitting node to provide the Downlink Data Delivery Status report, i.e. by introducing a “periodic reporting timer” in Bearer setup signalling or DL USER DATA PDU header.

- Support of High reliability

In LTE DC, the SeNB only indicates the successfully delivered PDCP Sequence Number, and the lost X2-U sequence Number to the MeNB, but the undelivered PDCP sequence number was not provided to the MeNB.

In 5G, high reliability transmission is required in many scenarios. as shown in the figure below, in case the splitting node2 is not able to deliver a PDCP PDU (e.g. PDU No.4) towards the UE (e.g. based on RLC status report from UE) while the following PDCP PDU are delivered successfully (e.g. radio condition gets better), as the splitting node does not have the information on the PDCP discard timer, it is not able to determine whether the packet need to be retransmitted or discarded by the splitting node, if the splitting node inform the non-delivered PDCP PDU SN to the anchor node, the anchor node will make the decision on whether to discard the packet or resend the packet to the UE.

Proposal 3: support non-delivered packet reporting, by e.g. provide the “non-delivered PDCP sequence number” in Downlink Data Delivery Status message.

- Support of PDCP duplication

As it was agreed in RAN2 that PDCP duplication is supported in some scenarios. in case DPCP duplication is used in DC scenarios, in order to avoid unnecessary DL radio resource waste, it is better for one node to remove the buffered PDCP PUDs in case another node has already successfully transmit the PDU to the UE.,

Based on current DDDS report, in case the anchor node1 receives the successfully delivered PDCP Sequence Number, the anchor node1 could not only remove the buffered PDCP PDUs for the split bearer, but also remove the PDCP packet in the duplication path, this can avoid the unnecessary redundant PDCP PDU delivery in different path.

On the other hand, if the anchor node has already delivered some PDCP PDUs (PDU No. 1, 2, 3 and 4 in Figure below) in one path, e.g. in MCG bearer, if there is a split bearer carries the same PDCP PDUs, it is also beneficial for the anchor node to inform the delivered PDU SN, and then the splitting node could remove the indicated PDCP PDUs (PDU No. 3 and 4) in the buffer storing “PDCP PDUs not start to deliver”. Note it could be achieved via e.g. allowing the Anchor node hosting PDCP to provide DDDS report to the splitting node.

Proposal 4: in case of PDCP duplication in DC, enable the anchor node to indicate the delivered PDU SN to the splitting node, to allow the splitting node to remove the PDCP PDU in the buffer.

Conclusion

In this contribution, we analyse the flow control for EN-DC, and it is proposed to agree the following proposals as the flow control principles for Dual connectivity:

Proposal 1: support bi-direction Downlink Data Delivery Status procedure in X2/Xn interface between master node and secondary node, to support MCG split bearer DL flow control and SCG split bearer DL flow control.

Proposal 2: enable the anchor node to request the splitting node to provide the Downlink Data Delivery Status report, i.e. by introducing a “periodic reporting timer” in Bearer setup signalling or DL USER DATA PDU header.

Proposal 3: support non-delivered packet reporting, by e.g. provide the “non-delivered PDCP sequence number” in Downlink Data Delivery Status message.

Proposal 4: in case of PDCP duplication in DC, enable the anchor node to indicate the delivered PDU SN to the splitting node, to allow the splitting node to remove the PDCP PDU in the buffer.