3GPP TSG-RAN WG2#AH1807	R2-1809709

# Montreal, Canada, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0057 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.300 to correct signalling flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some of the signalling flows described in the Stage 2 specification are not correct according to the agreements taken during the the latest 3GPP meetings (e.g. wrong RRC message names or incorrect messages flows). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates of the figures and description of the incorrect signalling flows. - Figure 9.2.2.4.1-1 has been updated with the RRCResumeComplete message; - Figure 9.2.2.4.2-1 has been updated with missing parts; - Figure 9.2.2.5-1 and has been updated considering that path switch should occur before the UE can be suspended in the 2-step RNA update (2-hop security). Text has been updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequence is the Stage 2 specification contains incorrect information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.4.1 UE triggered transition from RRC_INACTIVE to RRC_CONNECTED 9.2.2.4.2 Network triggered transition from RRC_INACTIVE to RRC_CONNECTED 9.2.2.5 RNA update |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF THE CHANGE

#### 9.2.2.4	State Transitions

##### 9.2.2.4.1	UE triggered transition from RRC_INACTIVE to RRC_CONNECTED

The following figure describes the UE triggered transition from RRC_INACTIVE to RRC_CONNECTED:

UE

gNB

Last Serving gNB

AMF

UE in RRC_INACTIVE /
CM-CONNECTED

1. RRCResumeRequest

4. RRCResume

2. RETRIEVE UE CONTEXT REQUEST

5. DATA FORWARD ADDRESS INDICATION

3. RETRIEVE UE CONTEXT RESPONSE

8. UE CONTEXT RELEASE

7. PATH SWITCH REQUEST ACKNOWLEDGE

6. PATH SWITCH REQUEST

UE in RRC_CONNECTED /
CM-CONNECTED

4a. RRCResumeComplete

Figure 9.2.2.4.1-1: UE triggered transition from RRC_INACTIVE to RRC_CONNECTED

1.	The UE resumes from RRC_INACTIVE, providing the I-RNTI, allocated by the last serving gNB.

2.	The gNB, if able to resolve the gNB identity contained in the I-RNTI, requests the last serving gNB to provide UE Context data.

3.	The last serving gNB provides UE context data.

4./4a.	The gNB completes the resumption of the RRC connection.

5.	If loss of DL user data buffered in the last serving gNB shall be prevented, the gNB provides forwarding addresses.

6./7. The gNB performs path switch.

8.	The gNB triggers the release of the UE resources at the last serving gNB.

After step 1 above, when the gNB decides to reject the Resume Request and keep the UE in RRC_INACTIVE without any reconfiguration, or when the gNB decides to setup a new RRC connection, SRB0 (without security) can be used. When the gNB decides to reconfigure the UE (e.g. with a new DRX cycle or RNA) or when the gNB decides to push the UE to RRC_IDLE, SRB1 (with at least integrity protection) shall be used.

NOTE:	SRB1 can only be used once the UE Context is retrieved i.e. after step 3.

##### 9.2.2.4.2	Network triggered transition from RRC_INACTIVE to RRC_CONNECTED

The following figure describes the network triggered transition from RRC_INACTIVE to RRC_CONNECTED:

UE

gNB

Last Serving gNB

AMF

UE in RRC_INACTIVE /
CM-CONNECTED

2. Paging

2a. RAN PAGING

1. RAN Paging trigger

3. Paging

4. Resuming from RRC_INACTIVE

Figure 9.2.2.4.2-1: Network triggered transition from RRC_INACTIVE to RRC_CONNECTED

1.	A RAN paging trigger event occurs (incoming DL user plane, DL signalling from 5GC, etc.).

2./2a.	RAN paging is triggered; either only in the cells controlled by the last serving gNB or also by means of Xn RAN Paging in cells controlled by other gNBs, configured to the UE in the RAN-based Notification Area (RNA).

3.	The UE is paged with the I-RNTI.

4.	If the UE has been successfully reached, it attempts to resume from RRC_INACTIVE, as described in sub-clause 9.2.2.4.1.

….

#### 9.2.2.5	RNA update

The following figure describes the UE triggered RNA update procedure when it moves out of the configured RNA involving context retrieval over Xn:

UE

gNB

Last Serving gNB

AMF

UE in RRC_INACTIVE /
CM-CONNECTED

1. RRCResumeRequest
(RNA Update)

7. RRCRelease
(Suspend indication)

2. RETRIEVE UE CONTEXT REQUEST

4. DATA FORWARD ADDRESS INDICATION

3. RETRIEVE UE CONTEXT RESPONSE

8. UE CONTEXT RELEASE

6. PATH SWITCH REQUEST ACKNOWLEDGE

5. PATH SWITCH REQUEST

Figure 9.2.2.5-1: RNA update procedure

1.	The UE resumes from RRC_INACTIVE, providing the I-RNTI allocated by the last serving gNB and appropriate cause value, e.g., RAN notification area update.

2.	The gNB, if able to resolve the gNB identity contained in the I-RNTI, requests the last serving gNB to provide UE Context.

3.	The last serving gNB provides UE context. The gNB may move the UE to RRC_CONNECTED, or to RRC_IDLE or keep the UE in RRC_INACTIVE state:
- If the UE is moved to RRC_CONNECTED, RRCResume message is sent to the UE and the procedure continues as in Ch.9.2.2.4.1.
- If the UE is moved to RRC_IDLE, RRCRelease message is sent to the UE and the procedure ends.
- If the UE is moved back to RRC_INACTIVE, the procedure continues with the following steps..

4.	If loss of DL user data buffered in the last serving gNB shall be prevented, the gNB provides forwarding addresses.

5./6. The gNB performs path switch.

7.	The gNB moves the UE back to RRC_INACTIVE state by sending RRCRelease with suspend indication.

8.	The gNB triggers the release of the UE resources at the last serving gNB.

END OF THE CHANGE