3GPP TSG-RAN WG2 Meeting NR AH 1807	R2-1809722

Montreal, Canada, 2nd – 6th July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0212 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on BWP switching for RA-BFR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 agreed to support linkage between UL and DL BWPs for the random access procedure. One consequence is this may introduce an unnecessary DL BWP switch after detecting beam failure on a certain DL BWP. Such switch may be an issue if the old and new DL BWPs do not overlap or if they have different CSI-RS experiencing different fading conditions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following sections have been changed: - Section 5.15: if the RA is initiated due to BFR, the UE does not switch its active DL BWP. - Section 5.17: Upon beam failure detection, the UE switches the active UL BWP to the UL BWP with the same bwp-Id as the active DL BWP, if the active UL BWP does not have the same bwp-Id as the active DL BWP Impact analysis Impacted functionality: Beam Failure Recovery, Bandwidth Part Operation Inter-operability: If the UE implements the CR and the network does not, the UE may switch its active UL BWP while the network assumes the UE has not. If the network implements the CR and the UE does not, the network may send the RAR on a DL BWP different from the UE’s active DL BWP. The network assumes the UE has not sitched its DL BWP though the UE may switch its DL BWP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Upon initiation of RA for BFR, UE may select a dedicated preamble corresponding to a beam that has not been measured while the BFI counter was increasing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15 and 5.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification 1>

## 5.15	Bandwidth Part (BWP) operation

In addition to clause 12 of TS 38.213 [6], this subclause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure. Upon addition of SpCell or activation of an SCell, the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

For each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	if a BWP is activated:

2>	transmit on UL-SCH on the BWP;

2>	transmit on RACH on the BWP;

2>	monitor the PDCCH on the BWP;

2>	transmit PUCCH on the BWP;

2>	transmit SRS on the BWP;

2>	receive DL-SCH on the BWP;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2.

1>	if a BWP is deactivated:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

Upon initiation of the Random Access procedure on a Serving Cell, the MAC entity shall for this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	switch the active UL BWP to BWP indicated by initialUplinkBWP;

2>	if the Serving Cell is a SpCell:

3>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is a SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP; and

3>	if the Random Access procedure was not initiated for beam failure recovery (see subclause 5.17):

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a serving cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in subclauses 5.1.4 and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure on the new activated BWP; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the active BWP.

If the bwp-InactivityTimer is configured, the MAC entity shall for each activated Serving Cell:

1>	if the defaultDownlinkBWP is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP; or

1>	if the defaultDownlinkBWP is not configured, and the active DL BWP is not the initialDownlinkBWP:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment:

3>	if there is no ongoing random access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if a PDCCH for BWP switching is received on the active DL BWP, and the MAC entity switches the active BWP:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if Random Access procedure is initiated on this Serving Cell:

3>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

3>	if the Serving Cell is SCell:

4>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP.

3>	else:

4>	perform BWP switching to the initialDownlinkBWP.

<end of modification 1>

<Start of modification 2>

## 5.17	Beam Failure Detection and Recovery procedure

The MAC entity may be configured by RRC with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity.

RRC configures the following parameters in the BeamFailureRecoveryConfig for the Beam Failure Detection and Recovery procedure:

-	beamFailureInstanceMaxCount for the beam failure detection;

-	beamFailureDetectionTimer for the beam failure detection;

-	beamFailureRecoveryTimer for the beam failure recovery procedure;

-	rsrp-ThresholdSSB: an RSRP threshold for the beam failure recovery;

-	powerRampingStep: powerRampingStep for the beam failure recovery;

-	preambleReceivedTargetPower: preambleReceivedTargetPower for the beam failure recovery;

-	preambleTransMax: preambleTransMax for the beam failure recovery;

-	ra-ResponseWindow: the time window to monitor response(s) for the beam failure recovery using contention-free Random Access Preamble;

-	prach-ConfigIndex: prach-ConfigIndex for the beam failure recovery;

-	ra-ssb-OccasionMaskIndex: ra-ssb-OccasionMaskIndex for the beam failure recovery;

-	ra-OccasionList: ra-OccasionList for the beam failure recovery.

The following UE variables are used for the beam failure detection procedure:

-	BFI_COUNTER: counter for beam failure instance indication which is initially set to 0.

The MAC entity shall:

1>	if beam failure instance indication has been received from lower layers:

2>	start or restart the beamFailureDetectionTimer;

2>	increment BFI_COUNTER by 1;

2>	if BFI_COUNTER >= beamFailureInstanceMaxCount:

3>	if beamFailureRecoveryConfig is configured:

4> if the active UL BWP does not have the same bwp-Id as the active DL BWP:

5> switch the active UL BWP to the UL BWP with the same bwp-Id as the active DL BWP;

4>	start the beamFailureRecoveryTimer, if configured;

4>	initiate a Random Access procedure (see subclause 5.1) on the SpCell by applying the parameters powerRampingStep, preambleReceivedTargetPower, and preambleTransMax configured in beamFailureRecoveryConfig.

3>	else:

4>	initiate a Random Access procedure (see subclause 5.1) on the SpCell.

1>	if the beamFailureDetectionTimer expires:

2>	set BFI_COUNTER to 0.

1>	if the Random Access procedure is successfully completed (see subclause 5.1):

2>	stop the beamFailureRecoveryTimer, if configured;

2>	consider the Beam Failure Recovery procedure successfully completed.

<end of modification 2>