3GPP TSG-RAN WG2 NR AH1807 Meeting	R2-1809828

(Revision of R2-1808302)

Montreal, Canada, 2nd - 6th July 2018

Agenda item:	10.2.4

Source:	Samsung

Title:	MR-DC bearer management in inactive state

Document for:	Discussion & Decision

1 Introduction

Previously in RAN2#99 meeting, RAN2 agreed not to support resume of DC configurations from suspend /inactive state [1]:

Agreements: For MR-DC and NR-DC we will not support that the UE can resume the DC configuration after the UE returns from suspended/inactive in Rel-15.

In RAN2#99bis meeting, SA2 requests to RAN2 in LS (S2-176158) [2].

SA2 prefers that dual connectivity configuration is maintained while the UE is in RRC inactive in order to potentially avoid unnecessary signalling between the RAN and CN, and within the CN, whenever the UE moves between RRC connected and RRC inactive states. In this respect it is ideal if the UE resumes the RRC connection in the Master RAN node, and if based on RAN conditions the Secondary RAN node configuration remains valid, the Secondary RAN node configuration is expected to be resumed too. SA2 requests RAN2 and RAN3 to study and provide specification for the signalling between the Secondary RAN node and Master RAN node if dual connectivity has been established for the UE for which there is at least one N3 tunnel termination to the Secondary RAN node for at least one 5G QoS Flow.

Then RAN2 replied to SA2 in R2-1712063 [3] as below.

RAN2 thanks SA2 for the LS on coexistence between RRC inactive and dual connectivity (S2-176158). RAN2 had discussed this in RAN2#99 and agreed not to maintain the dual connectivity configuration when UE returns from suspended/inactive in Rel-15. RAN2 is still discussing what this implies in terms of SDAP/PDCP configurations in the SCG.

With related discussion, RAN2 discussed in RAN2#100 as follows:

R2-1712613 Handling of EN-DC with suspend/resume ZTE Corporation, Sane Chips discussion Rel-15 - Intel thinks the latest RRC CR addresses this. The SCG configuration is always released for re-establishment and resume. For the radio bearer configurations they need to be kept and the MN can keep them or reconfigure them at re-establishment. For suspend case there are more cases that need to be discussed. - Ericsson think it is important to keep the PDCP configuration of the bearer. - Samsung think that for release 13 suspend the S1 connection is released completely so why is this needed. - Ericsson think in R13 the SCG config is released when the UE resumes, but now for R15 we can keep the bearer configuration but release the SCG configuration. Intel think this is sufficient from the UE point of view. - Samsung think keeping the bearer configuration only makes sense if the S1 is kept. => Noted

Although RAN2 clearly agreed to “not support that the UE can resume the DC configuration after the UE returns from suspended/inactive in Rel-15”, there’s SA2’s preference “that dual connectivity configuration is maintained while the UE is in RRC inactive” to reduce the signalling overhead.  Hence, now RAN2 have the further study on “what this implies in terms of SDAP/PDCP configurations in the SCG” especially from MR-DC context.

In this contribution, we discuss when and how SN (SCG bearer) can be released and what shall be done for the SN context while DC configuration is not supported for resumption from suspend/ inactive in Rel-15.

2 Discussion

From the previous agreements and LSs, we can summarize the agreements from RAN2 and SA2 as follows:

- RAN2 agreed not to maintain the dual connectivity configuration when UE returns from suspended/inactive in Rel-15

- RAN2 is still discussing what this implies in terms of SDAP/PDCP configurations in the SCG.

- SA2 prefers that dual connectivity configuration is maintained while the UE is in RRC inactive in order to potentially avoid unnecessary signalling

Based on the above agreement, UE re-configuration scenarios shown in Figure 1 can be considered upon transition to the INACTIVE state.

- Keep SDAP/PDCP configurations of SN: Figure 1(a) shows that SDAP/PDCP configuration of SN (SCG bearer) is maintained while RLC/MAC/PHY configurations are released, and the both of MN and SN are connected to CN.

- Release all the configurations of SN: Figure 1(b) shows that all the configurations of SN are released. The bearer terminated at SN can be re-located to MN before suspension, or just released. Also for MN terminated split bearer should be reconfigured to MN terminated MCG bearer. I.e. following two sub-procedures are required depending on the bearer configurations: (b-1) PDCP termination point change from SN PDCP to MN PDCP and (b-2) Bearer type change from split bearer to MCG bearer before suspension.

| - Keep SDAP/PDCP configurations of SN while releasing RLC/MAC/PHY layer configurations | - Release all the configurations of SN |
| --- | --- |

Figure 1: Example of two options (a) keep SN SDAP/PDCP configuration or (b) release all the SN configurations.

Then the additional signaling for suspension and resumption are summarized in the following Table 1.

Table 1. Comparison of required Xn/NG signalling

| Options |  | (a) Keep SN SDAP/PDCP configuration | (b) Release all the SN configurations |  |
| --- | --- | --- | --- | --- |
|  |  |  | (b-1) PDCP termination point change from SN PDCP to MN PDCP before suspension | (b-2) Bearer type change from split bearer to MCG bearer before suspension |
| Required signalling at suspension | Xn | Marginal | Marginal | Marginal |
|  | CN | Not required : SDAP/PDCP configuration is saved but SCG configuration is released when suspension | Required : Relocation of the SN terminated bearers requires CN signalling. | Not required : the split bearer to MCG bearer change does not require CN signalling. |
| Required signalling at resumption | Xn | Required : SCG configuration needs to be reconfigured | Required : SCG configuration needs to be reconfigured | Required : SCG configuration needs to be reconfigured |
|  | CN | Not required : no CN signalling is needed | Required : when the SN terminated bearers are reconfigured, CN signalling is needed to re-establish tunnel with UPF | Not required |

It is very clear that keeping SDAP/PDCP configuration requires the least NG signalling.

Observation 1: Keeping SDAP/PDCP configuration upon transition to INACTIVE can save additional signalling for suspension and DC re-configuration after resumption.

With option (a), the DRBs in SN are not released but suspended and the connection between SGW/UPF and SN is maintained. In order to realize the option (a), new RAN features including RAN-based paging initiated by SN and RAN procedure to keep dual connectivity after RNAU might be needed:

- RAN-based paging initiated by SN: if there are DL packets arriving for DRBs configured as SN-terminated DRBs, then INACTIVE UE shall be paged. This would require SN triggering RAN-based paging, and SN needs to indicate the MN to page the UE.

- Keeping dual connectivity after RNAU: if UE moves out of RAN Notification Area (RNA), RAN3 need to support new RNAU procedures to support re-establishing dual connection relationship between new MN (after RNAU) and the same SN considering the case that CU of SN would have wider coverage.

Even though one could argue that all the aforementioned RAN enhancements to keep the SN context are fully constrained in RAN3 and are not visible to UE, it anyway requires non-trivial amount of standardization efforts. Nevertheless, it could be premature to exclude immediately a possibility to keep the SN context, especially accounting for the fact that it is completely transparent to the UE. Based on these considerations we propose that it could be up to the network deployment and implementation to choose which option to follow. If the network deployment and implementation does not allow for keeping the SN context upon transition to INACTIVE, the network will command UE to change/release the corresponding bearers. If RAN3 related enhancements become available and are supported by the network, the network can always ask a UE to keep the current bearer configuration.

The suspension of the SN bearers should be feasible in option 2 (NR-NR DC). Option 4 and option 7 need further discussions because even though both the MN and SN are connected to 5GC, final design of the INACTIVE state for LTE connected to 5GC is not clear yet.

Proposal 1: Both options of keeping SN SDAP/PDCP configuration and releasing all the SN configurations should be supported for the future flexibility (to account further decision in RAN3).

Proposal 2: It is up to the network implementation whether to re-configure SN-terminated bearers and MN-terminated split bearers or to keep them.

Some additional RAN2 enhancements were proposed to reconfigure SCG radio configuration early at connection resumption:

- Early measurement report to help gNB configure SCG leg at connection resumption

- Indication on whether some of the last SCG cells at connection suspension are still the best cells on SN layers from UE

The enhancements will require substantial amount of meeting time in RAN2 and also RAN4 to discuss brand new RRC procedures and corresponding performance requirements. Considering that the late drop has wide work scope (i.e. architecture option 4, 7 and probably NR-DC) with limited number of meetings, it is proposed not to pursue these enhancements in rel.15. Blind SCG addition that has been available and supported from legacy LTE-DC can be used instead.

Proposal 3:	In MR-DC, SCG radio configuration can be reconfigured blindly at connection resumption. No additional enhancement is considered in rel.15.

3 Conclusion

RAN2 is requested to discuss and agree on the following proposals for suspension and resumption to/from inactive state in MR dual connectivity:

Observation 1: Keeping SDAP/PDCP configuration upon transition to INACTIVE can save additional signalling for suspension and DC re-configuration after resumption.

Proposal 1: Both options of keeping SN SDAP/PDCP configuration and releasing all the SN configurations should be supported for the future flexibility (to account further decision in RAN3).

Proposal 2: It is up to the network implementation whether to re-configure SN-terminated bearers and MN-terminated split bearers or to keep them.

Proposal 3:	In MR-DC, SCG radio configuration can be reconfigured blindly at connection resumption. No additional enhancement is considered in rel.15.

References

- RAN2#99 Chairman’s report

- S2-176158 LS on coexistence between RRC inactive and dual connectivity Qualcomm

- R2-1712063 Reply LS on coexistence between RRC inactive and dual connectivity Intel Corporation

- R2-1708808 Inactive and dual connectivity Intel Corporation

- R2-1711665 Suspension to INACTIVE in NR Dual connectivity Samsung