3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1810113

# Montreal, Canada, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0052 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RRC Resume procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is specified that if the receiving gNB does not find a valid UE Context, gNB performs establishment of a new RRC connection instead of resumption of the previous RRC connection. However, according to TS 38.331 the gNB can also send RRC Reject as a response for RRC Resume Request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that gNB may reject the resume attempt or gNB may perform establishment of a new RRC connection instead of resumption of the previous RRC connection. Impact analysis Impacted functionality: RRC Resume procedure. Inter-operability: - None foreseen as the corresponding requirements are already captured in Stage 3 specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and stage 3 specifications are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 9.2.2	Mobility in RRC_INACTIVE

#### 9.2.2.1	Overview

RRC_INACTIVE is a state where a UE remains in CM-CONNECTED and can move within an area configured by NG-RAN (the RNA) without notifying NG-RAN. In RRC_INACTIVE, the last serving gNB node keeps the UE context and the UE-associated NG connection with the serving AMF and UPF.

If the last serving gNB receives DL data from the UPF or DL signalling from the AMF (except the UE Release Command and Reset messages) while the UE is in RRC_INACTIVE, it pages in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s).

If the last serving NG-RAN node receives the UE Release Command message from the AMF while the UE is in RRC_INACTIVE, it shall reply with the UE Context Release Complete message.

Upon RAN paging failure, the gNB behaves according to 3GPP TS 23.501 [3].

The AMF provides to the NG-RAN node the RRC Inactive Assistant Information to assist the NG-RAN node's decision whether the UE can be sent to RRC_INACTIVE. The RRC Inactive Assistant Information includes the registration area configured for the UE, the UE specific DRX, Periodic Registration Update timer, an indication if the UE is configured with Mobile Initiated Connection Only (MICO) mode by the AMF, and UE Identity Index value. The UE registration area is taken into account by the NG-RAN node when configuring the RAN-based notification area. The UE specific DRX and UE Identity Index value are used by the NG-RAN node for RAN paging. The Periodic Registration Update timer is taken into account by the NG-RAN node to configure Periodic RAN Notification Area Update timer.

At transition to RRC_INACTIVE the NG-RAN node may configure the UE with a periodic RNA Update timer value. At periodic RNA Update timer expiry without notification from the UE, the gNB behaves as specified in 3GPP TS 23.501 [3].

If the UE accesses a gNB other than the last serving gNB, the receiving gNB triggers the XnAP Retrieve UE Context procedure to get the UE context from the last serving gNB and may also trigger a Data Forwarding procedure including tunnel information for potential recovery of data from the last serving gNB. Upon successful UE context retrieval, the receiving gNB shall perform the slice-aware admission control in case of receiving slice information and becomes the serving gNB and it further triggers the NGAP Path Switch Request procedure. After the path switch procedure, the serving gNB triggers release of the UE context at the last serving gNB by means of the XnAP UE Context Release procedure.

In case the UE is not reachable at the last serving gNB, the gNB shall fail AMF initiated UE-associated class 1 procedures if any, and shall trigger the NAS Non Delivery Indication procedure to report the non-delivery of any NAS PDUs received from the AMF for the UE.

If the UE accesses a gNB other than the last serving gNB and the receiving gNB does not find a valid UE Context, gNB may reject the resume attempt or gNB may perform establishment of a new RRC connection instead of resumption of the previous RRC connection.

A UE in the RRC_INACTIVE state is required to initiate RNA update procedure when it moves out of the configured RNA. When receiving RNA update request from the UE, the receiving gNB triggers the XnAP Retrieve UE Context procedure to get the UE context from the last serving gNB and may decide to send the UE back to RRC_INACTIVE state, move the UE into RRC_CONNECTED state, or send the UE to RRC_IDLE.