3GPP TSG RAN WG2 AH-1807 Meeting 		R2-1810122

Montreal, Canada, July 2nd – 6th, 2018

Agenda Item:	11.2

Source:	Sony

Title:	Considerations on channel access, LBT and mobility enhancements for NR unlicensed operations

Document for: Discussion / Decision

# Introduction

A new Study Item on “Study on NR-based Access to Unlicensed Spectrum” was approved in 3GPP RAN#75 meeting. The study includes the following objectives [1]:

- Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

  - Physical channels inheriting the choices of duplex mode, waveform, carrier bandwidth, subcarrier spacing, frame structure, and physical layer design made as part of the NR study and avoiding unnecessary divergence with decisions made in the NR WI

    - Consider unlicensed bands both below and above 6GHz, up to 52.6GHz

      - Consider unlicensed bands above 52.6GHz to the extent that waveform design principles remain unchanged with respect to below 52.6GHz bands

    - Consider similar forward compatibility principles made in the NR WI

  - Initial access, channel access. Scheduling/HARQ, and mobility including connected/inactive/idle mode operation and radio-link monitoring/failure

  - Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz , 37GHz, 60GHz bands

The above study will address the following architectural scenarios (RAN2):

  - An NR-based LAA cell(s) connects with an LTE or NR anchor cell operating in licensed spectrum

    - The study assumes the techniques for linking between Pcell (LTE or NR licensed CC) and Scell (NR unlicensed CCs) according to the NR WI

  - An NR-based cell operating standalone in unlicensed spectrum, connected to a 5G-CN network with priority on frequency bands above 6GHz, e.g., for private network deployments;

  - Study how to ensure from a RAN level that connection and security management can be integrated with the E-UTRAN, NG RAN and 5G CN architecture, including service continuity requirements for users moving between cells of licensed and unlicensed frequency bands, liaising with SA2 as required

In RAN1#92bis meeting, the following agreement was made [2].

Agreement:

- Study possible enhancements for HARQ operation

- Study changes needed for Configured Grant support in NR-U

- Baseline for study: If absence of Wi-Fi cannot be guaranteed (e.g. by regulation)

in the band (sub-7 GHz) where NR-U is operating, the NR-U operating bandwidth is an integer

multiple of 20MHz

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

- Study whether or not the following techniques enhance performance beyond the baseline LBT mechanisms

  - Techniques to cope with directional antennas/transmissions

  - Receiver assisted LBT : RTS/CTS type mechanism

    - On-demand receiver assisted LBT: For example receiver assisted LBT enabled only when needed

  - Techniques to enhance spatial reuse

  - Preamble detection

  - Enhancements to baseline LBT mechanisms above 7 GHz

- Note: LTE-LAA LBT mechanism are assumed as baseline for evaluations for 5GHz.

- Note: Other aspects are not precluded from being included

At the RAN2#102 meeting, the following agreements were made [5].

Agreements

1:	Both CBRA and CFRA are supported. Changes for NR-U operation will be studied

2:	4-step and 2 step CBRA procedure will be studied in conjunction with RAN1 progress

3: 	We will review the agreements made during Rel-14 eLAA WI regarding the random access procedure to determine if they can be the solution for CFRA access for NR-U

At RAN2#102 meeting also the following email discussion was initiated [6],

[102#69][NR] NR-U mobility (Qualcomm)

Identify the issues that could need changes to:

Measurement framework (e.g. SMTC configuration and its provisioning, additional measurements such as channel load)

RRM (e.g. applicability of events and how to guarantee robustness)

Any changes to the RRC procedures for measurements and handover

Considerations on Idle/Inactive cell reselection (e.g. changes to cell quality and ranking)

Intended outcome: Report to next meeting

Deadline:  Thursday 2018-06-14

In this contribution, we will discuss challenges and design options related to Frequency usage and Directional LBT, which we think RAN1and RAN2 should consider for unlicensed NR with respect to channel access procedures.

# Discussion

## Frequency usage

As agreed in RAN1#92bis the baseline for this study is that If absence of Wi-Fi cannot be guaranteed (e.g. by regulation) in the band (sub-7 GHz) where NR-U is operating, the NR-U operating bandwidth is an integer multiple of 20MHz. And further, for the same case LBT can be performed in units of 20MHz.

Within NR the BWP concept has been introduced, and this can be useful for such operation of NR with channel sensing split into smaller units of the total bandwidth. If the total bandwidth is split into multiple units using BWPs, each BWP could consist of one or multiple 20MHz units where the LBT procedure could operate per 20MHz unit. In case multiple BWPs are configured it could be beneficial if reconfigurations of the used BWPs over time could be done quickly, reducing the interference due to coexistence depending on the variations in channel utilization from different networks and systems.

In licensed NR, BWPs are activated via RRC signaling and DCI. However for use in unlicensed bands there are likely to be some limitations and drawbacks in the current BWP specifications for licensed NR operation. In order to utilize the BWP concept for fast and efficient access to the unlicensed channel when using e.g. LBT operations, we therefore propose to study BWP activation and switching enhancements within the unlicensed NR operations. Such enhancements could include considering new measurements and reporting to support system awareness of the current channel properties of configured BWPs.

In the email discussion [6], there were not so much support to study channel switching, but we suggest to study if multiple channels can be configured on RRC level and to use L1 signaling for efficient channel switching based on current radio conditions.

Further we consider an allocated carrier/channel may be relatively wide and in an unlicensed system it could be beneficial to view such wide channel as a multiple of 20MHz sub-channels with individual usage based on current radio conditions for each sub-channel.

Proposal 1: Study use of multiple BWPs configurations with fast activation and switching, aimed for improving the channel access.

## Directional LBT

In NR-based access to unlicensed spectrum, highly directional antennas bring two advantages. Transmission with beamforming allows high SINR to be achieved at the receiver, as supported in NR phase 1. Without beamforming, NR operation might not work in a high frequency band such as 60 GHz unlicensed spectrum due to severe propagation loss. In addition, through transmission with narrow beams, spatial reuse among several nodes could be enabled since the Tx node can reduce transmission in unnecessary directions.

Observation 1: Transmission with narrow beams would be beneficial for increasing the link gain and enabling beam-based spatial reuse.

As agreed in RAN1#92bis Techniques to cope with directional antennas/transmissions should be studied. Potential issues in applying high-gain beamforming on unlicensed spectrum is the increase in extra hidden nodes and handling of regulatory EIRP limitations. For example, in the case that the Tx node transmits a signal with high-gain beamforming but applies omni-directional LBT, the signal might reach a node which is located outside the coverage of the omni-directional LBT. If the Tx node transmits using beamforming, directional LBT might be useful for detecting other nodes. Another possible solution is RSSI measurement and reporting from UEs. In LTE LAA, RSSI measurement and reporting by UEs is introduced to detect hidden nodes seen from eNodeB. Also, in NR-based access to unlicensed spectrum, RSSI measurement and reporting would be useful for detecting hidden nodes. In addition, by using directional Rx antennas, UEs can measure RSSI directionally so that the gNodeB can acquire more detailed information on the communication environment around the gNodeB.

Observation: Directional LBT has been discussed in RAN1, but outcome has not been very clear. Potential RAN2 impact should be monitored based on further RAN1 input

Proposal 2: Study directional LBT and measurement mechanisms to alleviate hidden node problem especially in the higher frequency bands based on RAN1 input.

## Mobility enhancements

As per email discussion [6], most companies showed interest to further study mobility enhancements such as Conditional Handover.

Proposal 3: We think that e.g. mobility enhancements such as conditional handover or similar should be studied, as well as usage of uplink measurements.

# Conclusions

In this contribution, based on the above discussion we have the following observations and proposals:

Observation 1: Transmission with narrow beam would be beneficial for increasing link gain and enabling beam-based spatial reuse.

Proposal 1: Study use of multiple BWPs configurations with fast activation and switching, aimed for improving the channel access.

Proposal 2: Study directional LBT and measurement mechanism to alleviate hidden node problem especially on the higher frequency bands.

Proposal 3: We think that e.g. mobility enhancements such as conditional handover or similar should be studied, as well as usage of uplink measurements.

# References

- RP-172021, Study on NR-based Access to Unlicensed Spectrum, Dubrovnik, Croatia, March 6 - 9, 2017.

- Chairman’s Notes, RAN WG1 #92bis meeting, February 2018.

- TR 36.889, Study on Licensed-Assisted Access to Unlicensed Spectrum, v13.0.0, Jun 2015

- Chairman’s Notes, RAN WG1 #93 meeting, May 2018

- Chairman’s Notes, RAN WG2 #102 meeting, May 2018

- [102#69][NR] NR-U mobility (Qualcomm)