3GPP TSG-RAN WG2#AH1807	R2-1810138

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.2.6

Source:	Ericsson

Title:	DRB integrity protection and ciphering for NE-DC

Document for:	Discussion, Decision

# Introduction

During RAN2#101it was agreed that UP integrity protection will not be supported for EN-DC or for E-UTRA connected to 5GC:

Agreements

3	Data integrity protection will not be supported for E-UTRA connected to 5GC in rel-15.

Agreements for EN-DC

1:	Integrity protection of DRB is not supported for EN-DC.

Whereas for NR it was agreed:

Agreements

1	If integrity protected applies to a PDU session then it is applied to all DRBs of the PDU session.

2	If ciphering applies to a PDU session then it is applied to all DRBs of the PDU session.

Note: Signalling may still be per DRB and the above constraints ensured by network configuration. To be concluded in final ASN.1 design.

During RAN2#101bis, it was further agreed that UP IP will be configurable per DRB:

Agreements

1:	In RRC signalling the encryption [On Off] should be added in the PDCP-config (i.e. per RB) (as for DRB IP)

2	The DRBs associated to a PDU session should be configured with encryption in a consistent way. SRBs should be configured with encryption in a consistent way.

3	The SRB security configuration (encryption) and DRB security configuration (IP and encryption) does not change in the lifetime of the RB (if there is a change needed then release and add of DRB is available)

The agreements were made considering NR SA but did not exclude MR-DC.

This paper discusses the implication of supporting this configurability for NE-DC.

# Discussion

In the ‘Introduction of SA CR’ to TS 38.331 [1], a parameter to disable DRB ciphering has been introduced along with the existing parameter to configure integrity protection inside the PDCP-Config IE.

PDCP-Config ::=			SEQUENCE {

drb						SEQUENCE {

********** Omitted parts**********

integrityProtection		ENUMERATED { enabled }			OPTIONAL,	-- Cond ConnectedTo5GC

}														OPTIONAL,	-- Cond DRB

********** Omitted parts **********

...,

[[

cipheringDisabled		ENUMERATED {true} 				OPTIONAL 	-- Need S

]]

}

PDCP-Config field descriptions

cipheringDisabled If included, it indicates that cipherng is disabled for the DRB. The network does not configure this field for EN-DC.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The value of integrityProtection for a DRB can only be changed using reconfiguration with sync. FFS: text to indicate where to find the key.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB is being set up, not present for SRBs. Otherwise this field is optionally present, need M. |
| ConnectedTo5GC | The field is optionally present, need R, if EN-DC is not configured, and absent if EN-DC is configured. |

Since the condition for inclusion of the integrity protection only requires a DRB connected to 5GC, this will include NR SA, NE-DC, NN-DC as well as NGEN-DC. However, as it has been agreed that E-UTRA connected to 5GC will not support DRB integrity protection this will have implications for NE-DC and NGEN-DC.

E-UTRA connected to 5GC does not support DRB integrity protection

The requirement from SA2 is that the integrity protection configurations shall be configured per PDU session, if IP should be configured for DRBs in NE-DC, thus a PDU session with integrity protection enabled may only be mapped to bearers terminating in the MN.

Since NR SA may enable DRB integrity protection for DRBs in a PDU-Sessions, if the UE would then transition to NE-DC, i.e. add an E-UTRA cell as a secondary node, any DRB with IP enabled can continue with IP as long as it terminates in the MN.

If a UE in NR SA has DRB IP enabled and adds an E-UTRA cell as secondary (into NE-DC), the UE can continue with DRB IP for a PDU Session if all the DRBs of that PDU-Session continue to terminate in the MN

If a DRB terminating in the MN has IP enabled and is reconfigured to the SN, the entire PDU-Session would have to be released since the security requirements can no longer be fulfilled by the ng-eNB.

DRB IP is supported for NE-DC for PDU-sessions where all bearers terminate in the MN

A DRB with IP enabled cannot be reconfigured to an eNB or ng-eNB which doesn’t support DRB IP

# Conclusion

In section 2 we made the following observations:

Observation 1	E-UTRA connected to 5GC does not support DRB integrity protection

Observation 2	If a UE in NR SA has DRB IP enabled and adds an E-UTRA cell as secondary (into NE-DC), the UE can continue with DRB IP for a PDU Session if all the DRBs of that PDU-Session continue to terminate in the MN

Based on the discussion in section 2 we propose the following:

Proposal 1	DRB IP is supported for NE-DC for PDU-sessions where all bearers terminate in the MN

Proposal 2	A DRB with IP enabled cannot be reconfigured to an eNB or ng-eNB which doesn’t support DRB IP

# References

R2-1809239, Introduction of SA, Ericsson, RAN2#AH1807, Montreal, Canada, 2-6 July 2018