3GPP TSG-RAN2 Meeting #AH-1807	R2-1810166

# Montreal, Canada, 2nd Jul 2018 - 6th Jul 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to handle RLF in CA duplication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-06-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following agreements were reached on CA duplication at RAN2#102: When RLC max retransmissions are met for a logical channel restricted to one or multiple SCell(s), the information with the report identifies the problematic RLC entity without ambiguity. The report on RLC max retransmissions does not include measurement results. In the email discussion a majority of companies seemed to want a new RRC message for this indication. Details TBD in common session. If the failure is restricted to the SCG, the information need to be known by the SN, and for other cases the MN. UP session assumes SRB1 and SRB3 would be used accordingly. UP session confirm current behaviour that when RLC max retransmissions are met for a logical channel restricted to PCell or PSCell, RLF or SCG failure is triggered. LCH ID + MCG/SCG Indication is reported As the behaviour on reaching max retransmissions at RLC has been defined in 38.331, we capture the above agreements in a similar manner. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The section on the detection of radio link failure is updated to take into account that the RLC entity may not be associated with an SpCell. In this case, the behaviour follows the agreements mentioned above. A new procedure, i.e. secondary cell radio link failure is introduced along with the a new message to indicate this failure to the network. The transmission of the new message takes place on SRB1 or SRB3 based on whether the failure took place on MN or SN (in the case of EN-DC). Editor’s notes on RLF in the case of CA duplication are removed. This draft CR is only applicable to the SA release as it is related to data duplication and is based on R2-1809239. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The behaviour when maximum number of retransmissions are reached on an RLC entity restricted to a subset of SCells would be undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.10.3, 5.7.3.1, 5.7.4, 6.2.1, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change 1

### 5.3.10	Radio link failure related actions

#### 5.3.10.1	Detection of physical layer problems in RRC_CONNECTED

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications for the SpCell from lower layers while T311 is not running:

2>	start timer T310 for the corresponding SpCell.

#### 5.3.10.2	Recovery of physical layer problems

Upon receiving N311 consecutive "in-sync" indications for the SpCell from lower layers while T310 is running, the UE shall:

1>	stop timer T310 for the corresponding SpCell.

NOTE 1:	In this case, the UE maintains the RRC connection without explicit signalling, i.e. the UE maintains the entire radio resource configuration.

NOTE 2:	Periods in time where neither "in-sync" nor "out-of-sync" is reported by layer 1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications.

#### 5.3.10.3	Detection of radio link failure

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while T311 is not running; or

1>	upon indication from MCG RLC associated with the PCell that the maximum number of retransmissions has been reached:

Editor’s Note: FFS whether maximum ARQ retransmission is only criteria for RLC failure.

2>	consider radio link failure to be detected for the MCG i.e. RLF;

Editor’s Note: FFS Whether indications related to beam failure recovery may affect the declaration of RLF.

Editor’s Note: FFS: RLF related measurement reports e.g. VarRLF-Report is supported in NR.

2>	if AS security has not been activated:

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC associated with the PSCell that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

The UE shall:

1>	upon indication from RLC not associated with an SpCell that the maximum number of retransmissions has been reached:

2>	consider secondary cell radio link failure to be detected in the CG i.e. SCell-RLF;

2>	initiate the SCell failure information procedure as specified in 5.7.4 to report the SCell-RLF.

End of change 1

Start of change 2

### 5.7.3	SCG failure information

#### 5.7.3.1	General

Figure 5.7.3.1-1: SCG failure information

The purpose of this procedure is to inform EUTRAN or NR MN about an SCG failure the UE has experienced i.e. SCG radio link failure, e failure of SCG reconfiguration with sync, SCG configuration failure for RRC message on SRB3, SCG integrity check failure and exceeding the maximum uplink transmission timing difference.

Editor’s Note: SCG failure considers the case of exceeding the maximum uplink transmission timing difference if RAN1 decides that EN-DC supports the synchronised operation case. FFS how to capture

.

End of change 2

Start of change 3

### 5.7.4	SCell failure information

#### 5.7.4.1	General

Figure 5.7.3.1-1: SCell failure information

The purpose of this procedure is to inform NG-RAN about an SCell-RLF the UE has experienced i.e. secondary cell radio link failure.

#### 5.7.4.2	Initiation

A UE initiates the procedure to report SCell failure when the following condition is met:

1>	upon detecting SCell-RLF in a CG, in accordance with subclause 5.3.10.3.

Upon initiating the procedure, the UE shall:

1>	send the SCellFailureInformation message.

#### 5.7.4.3	Actions related to transmission of SCellFailureInformation message

The UE shall set the contents of the SCellFailureInformation message as follows:

1> set the CellGroupId to the identity of the CG on which SCell-RLF is triggered;

1>	set the LogicalChannelIdentity to the identity of the logical channel corresponding to the RLC bearer on which SCell-RLF is triggered.

The UE shall:

1>	if SCell-RLF is triggered on the MCG:

2>	submit the SCellFailureInformation message via SRB1 to lower layers for transmission;

1>	else:

2> if the UE is configured with EN-DC:

3>	if SRB3 is configured;

4>	submit the SCellFailureInformation message via SRB3 to lower layers for transmission, upon which the procedure ends;

3>	else:

4>	submit the SCellFailureInformation message via the EUTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

End of change 3

Start of change 4

### 6.2.1	General message structure

<Skip unrelated Part>

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the network on the uplink DCCH logical channel.

-- ASN1START

-- TAG-UL-DCCH-MESSAGE-START

UL-DCCH-Message ::= SEQUENCE {

message											UL-DCCH-MessageType

}

UL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

measurementReport							MeasurementReport,

rrcReconfigurationComplete					RRCReconfigurationComplete,

rrcSetupComplete							RRCSetupComplete,

rrcReestablishmentComplete					RRCReestablishmentComplete,

rrcResumeComplete							RRCResumeComplete,

securityModeComplete						SecurityModeComplete,

securityModeFailure							SecurityModeFailure,

ulInformationTransfer						ULInformationTransfer,

locationMeasurementIndication				LocationMeasurementIndication,

sCellFailureInformation						SCellFailureInformation,

spare6 NULL,

spare5 NULL, spare4 NULL, spare3 NULL,

spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-UL-DCCH-MESSAGE-STOP

-- ASN1STOP

### 6.2.2	Message definitions

<Skip unrelated Part>

#### –	SCellFailureInformation

The SCellFailureInformation message is used to provide information regarding SCell radio link failure detected by the UE.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCellFailureInformation message

-- ASN1START

-- TAG-SCELLFAILUREINFORMATION-START

SCellFailureInformation ::=	SEQUENCE {

criticalExtensions					CHOICE {

scellFailureInformation			SCellFailureInformation-IEs,

criticalExtensionsFuture		SEQUENCE {}

}

}

SCellFailureInformation-IEs ::=		SEQUENCE {

logicalChannelIdentity						LogicalChannelIdentity,

cellGroupId									CellGroupId,

...

}

-- TAG-SCELLFAILUREINFORMATION-STOP

-- ASN1STOP

SCellFailureInformation field descriptions

logicalChannelIdentity Logical channel ID, as specified in section 5.7.4.3, corresponding to one or more SCells that have experienced a radio link failure.

cellGroupId Identity of the cell group, as specified in section 5.7.4.3, to which one or more SCells that have experienced a radio link failure belong.

End of change 4