3GPP TSG-RAN WG2 AH 1807	Tdoc R2-1810180

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.3.1.11

Source:	Ericsson

Title:	LCP restrictions for duplication and non-duplication

Document for:	Discussion, Decision

# Introduction

At RAN2#102, companies have presented views on whether to also apply the LCP restriction allowedServingCells for non-duplication cases [1].  We analyse this issue in this paper and reveal also a required correction for the duplication case.

# Discussion

In the current LCP procedure, there is no limit that the mapping restriction allowedServingCells is only applicable to the duplication case. See the existing LCP procedure as below:

5.4.3.1.1 General

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to TRUE in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant.

It is seen from the current LCP procedure, the mapping restriction allowedServingCells can be applied for an LCH regardless of whether it is configured with duplication or not.

In the existing MAC LCP procedure, there is no limit on whether the mapping restriction allowedServingCells is only applicable to duplication case.

However, in the section 5.10 regarding PDCP duplication in the existing MAC specification, it is specified when to apply the mapping restriction allowedServingCells upon the reception of the MAC CE for controlling duplication. The procedure is illustrated below:

The MAC entity shall for each DRB configured with duplication:

1>	if a Duplication Activation/Deactivation MAC CE is received activating the PDCP duplication of the DRB:

2>	indicate the activation of PDCP duplication of the DRB to upper layers;

2>	apply the allowedServingCells to the logical channels of the DRB.

1>	if a Duplication Activation/Deactivation MAC CE is received deactivating the PDCP duplication of the DRB:

2>	indicate the deactivation of PDCP duplication of the DRB to upper layers;

2>	not apply the allowedServingCells to the logical channels of the DRB.

The section on duplication handling in MAC specifies when to apply the allowedServingCells restriction upon reception of a MAC CE for duplication.

For duplication operation, from the section above, it appears that the mapping restrictions allowedServingCells are only applied or not upon the reception of the MAC CE controlling duplication indicating activation or deactivation. This is not the intended behaviour, since both RRC signalling and the MAC CEs can control the applicability of the duplication function.

1:	In CA, after the duplication is deactivated, the logical channel to carrier mapping restriction is not applied. UE sends new data via one specified logical channel.

Upon duplication control via RRC (activation/deactivation), allowedServingCells restriction should be also affected (i.e., applied/not applied), the current specification is incomplete, since only the case upon reception of MAC CE is covered.

Generally, also to the non-duplication case, we think the mapping restriction allowedServingCells can be applied. In NR, each cell may be configured with a different numerology, PUSCH transmission duration compared to other cells. In a CA scenario, a UE may be configured up to 16 CCs and where each CC may be configured with different numerology, PUSCH transmission duration etc. In this case some of the LCHs not requiring strict latency requirements, such as eMBB like services, are suitable to be mapped with CCs associated with low numerology and long PUSCH transmission duration, while other LCHs carrying latency critical services such as URLLC like services are beneficial to be mapped with CCs associated with higher numerology and short PUSCH transmission duration.  For such cases, there is no need to define new LCP restriction. The mapping restriction allowedServingCells can be directly applied in this case. An even simpler use-case is traffic separation per carrier, i.e. serve URLLC traffic (independent of whether duplication is used) only a certain carrier. Therefore, we think the mapping restriction allowedServingCells must be applicable for both duplication and non-duplication cases.

The LCH restriction allowedServingCells is beneficial to apply to non-duplication URLLC use-cases, e.g. for segregating URLLC traffic to a certain carrier.

While it seems clear from RRC specification as well as from LCP procedure in MAC that allowedServingCells is applicable for all cases, the MAC section on reception of MAC CEs for duplication seems to make the allowedServingCells configuration to apply only to duplication and that only upon MAC CE reception. We would like to clarify this and propose:

RAN2 confirms that LCH allowedServingCells restriction applies also to non-duplication case.

Therefore, we propose in [2] a CR clarification focusing on both the aspects of allowedServingCells being applicable to non-duplication case, as well as that in the duplication case, the restrictions are used/lifted not only upon MAC CE reception, but generally.

# Conclusion

We have observed:

Observation 1	In the existing MAC LCP procedure, there is no limit on whether the mapping restriction allowedServingCells is only applicable to duplication case.

Observation 2	The section on duplication handling in MAC specifies when to apply the allowedServingCells restriction upon reception of a MAC CE for duplication.

Observation 3	Upon duplication control via RRC (activation/deactivation), allowedServingCells restriction should be also affected (i.e., applied/not applied), the current specification is incomplete, since only the case upon reception of MAC CE is covered.

Observation 4	The LCH restriction allowedServingCells is beneficial to apply to non-duplication URLLC use-cases, e.g. for segregating URLLC traffic to a certain carrier.

We propose:

Proposal 1	RAN2 confirms that LCH allowedServingCells restriction applies also to non-duplication case.

# References

- R2-1808514 - Configuration of LCP restriction, LG.

- R2-1810181 - Correction for LCP restriction for duplication and non-duplication, Ericsson