3GPP TSG-RAN WG2 Meeting #AH1807	R2-1810193

Montreal, Canada, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [E137, E167, E170] CR to 38.331 for inter-RAT mobility between NR and E-UTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inter-RAT handover between NR and LTE is currently not captured in TS 38.331. The parameter nas-SecurityParamToNGRAN is no longer valid according to SA2 specification TS 23.501. The new parameter will be ‘S1 Mode to N1 mode NAS transparent container’, which will have the size 8 or 10 octets compared to the nas-SecurityParamtToNGRAN which were 6 octets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5.3.5.7 Security Key Update [E167] - parameter nas-SecurityParamToNGRAN is changed to s1ModeToN1ModeNAS-TransparentContainer 5.4 Inter-RAT mobility [E137] - Full procedures are provided for handover from NR to LTE and LTE to NR 6.2. RRC messages - MobilityFromNRCommand is introduced [E137] - RadioBearerConfig is updated with parameter s1ModeToN1ModeNAS-TransparentContainer [E170] |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-RAT handover will not be supported in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.7 Security Key Update 5.4 Inter-RAT mobility 6.2. RRC messages - MobilityFromNRCommand - RadioBearerConfig |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### 5.3.5.7	Security key update

>>>>>>>>>>>>>>> Omitted unchanged parts <<<<<<<<<<<<<<<<<

2>	if the s1ModeToN1ModeNAS-TransparentContainer is included in the received keyRefresh:

3>	forward the s1ModeToN1ModeNAS-TransparentContainer to the upper layers;

3>	derive the KgNB key as specified in TS 33.501 [11];

3> derive the keys associated with KgNB as follows:

END OF FIRST CHANGE

START OF SECOND CHANGE

## 5.4	Inter-RAT mobility

#### 5.4.1	Introduction

NR support network controlled inter-RAT handover to/from NR from/to E-UTRA connected to EPC or 5GC.

#### 5.4.2	Handover to NR

##### 5.4.2.1	General

Figure 5.4.2.1-1: Handover to NR, successful

The purpose of this procedure is to, under the control of the network, transfer a connection between the UE and another Radio Access Network (e.g. E-UTRAN) to NR.

The handover to NR procedure applies when SRBs, possibly in combination with DRBs, are established in another RAT. Handover from E-UTRA to NR applies only after integrity has been activated in E-UTRA.

##### 5.4.2.2	Initiation

The RAN using another RAT initiates the handover to NR procedure, in accordance with the specifications applicable for the other RAT, by sending the RRCReconfiguration message via the radio access technology from which the inter-RAT handover is performed.

The network applies the procedure as follows:

-	to activate ciphering, possibly using NULL algorithm, if not yet activated in the other RAT;

-	to re-establish SRBs and one or more DRBs;

##### 5.4.2.3	Reception of the RRCReconfiguration by the UE

The UE shall:

1>	if the received RRCReconfiguration includes the securityConfig:

2>	perform key update procedure as specified in subclause 5.3.5.7;

- perform the radio configuration procedure as specified in subclause 5.3.5.

##### 5.4.2.4	Reconfiguration failure

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message:

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT;

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.

#### 5.4.3	Mobility from NR

##### 5.4.3.1	General

Figure 5.4.3.1-1: Mobility from NR, successful

Figure 5.4.3.1-2: Mobility from NR, failure

The purpose of this procedure is to move a UE in RRC_CONNECTED to a cell using E-UTRA Radio Access Technology (RAT). The mobility from NR procedure covers the following type of mobility:

-	handover, i.e. the MobilityFromNRCommand message includes radio resources that have been allocated for the UE in the target cell;

##### 5.4.3.2	Initiation

The network initiates the mobility from NR procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport message, by sending a MobilityFromNRCommand message. The network applies the procedure as follows:

-	the procedure is initiated only when AS-security has been activated, and SRB2 with at least one DRB are setup and not suspended;

##### 5.4.3.3	Reception of the MobilityFromNRCommand by the UE

The UE shall be able to receive a MobilityFromNRCommand message, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	if the targetRAT-Type is set to eutra-EPC:

2>	consider inter-RAT mobility as initiated towards E-UTRA connected to EPC;

1>	else if the targetRAT-Type is set to eutra-5GC:

2>	consider inter-RAT mobility as initiated towards E-UTRA connected to 5GC;

1>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

##### 5.4.3.4	Successful completion of the mobility from NR

Upon successfully completing the handover the UE shall:

1>	stop all timers that are running;

1>	release all cell group configurations, including release of the RLC entity and the MAC configuration for all established RBs;

1>	if the targetRAT-Type is set to eutra-EPC:

2>	release the associated PDCP and SDAP entity for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

##### 5.4.3.5	Mobility from NR failure

The UE shall:

1>	if the UE does not succeed in establishing the connection to the target radio access technology; or

1>	if the UE is unable to comply with (part of) the configuration included in the MobilityFromNRCommand message; or

1>	if there is a protocol error in the inter RAT information included in the MobilityFromNRCommand message, causing the UE to fail the procedure according to the specifications applicable for the target RAT:

2>	revert back to the configuration used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in subclause 5.3.7;

END OF SECOND CHANGE

START OF THIRD CHANGE

## 6.2	RRC messages

### 6.2.2	Message definitions

#### –	MobilityFromNRCommand

The MobilityFromNRCommand message is used to command handover from NR to E-UTRA (connected to EPC or 5GC).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

MobilityFromNRCommand message

-- ASN1START

MobilityFromNRCommand ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

mobilityFromNRCommand				MobilityFromNRCommand-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

MobilityFromNRCommand-IEs ::=	SEQUENCE {

targetRAT-Type						ENUMERATED {

eutra-EPC, eutra-5GC, spare2, spare1, ...},

targetRAT-MessageContainer			OCTET STRING

lateNonCriticalExtension				OCTET STRING								OPTIONAL,

nonCriticalExtension					SEQUENCE {}									OPTIONAL

}

-- ASN1STOP

MobilityFromNRCommand field descriptions

targetRAT-Type Indicates the target RAT type and target CN type.

targetRAT-MessageContainer The field contains a message specified in another standard, as indicated by the targetRAT-Type, and carries information about the target cell identifier(s) and radio parameters relevant for the target radio access technology. NOTE 1. A complete message is included, as specified in the other standard.

NOTE 1:	The correspondence between the value of the targetRAT-Type, the standard to apply and the message contained within the targetRAT-MessageContainer is shown in the table below:

| targetRAT-Type | Standard to apply | targetRAT-MessageContainer |
| --- | --- | --- |
| eutra-EPC/eutra-5GC | 3GPP TS 36.331 (subclause 5.4.2) | RRCCONNECTIONreconfiguration |

END OF THIRD CHANGE

START OF FOURTH CHANGE

### 6.3.2	Radio resource control information elements

#### –	RadioBearerConfig

>>>>>>>>>>>>>Omitted unchanged parts <<<<<<<<<<<<<<

SecurityConfig ::=						SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig									OPTIONAL,	-- Cond RBTermChange

keyToUse								ENUMERATED{master, secondary}								OPTIONAL,	-- Cond RBTermChange

...,

[[

keyRefresh							KeyRefresh													OPTIONAL		-- Need M

]]}

KeyRefresh ::=					SEQUENCE {

keySetChangeIndicator			BOOLEAN														OPTIONAL,

nextHopChainingCount			NextHopChainingCount										OPTIONAL,

s1ModeToN1ModeNAS-TransparentContainer	CHOICE {

smallNAS-Container				OCTET STRING (SIZE(8)),

largeNAS-Container				OCTET STRING (SIZE(10))

OCTET STRING 									}																								OPTIONAL	-- Cond InterSystemHO

}

>>>>>>>>>>>>>Omitted unchanged parts <<<<<<<<<<<<<<

SecurityConfig field descriptions

s1ModeToN1ModeNAS-TransparentContainer This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS- security after inter-system handover to NG-RAN. The content is defined in TS 24.501 [23, 9.10.2.7]. The largeNAS-Container is used if the EPS UE security capability are provided by the AMF; otherwise the smallNAS-Container is used.

END OF FOURTH CHANGE