3GPP TSG-RAN AH 1807                                                                                              R2-1810479

Montreal, Canada, 2-6 July, 2018                                                         Resubmit of   R2-1807460

Agenda Item:	    10.4.1.3.7

Source: 	    MediaTek Inc.

Title:		    NAS trigger RRC Connection Release

Document for:	    Discussion and decision

# 1 Introduction

In the LTE RRC SPEC, NAS could trigger RRC connection release and may also bar the current LTE serving cell. In this paper, we discuss whether NR RRC should have similar behavior as in LTE.

# 2 Discussion

In current NAS SPEC 24.501 5.4.1.3.7 - Abnormal cases [1], there is a procedure text about local release RRC connection and bar cell while authentication check fails.

g)	Network failing the authentication check:

If the UE deems that the network has failed the authentication check, then it shall request RRC to locally release the RRC connection and treat the active cell as barred (see 3GPP TS 38.304 [28]). The UE shall start any retransmission timers (e.g. T3510, T3517 or T3521), if they were running and stopped when the UE received the first AUTHENTICATION REQUEST message containing an invalid MAC or SQN or if the new ngKSI was already in use.

Editor's note:	It is FFS if the RRC will provide the barring service.

Although there seems no formal LS from CT1 about NAS trigger RRC connection release, the behaviour is specified in current TS. From our point of view, it is reasonable to follow the same principle as in LTE. The current TS 38.331 in this section is still empty. Thus we propose to support the NAS trigger RRC connection release as in LTE. We also provide a TP for this (very similar to LTE).

Proposal 1: Support NAS trigger RRC connection release in NR.

Proposal 2: RAN2 adopts the TP in Annex.

# 3 Conclusions

Base on the discussion in section 2, we propose the following:

Proposal 1: Support NAS trigger RRC connection release in NR.

Proposal 2: RAN2 adopts the TP in Annex.

# 4 References

[1] TS 24.501 v15.0.0

# Annex: TP for 38.331

### 5.3.9	RRC connection release requested by upper layers

#### 5.3.9.1	General

The purpose of this procedure is to release the RRC connection. Access to the current PCell may be barred as a result of this procedure.

NOTE:	Upper layers invoke the procedure, e.g. upon determining that the network has failed an authentication check, see TS 24.501 [23].

#### 5.3.9.2	Initiation

The UE initiates the procedure when upper layers request the release of the RRC connection. The UE shall not initiate the procedure for power saving purposes.

The UE shall:

1>	if the upper layers indicate barring of the PCell:

2>	treat the PCell used prior to entering RRC_IDLE as barred according to TS 38.304 [21];

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';