3GPP TSG-RAN WG2 NR AH1807 Meeting	R2-1810501

Montreal, Canada, 2nd - 6th July 2018

Agenda Item:	10.3.1.4.2

Source:	MediaTek Inc.

Title:	Remaining issue on CFRA

Document for:     Discussion and Decision

# 1 Introduction

MAC spec has captured the following text to specify the behavior of MAC triggered BWP switching upon RACH initiation [1]:

Upon initiation of the Random Access procedure on a Serving Cell, the MAC entity shall for this Serving Cell: 1> if PRACH occasions are not configured for the active UL BWP: 2> switch the active UL BWP to BWP indicated by initialUplinkBWP; 2> if the Serving Cell is a SpCell: 3> switch the active DL BWP to BWP indicated by initialDownlinkBWP. 1> else: 2> if the Serving Cell is a SpCell: 3> if the active DL BWP does not have the same bwp-Id as the active UL BWP: 4> switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP. 1> perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

In this paper, we list the remaining issues to support CFRA on PCell and on SCell, and propose corresponding alternatives.

# 2 Discussion

## 2.1 CFRA on SCell

Same as in LTE, in NR CBRA is supported only on PCell, not on SCell, which means RACH procedure on SCell does not support CBRA fallback. UE behavior when a CFRA is on SCell and all dedicated PRACH resources are inapplicable for CFRA, i.e. all dedicated PRACH resources are associated with SSB/CSI-RS below RSRP threshold, is still unspecified.

which means there is no CBRA fallback on SCell if all dedicated PRACH resources are associated with SSB/CSI-RS below RSRP threshold. Currently, how UE handle the case of no applicable dedicated PRACH resource on SCell is still not specified.

Observation 1: CBRA fallback on SCell is not supported, and the UE behavior is still unspecified when CFRA is on SCell and all dedicated PRACH resources are associated with SSB/CSI-RS below RSRP threshold.

There are two simple alternatives to handle the case when there is no dedicated PRACH for CFRA on SCell:

- Option A: UE select any DL beams associated with CFRA PRACH resources.

- Option B: UE considers the CFRA on SCell unsuccessfully completed.

We prefer Option B because if no dedicated PRACH is associated with a good beam for CFRA, probably the SCell is not usable and therefore UE should not waste power to retry. Network anyway can detect and handle this issue if there is no preamble received some time after network initiates CFRA.

Proposal 1. If UE perform CFRA on SCell and all DL beams associated with dedicated PRACH are below RSRP threshold for CFRA, UE considers the CFRA on SCell unsuccessfully completed.

## 2.2 CFRA on PCell

The remaining issue for CFRA on PCell is below:

- Upon CFRA initiation, whether UE should switch DL BWP if UE’s active DL BWP is not paired with active UL BWP.

Two alternatives are below:

- Option 1: Switch to the DL BWP paired with UL BWP upon CFRA initiation

- Option 2: Perform CFRA on current active DL BWP.

The comparison between option 1 and option 2 are as follows:

- Option 1 always switch DL BWP regardless of whether there is good beam for CFRA, and therefore will in some extent interrupt ongoing data transmission.

  - For network initiated CFRA, BWP switching upon CFRA initiation is expected to the network, but sill has some impact on ongoing data transmission, i.e. BWP switching latency. In addition, UE needs to perform measurement on the new DL BWP.

  - For MAC-initiated CFRA (i.e. CFRA BFR),  BWP switching upon CFRA initiation is not expected to the network and therefore will interrupt ongoing data transmission

In this case since network does not know when CFRA BFR is triggered, upon CFRA initiation the DL BWP switching will interrupt ongoing data transmission. The interruption continues until network detects the preamble on dedicated PRACH for CFRA BFR.

The motivation of CFRA BFR is to have a quick recovery on beam failure and continues data transmission smoothly. However, option 1 forces BWP switching and defeats the purpose.

Observation 2: Switching BWP upon CFRA initiation will interrupt ongoing data transmission especially when CFRA is for BFR.

- For Option 2, we have the following observations:

  - If CBRA fallback does not occur, there is no interruption on data transmission even for CFRA BFR.

  - To support switch between CFRA and CBRA, UE is required to perform dynamic switching between DL BWPs

    - In option 2, the DL BWPs for CFRA and for CBRA are different, and network does not know when UE will switch DL BWP. To support switch between CFRA and CBRA, UE is requested to monitor DL beam quality on the two DL BWPs, and accordingly decides whether to switch DL BWP for CFRA/CBRA.

Observation 3: To support switching between CFRA and CBRA, performing CFRA on current active BWP requires UE to dynamically switch between two DL BWPs.

Based on discussion above, we prefer option 2. The reason is that CBRA fallback will not frequently happen, and therefore dynamic switch between two DL BWPs during RACH is really a rare case. To avoid data transmission interruption, UE should not switch BWP when some dedicated beams are indeed applicable for CFRA BFR.

Proposal 2: UE perform CFRA on current active DL BWP regardless of whether active DL BWP is paired with UL BWP for CBRA.

# 3 Conclusion

Based on the observation:

Observation 1: CBRA fallback on SCell is not supported, and the UE behavior is still unspecified when CFRA is on SCell and all dedicated PRACH resources are associated with SSB/CSI-RS below RSRP threshold.

Observation 2: Switching BWP upon CFRA initiation will interrupt ongoing data transmission especially when CFRA is for BFR.

Observation 3: To support switching between CFRA and CBRA, performing CFRA on current active BWP requires UE to dynamically switch between two DL BWPs.

We propose:

Proposal 1. If UE perform CFRA on SCell and all DL beams associated with dedicated PRACH are below RSRP threshold for CFRA, UE considers the CFRA on SCell unsuccessfully completed.

Proposal 2: UE perform CFRA on current active DL BWP regardless of whether active DL BWP is paired with UL BWP for CBRA.

The corresponding CR is provided in [2].

# 4 Reference

- 3GPP TS 38.321 V15.2.0 (2018-06)

- R2-1810502, Correction to support CFRA, CR 0270, MediaTek Inc., NR AH1807