3GPP TSG-RAN2 NR AH1807 Meeting 	R2-1810571

# Montreal, Canada, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCCH Configuration in NR (38.331 RIL M032) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 has made agreements on paging configurations, and it is capured in TS 38.304 that “Parameters nB, PF_offset, and the length of default DRX Cycle are signaled in SystemInformationBlock1.” Therefore, corresponding configurations should be introduced in RRC specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on endorsed CR R2-1809239, we introduce the following changes: <1> Fill in the details of PCCH-Config in DownlinkConfigCommonSIB. Field defaultPagingCycle describes the default paging cycle, used to derive 'T' in TS 38.304. Field nBAndPagingFrameOffset is used to derive the ‘nB’ and ‘PF_offset’ in TS 38.304. For nB larger than or equal to T, PF_offset is not configured (NULL); for nB less than T, PF_offset ranges from 0 to T/nB - 1. <2> Add ‘SIB’ after ‘DownlinkConfigCommon’ in the descrption text. Impact analysis Impacted functionality: PCCH configuration in NR Inter-operability: If the network is implemented according to this CR while the UE is not, UE does not know how to monitor paging occasions. If the UE is implemented according to this CR while the network is not, UE does not know how to monitor paging occasions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE does not know how to monitor paging occasions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# [Start of Change]

6.3.2	Radio resource control information elements

# [Skip unchanged parts]

–	DownlinkConfigCommonSIB

The IE DownlinConfigCommonSIB provides common downlink parameters of a cell.

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINK-CONFIG-COMMON-SIB-START

DownlinkConfigCommonSIB ::=		SEQUENCE {

frequencyInfoDL					FrequencyInfoDLSIB,

initialDownlinkBWP				BWP-DownlinkCommon,

bcch-Config 						BCCH-Config,

pcch-Config 						PCCH-Config,

...

}

BCCH-Config ::=					SEQUENCE {

modificationPeriodCoeff			ENUMERATED {n2, n4, n8, n16}

}

PCCH-Config ::=					SEQUENCE {

defaultPagingCycle					ENUMERATED {rf32, rf64, rf128, rf256},

nBAndPagingFrameOffset				CHOICE {

fourT									NULL,

twoT										NULL,

oneT										NULL,

halfT									INTEGER (0..1),

quarterT									INTEGER (0..3),

oneEighthT								INTEGER (0..7),

oneSixteenthT							INTEGER (0..15)

}

}

-- TAG-DOWNLINK-CONFIG-COMMON-SIB-STOP

-- ASN1STOP

DownlinkConfigCommonSIB field descriptions

frequencyInfoDLSIB Basic parameters of a downlink carrier and transmission thereon

initialUplinkBWP The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG).

bcch-Config The modification period related configuration.

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 38.304 [20]. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on.

nBAndPagingFrameOffset nB and PF_offset in TS 38.304 [22]. Parameter: nB is used as one of parameters to derive the Paging Frame and Paging Occasion according to TS 38.304 [22]. Value in multiples of 'T' as defined in TS 36.304 [22]. A value of fourT corresponds to 4 * T, a value of twoT corresponds to 2 * T and so on. Parameter PF_offset is used as one of parameters to derive the Paging Frame according to TS 38.304 [22]. The configured PF_offset applies to all paging frames.

# [End of Change]