3GPP TSG RAN WG2 NR ASN.1 Ad-hoc electronic	R2-2204307

e-Meeting, April 20th – 22nd, 2022

Title:	[Draft] Reply LS on updated Rel-17 RAN1 UE features list for NR

Response to:	R2-2203746/R1-2202927

Release:	Rel-17

Work Items:	NR_FeMIMO, NR_ext_to_71GHz, NR_IIOT_URLLC_enh, NR_NTN_solutions, NR_pos_enh, NR_redcap, NR_UE_pow_sav_enh, NR_cov_enh, NR_IAB_enh, NR_SL_enh, NR_MBS, NR_DSS, LTE_NR_DC_enh2, NR_DL1024QAM_FR1, NR_RF_FR1_enh, NR_SmallData_INACTIVE

Source:	Intel Corporation [RAN2]

To:	RAN WG1

CC:

Contact Person:

#### Name:	Seau Sian Lim

E-mail Address:	seau.s.lim@intel.com

Attachment:

1. Overall Description:

RAN2 would like to thank RAN1 for the latest version of the Rel-17 RAN1 UE feature list for NR. In addition, RAN2 would like to get some feedback on the following R1 features:

A) R1 23-8-3

RAN2 understanding is that this feature is an extension of srs-TxSwitch/srs-TxSwitch-v1610 to support SRS antenna switching xTyR with y>4.  However, RAN2 is unclear on how this new capability is populated for a band in a band combination where at least one band in the band combination supports xTyR with y>4 and how it works with the existing srs-TxSwitch/srs-TxSwitch-v1610. There are 2 interpretations:

(a) the new capabilities is populated for a band in the band combination only if the band supports y>4

(b) the new capabilities is populated for a band in the band combination regardless of whether the band supports y>4

If the interpretation is (a), the following note will not occur since the candidate value with xTyR combinations with y>4 are {t2r6, t1r6, t4r8, t2r8, t1r8} where x and y are never equal. For b), the Note can be applicable to, but only in band of the band combination where xTyR supports y ≤ 4, while not applicable in band of the band combination where xTyR supports y>4.

Note: Component 2 and component 3 is not reported if component 1 is reported as xTyR with x=y.

For both interpretations (a) and (b), should the component 1 be set consistently with the existing supportedSRS-TxPortSwitch/ supportedSRS-TxPortSwitch-v1610 in srs-TxSwitch/srs-TxSwitch-v1610 (i.e. the R1 23-8-3 bitmap in component 1 be aligned with the existing supportedSRS-TxPortSwitch/supportedSRS-TxPortSwitch-v1610 for the bit entries where y<=4). Also how are component 2 and 3 being set with respect to the Rel-16 capabilities if bit entries in component 1 contains the xTyR combination(s) in the existing Rel-15/16 capabilities.

RAN2 would also like to inform that R1 23-8-3 will be implemented as per band per BC (in the BandParameters of a band combination) rather than per FS as components 2 and 3 require setting them to the band entry of a reported band combination.

B). R1 27-16 and 27-19

R1 27-16 and 27-19 have a component description of ‘Same as RRC OLPC-SRS-Pos-r16’ and ‘Same as RRC SpatialRelationsSRS-Pos-r16’ respectively. It is unclear to RAN2 whether the pre-requisite in R1 27-16/27-19 should be srs-PosResources-r16 as in RRC OLPC-SRS-Pos-r16/SpatialRelationsSRS-Pos-r16 or should be “srs-PosResourcesRRC-Inactive-r17” (i.e. R1 27-15)). RAN2 would like RAN1 to clarify the pre-requisite used in the R1 27-16 and 27-19.

C) R1 24-2 and 24-3

Both of the features have N/A in the column of “Need for the gNB to know if the feature is supported” while indicate in the column of “Mandatory/Optional” as “optional with capability signalling”. From RAN2 perspective, if there is no need for gNB to know whether a feature is supported or not, no capability signalling should be defined. RAN2 also noticed that there are other features in NTN that have such ambiguities (e.g. R1 26-1/26-8 for NTN WI).  RAN2 would like to know whether such capabilities are really “optional with capability signalling”

2. Actions:

To RAN WG1

ACTION: RAN2 kindly asks RAN1 to provide feedback on A) – C).

3. Date of Next RAN WG2 Meetings:

TSG-RAN WG2 Meeting #118-e		May 9 to May 20, 2022		E-meeting