3GPP TS 36.133 V16.4.0 (2019-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	87

1	Scope	88

2	References	88

3	Definitions, symbols and abbreviations	90

3.1	Definitions	90

3.2	Symbols	92

3.3	Abbreviations	93

3.4	Test tolerances	95

3.5	Additional notation	96

3.5.1	Groups of bands	96

3.6	General	98

3.6.1	Applicability of requirements in this specification version	98

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	104

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	106

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	107

3.6.2	Applicability of requirements for EN-DC operation	108

3.6.3	Applicability of requirements for NE-DC operation	109

3.6.4	Applicability of requirements for NGEN-DC operation	110

4	E-UTRAN RRC_IDLE state mobility	110

4.1	Cell Selection	110

4.2	Cell Re-selection	110

4.2.1	Introduction	110

4.2.2	Requirements	110

4.2.2.1	Measurement and evaluation of serving cell	111

4.2.2.2	Void	112

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	112

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	114

4.2.2.5	Measurements of inter-RAT cells	116

4.2.2.5.1	Measurements of UTRAN FDD cells	116

4.2.2.5.2	Measurements of UTRAN TDD cells	118

4.2.2.5.3	Measurements of GSM cells	119

4.2.2.5.4	Measurements of HRPD cells	120

4.2.2.5.5	Measurements of cdma2000 1X	121

4.2.2.5.6	Measurements of NR cells	122

4.2.2.6	Evaluation of cell re-selection criteria	123

4.2.2.7	Maximum interruption in paging reception	124

4.2.2.8	void	124

4.2.2.9	UE measurement capability	124

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	125

4.2.2.10	Reselection to CSG cells	125

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	126

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	126

4.2.2.11	Void	127

4.2.2.12	Void	127

4.2.2.13	Void	127

4.3	Minimization of Drive Tests (MDT)	127

4.3.1	Introduction	128

4.3.2	Measurements	128

4.3.2.1	Requirements	128

4.3.3	Relative Time Stamp Accuracy	128

4.3.3.1	Requirements	128

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	128

4.3.4.1	 Requirements	128

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	129

4.3.5.1	Requirements for timeSinceFailure	129

4.4	MBSFN Measurements	129

4.4.1	Introduction	129

4.4.2	MBSFN RSRP measurements	129

4.4.3	MBSFN RSRQ measurements	129

4.4.4	MCH BLER measurements	130

4.5	Proximity-based Services	130

4.5.1	Introduction	130

4.5.2	Requirements	130

4.5.2.1	Interruptions with ProSe Direct Discovery	130

4.5.2.2	Interruptions with ProSe Direct Communication	130

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	130

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	131

4.6	Cell Selection and Re-selection Requirements for UE category NB1	131

4.6.1	Cell Selection	131

4.6.2	Cell Re-selection	131

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	132

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	132

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	133

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	134

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	135

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	136

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	138

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	139

4.6.2.7	Maximum interruption in paging reception in normal coverage	140

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	140

4.6.2.8	UE measurement capability	140

4.6.2.9	WUS receptions for NB1	141

4.7	Cell Selection and Re-selection Requirements for UE category M1	141

4.7.1	Cell Selection	141

4.7.2	Cell Re-selection	141

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	141

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	141

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	142

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	142

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	144

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	145

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	145

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	146

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	146

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	147

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	148

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	149

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	151

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	151

4.7.2.3	WUS receptions for UE category M1	152

4.8	Idle State Positioning Measurement Requirements for UE category NB1	152

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	152

4.8.1.1	RSTD Measurement Reporting Delay	154

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	154

4.8.2.1	RSTD Measurement Reporting Delay	155

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	156

4.8.3.1	RSTD Measurement Reporting Delay	157

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	158

4.8.4.1	RSTD Measurement Reporting Delay	159

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	160

4.8.5.1	Measurement Reporting Delay	161

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	161

4.8.6.1	Measurement Reporting Delay	162

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	162

4.8.7.1	Measurement Reporting Delay	163

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	164

4.8.8.1	Measurement Reporting Delay	165

4.9	Idle Mode CA Measurement	166

4.9.1	Introduction	166

4.9.2	Requirement	166

4.9.2.1	Detected cell requirement during state transition and Idle mode	166

4.9.2.2	Measurements of inter-frequency CA candidate cells	166

4.9.2.3	Measurements on serving cell	167

4A	E-UTRAN RRC_INACTIVE state mobility	167

4A.1	Cell Re-selection	167

4A.1.1	Introduction	167

4A.1.2	Requirements	167

4A.1.2.1	UE measurement capability	167

4A.1.2.2	Measurement and evaluation of serving cell	167

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	167

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	167

4A.1.2.5	Evaluation of cell re-selection criteria	168

4A.1.2.6	Maximum interruption in paging reception	168

4A.1.2.7	Measurements of inter-RAT NR cells	168

5	E-UTRAN RRC_CONNECTED state mobility	168

5.1	E-UTRAN Handover	168

5.1.1	Introduction	168

5.1.2	Requirements	168

5.1.2.1	E-UTRAN FDD – FDD	168

5.1.2.1.1	Handover delay	168

5.1.2.1.2	Interruption time	169

5.1.2.2	E-UTRAN FDD – TDD	170

5.1.2.2.1	(Void)	170

5.1.2.2.2	(Void)	170

5.1.2.3	E-UTRAN TDD – FDD	170

5.1.2.3.1	(Void)	170

5.1.2.3.2	(Void)	170

5.1.2.4	E-UTRAN TDD – TDD	170

5.1.2.4.1	Handover delay	170

5.1.2.4.2	Interruption time	171

5.1.2.5	E-UTRAN HD–FDD	172

5.1.2.5.1	Handover delay	172

5.1.2.5.2	Interruption time	173

5.2	Void	174

5.3	Handover to other RATs	174

5.3.1	E-UTRAN - UTRAN FDD Handover	174

5.3.1.1	Introduction	174

5.3.1.1.1	Handover delay	174

5.3.1.1.2	Interruption time	174

5.3.2	E-UTRAN - UTRAN TDD Handover	175

5.3.2.1	Introduction	175

5.3.2.2	Requirements	175

5.3.2.2.1	Handover delay	175

5.3.2.2.2	Interruption time	175

5.3.3	E-UTRAN - GSM Handover	176

5.3.3.1	Introduction	176

5.3.3.2	Requirements	176

5.3.3.2.1	Handover delay	176

5.3.3.2.2	Interruption time	176

5.3.4	E-UTRAN - NR FR1 Handover	177

5.3.4.1	Introduction	177

5.3.4.2	Handover delay	177

5.3.4.3	Interruption time	177

5.3.5	E-UTRAN - NR FR2 Handover	177

5.3.5.1	Introduction	177

5.3.5.2	Handover delay	178

5.3.5.3	Interruption time	178

5.4	Handover to Non-3GPP RATs	179

5.4.1	E-UTRAN – HRPD Handover	179

5.4.1.1	Introduction	179

5.4.1.1.1	Handover delay	179

5.4.1.1.2	Interruption time	179

5.4.2	E-UTRAN – cdma2000 1X Handover	179

5.4.2.1	Introduction	179

5.4.2.1.1	Handover delay	179

5.4.2.1.2	Interruption time	180

5.5	E-UTRAN Handover for Cat-M1 UEs	180

5.5.1	Introduction	180

5.5.2	Requirements in CEModeA	180

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	180

5.5.2.1.1	Handover delay	180

5.5.2.1.2	Interruption time	180

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	181

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	181

5.5.2.3.1	Void	181

5.5.2.3.2	Void	181

5.5.3	Requirements in CEModeB	181

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	181

5.5.3.1.1	Handover delay	181

5.5.3.1.2	Interruption time	182

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	182

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	182

5.6	Void	182

6	RRC Connection Mobility Control	182

6.1	RRC Re-establishment	182

6.1.1	Introduction	183

6.1.2	Requirements	183

6.1.2.1	UE Re-establishment delay requirement	183

6.2	Random Access	183

6.2.1	Introduction	183

6.2.2	Requirements	183

6.2.2.1	Contention based random access	184

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	184

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	184

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	184

6.2.2.1.4	Void	184

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	184

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	184

6.2.2.2	Non-Contention based random access	184

6.2.2.2.1	Correct behaviour when receiving Random Access Response	184

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	184

6.2.3	Requirements for Cat-M1 UEs	185

6.3	RRC Connection Release with Redirection	185

6.3.1	Introduction	185

6.3.2	Requirements	185

6.3.2.1	RRC connection release with redirection to UTRAN FDD	185

6.3.2.2	RRC connection release with redirection to GERAN	185

6.3.2.3	RRC connection release with redirection to UTRAN TDD	186

6.3.2.4	RRC connection release with redirection to NR	186

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	187

6.4.1	Introduction	187

6.4.2	Requirements	187

6.5	RRC Re-establishment for NB-IoT UEs	187

6.5.1	Introduction	187

6.5.2	Requirements	188

6.5.2.1	UE Re-establishment delay requirement in normal coverage	188

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	188

6.6	Random Access for UE category NB1	189

6.6.1	Introduction	189

6.6.2	Requirements	189

6.6.2.1	Correct behaviour when receiving Random Access Response reception	189

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	189

6.6.2.3	Correct behaviour when receiving a NACK on msg3	189

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	189

6.6.2.5	Correct behaviour when contention Resolution timer expires	190

6.6.2.6	MSG3-based channel quality report for UE Category NB1	190

6.6.3	Requirements for NPRACH configuration	190

6.7	RRC Re-establishment for Cat-M1 UEs	190

6.7.1	Introduction	190

6.7.2	Requirements	191

6.7.2.1	UE Re-establishment delay requirement for CEModeA	191

6.7.2.2	UE Re-establishment delay requirement for CEModeB	191

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	192

6.8.1	Introduction	192

6.8.2	Requirements	192

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	192

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	193

6.9.1	Introduction	193

6.9.2	Requirements	193

7	Timing and signalling characteristics	193

7.1	UE transmit timing	193

7.1.1	Introduction	193

7.1.2	Requirements	194

7.2	UE timer accuracy	195

7.2.1	Introduction	195

7.2.2	Requirements	195

7.3	Timing Advance	196

7.3.1	Introduction	196

7.3.2	Requirements	196

7.3.2.1	Timing Advance adjustment delay	196

7.3.2.2	Timing Advance adjustment accuracy	196

7.4	Cell phase synchronization accuracy (TDD)	196

7.4.1	Definition	196

7.4.2	Minimum requirements	196

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	197

7.5.1	Introduction	197

7.5.2	eNodeB Synchronization Requirements	197

7.5.2.1	Synchronized E-UTRAN	197

7.5.2.2	Non-Synchronized E-UTRAN	197

7.6	Radio Link Monitoring	198

7.6.1	Introduction	198

7.6.2	Requirements	199

7.6.2.1	Minimum requirement when no DRX is used	199

7.6.2.2	Minimum requirement when DRX is used	200

7.6.2.3	Minimum requirement at transitions	201

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	201

7.6.2.5	Minimum requirement under IDC Interference	202

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	202

7.7.1	Introduction	202

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	202

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	203

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	204

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	206

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	207

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	207

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	208

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	208

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	208

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	210

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	210

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	211

7.7.14	SCell Activation Delay Requirement for Dormant SCell	212

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	213

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	214

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	215

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	215

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	217

7.8	Interruptions with Carrier Aggregation	219

7.8.1	Introduction	219

7.8.2	Requirements	220

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	220

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	220

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	220

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	220

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	220

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	220

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	221

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	221

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	221

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	222

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	222

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	223

7.8.2.13	Interruptions at SRS carrier based switching	223

7.8.2.14	Interruptions at SCell activation of dormant SCell for intra-band CA	224

7.8.2.15	Interruptions at SCell activation of dormant SCell for inter-band CA	224

7.8.2.16	Interruptions at SCell activation of multiple dormant SCells	224

7.8.2.17	Interruptions during CQI measurement on dormant SCell	225

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	225

7.9.1	Introduction	225

7.9.2	Minimum Requirements for Interband Carrier Aggregation	225

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	225

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	226

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	226

7.10.1	Introduction	226

7.10.2	Requirements	226

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	226

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	226

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	227

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	227

7.11	Radio Link Monitoring for UE Category 0	227

7.11.1	Introduction	227

7.11.2	Requirements for FD-FDD and TDD	229

7.11.2.1	Minimum requirement when no DRX is used	229

7.11.2.2	Minimum requirement when DRX is used	229

7.11.2.3	Minimum requirement at transitions	230

7.11.3	Requirements for HD-FDD	230

7.11.3.1	Minimum requirement when no DRX is used	230

7.11.3.2	Minimum requirement when DRX is used	230

7.11.3.3	Minimum requirement at transitions	231

7.12	Interruptions with Dual Connectivity	231

7.12.1	Introduction	231

7.12.2	Requirements	231

7.12.2.1	Interruptions at PSCell addition/release	231

7.12.2.2	Interruptions at transitions between active and non-active during DRX	232

7.12.2.3	Interruptions at transitions from non-DRX to DRX	232

7.12.2.4	Interruptions at SCell addition/release	232

7.12.2.5	Interruptions at SCell activation/deactivation	232

7.12.2.6	Interruptions during measurements on SCC	233

7.12.2.7	Interruptions at SRS carrier based switching	233

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	234

7.13.1	Definition	234

7.13.2	Minimum requirements	234

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	234

7.14.1	Introduction	234

7.14.2	PSCell Addition Delay Requirement	234

7.14.3	PSCell Release Delay Requirement	235

7.15	Maximum Receive Timing Difference in Dual Connectivity	235

7.15.1	Introduction	235

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	235

7.16	Proximity-based Services	236

7.16.1	Introduction	236

7.16.2	Requirements	236

7.16.2.1	ProSe UE transmission timing	236

7.16.2.1.1	Serving cell or PCell as timing reference	236

7.16.2.1.2	SCell or non-serving cell as timing reference	236

7.16.3	Interruptions with ProSe	236

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	237

7.16.3.2	Interruptions at ProSe Direct Communication configuration	237

7.16.3.3	Interruptions during ProSe Direct Discovery	237

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	237

7.16.3.5	Interruptions during ProSe Direct Communication	238

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	238

7.16.4.1	Measurement and evaluation of selected cell	238

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	238

7.16.5	Selection / Reselection of ProSe relay UE	239

7.16.6	ProSe operation under deactivated SCell	239

7.17	Maximum Transmission Timing Difference in Dual Connectivity	240

7.17.1	Introduction	240

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	240

7.18.1	Introduction	240

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	240

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	240

7.19	Radio Link Monitoring for UE Category M1	240

7.19.1	Introduction	240

7.19.2	Requirements for FD-FDD and TDD CE mode A	241

7.19.2.1	Minimum requirement when no DRX is used	242

7.19.2.2	Minimum requirement when DRX is used	243

7.19.2.3	Minimum requirement at transitions	244

7.19.3	Requirements for HD-FDD with CE mode A	244

7.19.3.1	Minimum requirement when no DRX is used	244

7.19.3.2	Minimum requirement when DRX is used	244

7.19.3.3	Minimum requirement at transitions	246

7.19.4	Requirements for FD-FDD and TDD with CE mode B	246

7.19.4.1	Minimum requirement when no DRX is used	247

7.19.4.2	Minimum requirement when DRX is used	248

7.19.4.3	Minimum requirement at transitions	249

7.19.5	Requirements for HD-FDD with CE mode B	249

7.19.5.1	Minimum requirement when no DRX is used	249

7.19.5.2	Minimum requirement when DRX is used	249

7.19.5.3	Minimum requirement at transitions	251

7.20	UE transmit timing for NB-IoT	251

7.20.1	Introduction	251

7.20.2	Requirements	251

7.21	 UE timer accuracy for NB-IoT	252

7.21.1	Introduction	252

7.21.2	Requirements	252

7.22	Timing Advance for NB-IoT	252

7.22.1	Introduction	252

7.22.2	Requirements	252

7.22.2.1	Timing Advance adjustment delay	252

7.22.2.2	Timing Advance adjustment accuracy	252

7.23	Radio Link Monitoring for Category NB1 UE	252

7.23.1	Introduction	252

7.23.2	Requirements for Category NB1 UE	252

7.23.2.1	Minimum requirement when no DRX is used	253

7.23.2.2	Minimum requirement when DRX is used	253

7.23.2.3	Minimum requirement at transitions	254

7.24	UE transmit timing for Category M1	254

7.24.1	Introduction	254

7.24.2	Requirements	254

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	255

7.25.1	Definition	255

7.25.2	Minimum requirements	255

7.26	UE transmit timing for Category M2	256

7.26.1	Introduction	256

7.26.2	Requirements	256

7.27	UE timer accuracy for category M1	256

7.27.1	Introduction	256

7.27.2	Requirements	256

7.28	Timing Advance for Category M1	256

7.28.1	Introduction	256

7.28.2	Requirements	256

7.29	Interruptions requirements with FeMBMS	256

7.29.1	Introduction	256

7.29.2	Requirements	257

7.30	Numerology switching delay requirements with FeMBMS	257

7.30.1	Introduction	257

7.30.2	Requirements	257

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	257

7.31.1	Introduction	257

7.31.2	NR PSCell Addition Delay Requirement	257

7.31.3	NR PSCell Release Delay Requirement	258

7.32	Interruptions with EN-DC	258

7.32.1	Introduction	258

7.32.2	Requirements	259

7.32.2.1	Interruptions at PSCell addition/release	259

7.32.2.2	Interruptions at transitions between active and non-active during DRX	259

7.32.2.3	Interruptions at transitions from non-DRX to DRX	259

7.32.2.4	Interruptions at SCell addition/release	259

7.32.2.5	Interruptions at SCell activation/deactivation	259

7.32.2.6	Interruptions during measurements on SCC	260

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	260

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	260

7.32.2.7	Interruptions at active BWP switching	260

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	261

7.33.1	Introduction	261

7.33.2	Requirements	261

7.34	Void	261

7.35	Interruptions with SFTD measurements	261

7.35.1	Introduction	261

7.35.2	Requirements	261

7.36	Interruptions with NE-DC	262

7.32.1	Introduction	262

7.36.2	Requirements	262

7.36.2.1	Interruptions at transitions between active and non-active during DRX	262

7.36.2.2	Interruptions at transitions from non-DRX to DRX	262

7.36.2.3	Interruptions at SCell addition/release	262

7.36.2.4	Interruptions at SCell activation/deactivation	263

7.36.2.5	Interruptions during measurements on SCC	263

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	263

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	263

7.36.2.6	Interruptions at active BWP switching	263

8	UE Measurements Procedures in RRC_CONNECTED State	264

8.1	General Measurement Requirements	264

8.1.1	Introduction	264

8.1.2	Requirements	264

8.1.2.1	UE measurement capability	264

8.1.2.1.1	Monitoring of multiple layers using gaps	271

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	272

8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)	273

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	275

8.1.2.1.2	Network controlled small gap	275

8.1.2.2	E-UTRAN intra frequency measurements	277

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	277

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	282

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	286

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	287

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	288

8.1.2.3	E-UTRAN inter frequency measurements	289

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	289

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	295

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	301

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	301

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	302

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	303

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	304

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	305

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	306

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	313

8.1.2.4	Inter RAT measurements	313

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	313

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	318

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	318

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	322

8.1.2.4.5	E-UTRAN FDD – GSM measurements	322

8.1.2.4.6	E-UTRAN TDD – GSM measurements	327

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	327

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	329

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	329

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	329

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	330

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	330

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	330

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	330

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	332

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	332

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	332

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	332

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	333

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	333

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	335

8.1.2.4.21	E-UTRAN FDD – NR measurements	335

8.1.2.4.22	E-UTRAN TDD – NR measurements	338

8.1.2.4.23	Void	338

8.1.2.4.24	Void	338

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	338

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	340

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	340

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	340

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	342

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	344

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	345

8.1.2.6.5	Void	347

8.1.2.6.6	Void	347

8.1.2.6.7	Void	347

8.1.2.6.8	Void	347

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	347

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	348

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	349

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	351

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	353

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	355

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	356

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	358

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	360

8.1.2.7	E-UTRAN E-CID Measurements	362

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	362

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	363

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	364

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	365

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	365

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	365

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	368

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	371

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	375

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	378

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	378

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	379

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	379

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	380

8.1.2.10	Void	380

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	380

8.2.1	Introduction	380

8.2.2	Requirements	380

8.3	Measurements for E-UTRA carrier aggregation	385

8.3.1	Introduction	385

8.3.2	Measurements of the primary component carrier	385

8.3.3	Measurements of a secondary component carrier	385

8.3.3.1	Measurements of a secondary component carrier with active SCell	385

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	386

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	386

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	387

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	389

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	389

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	389

8.4.1	Introduction	389

8.4.2	Measurements on the primary component carrier	390

8.4.3	Measurements on a secondary component carrier	390

8.4.4	Measurements on both primary component carrier and a secondary component carrier	391

8.4.5	Measurements on different secondary component carriers	392

8.5	Measurements for UE category 0	393

8.5.1	Introduction	393

8.5.2 	Requirements	393

8.5.2.1 	E-UTRAN intra frequency measurements	393

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	393

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	397

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	399

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	402

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	403

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	404

8.6	Discovery signal measurements	405

8.6.1	Introduction	405

8.6.2	Requirements for CRS based discovery signal measurements	405

8.6.2.1	E-UTRAN intra frequency measurements	405

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	405

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	408

8.6.2.2	E-UTRAN inter frequency measurements	410

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	410

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	413

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	415

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	416

8.6.3	Requirements for CSI-RS based discovery signal measurements	416

8.6.3.1	E-UTRAN intra frequency measurements	416

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	416

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	419

8.6.3.2	E-UTRAN inter frequency measurements	421

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	421

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	424

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	427

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	427

8.7	Discovery signal measurements for E-UTRA carrier aggregation	427

8.7.1	Introduction	427

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	427

8.7.2.1	Measurements of the primary component carrier	427

8.7.2.2	Measurements of a secondary component carrier	427

8.7.2.3	Measurements of a secondary component carrier with active SCell	427

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	428

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	428

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	429

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	431

8.7.3.1	Measurements of the primary component carrier	431

8.7.3.2	Measurements of a secondary component carrier	431

8.7.3.3	Measurements of a secondary component carrier with active SCell	431

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	431

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	431

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	432

8.8	Measurements for E-UTRA dual connectivity	434

8.8.1	Introduction	434

8.8.2	Intra-frequency measurements requirements on PCell	434

8.8.3	Intra-frequency measurements requirements on PSCell	434

8.8.4	Inter-frequency and inter-RAT measurement requirements	434

8.8.5	Intra-frequency measurements with autonomous gaps	435

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	435

8.8.5.2	ECGI reporting delay	436

8.8.6	Inter-frequency measurements with autonomous gaps	436

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	436

8.8.6.2	ECGI reporting delay	437

8.8.7	SSTD Measurements	437

8.8.7.1	Introduction	437

8.8.7.2	SSTD Measurement requirements	437

8.8.7.3	SSTD Measurement Reporting Delay	437

8.8.8	Intra-frequency measurements requirements on SCell	438

8.9	MBSFN Measurements	438

8.9.1	Introduction	438

8.9.2	MBSFN RSRP Measurements	438

8.9.3	MBSFN RSRQ Measurements	438

8.9.4	MCH BLER Measurements	439

8.10	Proximity-based Services	439

8.10.1	Introduction	439

8.10.2	Requirements	439

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	439

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	439

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	440

8.11.1	Introduction	440

8.11.2	CRS based discovery signal measurements	440

8.11.2.1	E-UTRAN intra-frequency measurements	440

8.11.2.1.1	Requirements	441

8.11.2.1.1.1	Requirements when no DRX is used	441

8.11.2.1.1.1.1	Measurement Reporting Requirements	442

8.11.2.1.1.2	Requirements when DRX is used	443

8.11.2.1.1.2.1	Measurement Reporting Requirements	445

8.11.2.2	E-UTRAN inter-frequency measurements	446

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	446

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	450

8.11.3	CSI-RS based discovery signal measurements	450

8.11.3.1	E-UTRAN intra-frequency measurements	450

8.11.3.1.1	Requirements	450

8.11.3.1.1.1	Requirements when no DRX is used	450

8.11.3.1.1.1.1	Measurement Reporting Requirements	452

8.11.3.1.1.2	Requirements when DRX is used	452

8.11.3.1.1.2.1	Measurement Reporting Requirements	453

8.11.3.2	E-UTRAN inter-frequency measurements	454

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	454

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	458

8.11.4	RSSI measurements	458

8.11.4.1	E-UTRAN intra-frequency measurements	458

8.11.4.2	E-UTRAN inter-frequency measurements	458

8.11.5	Channel occupancy measurements	458

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	458

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	459

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	459

8.12.1	Introduction	459

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	459

8.12.2.1	Introduction	459

8.12.2.2	Measurements of a secondary component carrier	459

8.12.2.3	Measurements of a secondary component carrier with active SCell	459

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	459

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	460

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	462

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	465

8.12.3.1	Introduction	465

8.12.3.2	Measurements of a secondary component carrier	465

8.12.3.3	Measurements of a secondary component carrier with active SCell	465

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	465

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	465

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	467

8.13	Measurements for UE Category M1	469

8.13.1	Introduction	469

8.13.2	Requirements for UE category M1 with CE mode A	469

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	470

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	470

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	474

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	476

8.13.2.2	Void	480

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	480

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	480

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	483

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	485

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	486

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	486

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	488

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	490

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	491

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	491

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	491

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	492

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	492

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	492

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	493

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	493

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	494

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	495

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	495

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	496

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	500

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	502

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	506

8.13.3	Requirements for UE category M1 with CE mode B	506

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	506

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	506

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	511

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	513

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	519

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	520

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	520

8.13.3.2	Void	521

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	521

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	521

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	524

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	526

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	527

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	527

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	527

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	527

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	528

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	528

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	529

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	529

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	529

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	533

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	536

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	540

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	541

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	541

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	543

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	545

8.14	Measurements for UE category NB1	546

8.14.1	Introduction	546

8.14.2	NB-IoT intra frequency measurements under normal coverage	546

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	546

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	546

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	546

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	546

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	546

8.15	Void	547

8.16	Measurements for UE Category M2	547

8.16.1	Introduction	547

8.16.2	Requirements for UE category M2 with CE mode A	547

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	547

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	548

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	548

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	549

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	549

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	549

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	550

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	550

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	552

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	555

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	555

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	555

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	557

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	560

8.16.3	Requirements for UE category M2 with CE mode B	560

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	560

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	560

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	563

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	565

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	566

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	566

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	568

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	570

8.17	Measurements for E-UTRA – NR Dual Connectivity	571

8.17.1	Introduction	571

8.17.1.1	Measurement Gap Sharing	571

8.17.1A	Intrafrequency Measurements	572

8.17.2	SFTD Measurements	572

8.17.2.1	Introduction	572

8.17.2.2	SFTD Measurement requirements	572

8.17.2.3	SFTD Measurement Reporting Delay	572

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	573

8.17.3.1	Introduction	573

8.17.3.2	E-UTRAN FDD inter frequency measurements	573

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	573

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	574

8.17.3.3	E-UTRAN TDD inter frequency measurements	576

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	576

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	578

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	580

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	580

8.17.4.1.1	NR Inter-RAT cell identification	580

8.17.4.1.2	NR Inter-RAT measurement	582

8.17.4.1.3	NR Inter-RAT measurement reporting	583

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	583

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	583

8.17.5.1	Introduction	583

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	584

8.17.5.2.1	Identification of a new UTRA FDD cell	584

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	584

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	584

8.17.5.2.4	Periodic Reporting	585

8.17.5.2.5	Event Triggered Reporting	585

8.17.5.2.6	Event-triggered Periodic Reporting	585

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	585

8.17.5.3.1	Periodic Reporting	586

8.17.5.3.2	Event Triggered Reporting	586

8.17.5.3.3	Event-triggered Periodic Reporting	587

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	587

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	587

8.17.7.1	Introduction	587

8.17.7.2	Identification of a new UTRA FDD cell for SON	587

8.17.7.2.1	Requirements when no DRX is used	587

8.17.7.2.2	Requirements when DRX is used	588

8.17.7.2.3	Reporting Delay	588

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	589

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	589

8.17.9.1	Introduction	589

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	589

8.17.9.2.1	Identification of a new UTRA FDD cell	589

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	589

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	590

8.17.9.2.4	Periodic Reporting	590

8.17.9.2.5	Event Triggered Reporting	590

8.17.9.2.6	Event-triggered Periodic Reporting	591

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	591

8.17.9.3.1	Periodic Reporting	592

8.17.9.3.2	Event Triggered Reporting	592

8.17.9.3.3	Event-triggered Periodic Reporting	592

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	593

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	593

8.17.11.1	Introduction	593

8.17.11.2	Identification of a new UTRA TDD cell for SON	593

8.17.11.2.1	Requirements when no DRX is used	593

8.17.11.2.2	Requirements when DRX is used	593

8.17.11.2.3	Reporting Delay	594

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	594

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	595

8.17.13.1	Introduction	595

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	595

8.17.13.2.1	GSM carrier RSSI	595

8.17.13.2.2	BSIC verification	595

8.17.13.2.3	Enhanced BSIC verification	597

8.17.13.2.4	Periodic Reporting	597

8.17.13.2.5	Event Triggered Reporting	597

8.17.13.2.6	Event-triggered Periodic Reporting	598

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	598

8.17.13.3.1	GSM carrier RSSI	598

8.17.13.3.2	BSIC verification	598

8.17.13.3.3	Periodic Reporting	600

8.17.13.3.4	Event Triggered Reporting	600

8.17.13.3.5	Event-triggered Periodic Reporting	600

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	600

8.18	Measurements for non-BL/CE UE	600

8.18.1	Introduction	600

8.18.2	Requirements for non-BL/CE UE with CE Mode B	600

8.18.2.1	E-UTRAN intra frequency measurements	600

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	600

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	601

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	601

8.19	Measurements for NR – E-UTRA Dual Connectivity	602

8.19.1	Introduction	602

8.19.2	Intra-frequency Measurements	603

8.19.3	Inter-frequency Measurements	603

8.19.4	Intra-frequency RSTD Measurements	603

8.19.5	Intra-frequency E-CID Measurements	603

9	Measurements performance requirements for UE	603

9.1	E-UTRAN measurements	604

9.1.1	Introduction	604

9.1.2	Intra-frequency RSRP Accuracy Requirements	604

9.1.2.1	Absolute RSRP Accuracy	604

9.1.2.2	Relative Accuracy of RSRP	605

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	606

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	606

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	607

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	608

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	609

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	610

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	611

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	611

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	611

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	612

9.1.2B.1	Introduction	612

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	612

9.1.3	Inter-frequency RSRP Accuracy Requirements	613

9.1.3.1	Absolute RSRP Accuracy	613

9.1.3.2	Relative Accuracy of RSRP	614

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	615

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	615

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	616

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	616

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	617

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	618

9.1.3B.1	Introduction	618

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	618

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	618

9.1.4	RSRP Measurement Report Mapping	619

9.1.5	Intra-frequency RSRQ Accuracy Requirements	619

9.1.5.1	Absolute RSRQ Accuracy	619

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	620

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	621

9.1.5.4	Absolute WB-RSRQ Accuracy	622

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	623

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	624

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	624

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	624

9.1.5B.1	Introduction	624

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	624

9.1.6	Inter-frequency RSRQ Accuracy Requirements	625

9.1.6.1	Absolute RSRQ Accuracy	625

9.1.6.2	Relative Accuracy of RSRQ	626

9.1.6.3	Absolute WB-RSRQ Accuracy	627

9.1.6.4	Relative WB-RSRQ Accuracy	627

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	628

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	629

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	629

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	629

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	630

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	631

9.1.6B.1	Introduction	631

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	631

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	631

9.1.7	RSRQ Measurement Report Mapping	632

9.1.8	Power Headroom	632

9.1.8.1	Period	633

9.1.8.2	Reporting Delay	633

9.1.8.3	Void	633

9.1.8.4	Report Mapping	633

9.1.9	UE Rx – Tx time difference	633

9.1.9.1	Measurement Requirement	633

9.1.9.2	Measurement Report mapping	634

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	635

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	636

9.1.10	Reference Signal Time Difference (RSTD)	637

9.1.10.1	Intra-Frequency Accuracy Requirement	637

9.1.10.2	Inter-Frequency Accuracy Requirement	638

9.1.10.3	RSTD Measurement Report Mapping	640

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	640

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	641

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	642

9.1.11	Carrier aggregation measurement accuracy	643

9.1.11.1	Primary component carrier accuracy requirement	644

9.1.11.2	Secondary component carrier accuracy requirement	644

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	644

9.1.11.4	Secondary component carrier relative accuracy requirement	644

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	644

9.1.13	Measurement accuracy for UE category 0	645

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	645

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	645

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	646

9.1.14	Accuracy requirements for Discovery Signal Measurements	647

9.1.14.1	Introduction	647

9.1.14.2	RSRP measurements in discovery signal occasions	647

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	647

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	647

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	647

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	648

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	649

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	649

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	649

9.1.14.3.3	CSI-RSRP measurement report mapping	650

9.1.14.4	RSRQ measurements in discovery signal occasions	650

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	650

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	651

9.1.15.1.1	Primary component carrier accuracy requirement	651

9.1.15.1.2	Secondary component carrier accuracy requirement	651

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	651

9.1.15.1.4	Secondary component carrier relative accuracy requirement	651

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	651

9.1.15.2.1	Primary component carrier accuracy requirement	651

9.1.15.2.2	Secondary component carrier accuracy requirement	651

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	651

9.1.15.2.4	Secondary component carrier relative accuracy requirement	651

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	652

9.1.17	RS-SINR Measurements	652

9.1.17.1	Measurement Report Mapping	652

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	653

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	653

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	653

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	653

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	654

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	655

9.1.18.1	Introduction	655

9.1.18.2	RSRP measurements	655

9.1.18.2.1	RSRP measurement report mapping	655

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	655

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	656

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	656

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	657

9.1.18.3	RSRQ measurements	657

9.1.18.3.1	RSRQ measurement report mapping	657

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	657

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	658

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	658

9.1.18.4	CSI-RSRP measurements	659

9.1.18.4.1	CSI-RSRP measurement report mapping	659

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	659

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	659

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	660

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	660

9.1.18.5	RSSI measurements	661

9.1.18.5.1	RSSI measurement report mapping	661

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	661

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	662

9.1.18.6	Channel occupancy measurements	662

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	662

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	662

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	663

9.1.19.1	Introduction	663

9.1.19.2	Accuracy requirements for measurements on SCC	663

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	663

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	663

9.1.20	SFN and Subframe Time Difference (SSTD)	664

9.1.20.1	SSTD Accuracy Requirement	664

9.1.20.2	SSTD Measurement Report Mapping	664

9.1.21	Measurement accuracy for UE category M1	665

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	665

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	666

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	667

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	668

9.1.21.5	RSRP Measurement Report Mapping	669

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	669

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	670

9.1.21.8	RSRQ Measurement Report Mapping	671

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	671

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	672

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	673

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	674

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	676

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	676

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	677

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	678

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	680

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	680

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	681

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	682

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	685

9.1.22	Measurement accuracy for UE Category NB1	688

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	688

9.1.22.2	Void	689

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	689

9.1.22.4	Void	690

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	690

9.1.22.6	Void	691

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	691

9.1.22.8	Void	692

9.1.22.9	NRSRP Measurement Report Mapping	692

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	692

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	693

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	694

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	695

9.1.22.14	NRSRQ Measurement Report Mapping	696

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	697

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	697

9.1.23	Power Headroom for UE Category NB1	698

9.1.23.1	Period	698

9.1.23.2	Reporting Delay	698

9.1.23.3	Report Mapping for UE Category NB1	698

9.1.23.3.1	Void	700

9.1.23.3.2	Void	700

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	700

9.1.24	Void	701

9.1.25	Measurement accuracy for UE category M2	701

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	701

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	702

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	703

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	704

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	706

9.1.26	Measurement Accuracy for non-BL CE UE	707

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	708

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	708

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	709

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	710

9.1.26.5	RSRP Measurement Report Mapping	711

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	711

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	711

9.1.26.8	RSRQ Measurement Report Mapping	712

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	712

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	713

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	714

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	715

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	716

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	716

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	716

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	717

9.1.27	SFN and frame Timing Difference (SFTD)	718

9.1.27.1	SFTD Accuracy Requirement	718

9.2	UTRAN FDD Measurements	720

9.2.1	UTRAN FDD CPICH RSCP	720

9.2.2	Void	721

9.2.3	UTRAN FDD CPICH Ec/No	721

9.3	UTRAN TDD Measurements	721

9.3.1	UTRAN TDD P-CCPCH RSCP	722

9.3.2	Void	722

9.3.3	Void	722

9.4	GSM Measurements	722

9.4.1	GSM carrier RSSI	722

9.5	CDMA2000 1x RTT Measurements	722

9.5.1	CDMA2000 1x RTT Pilot Strength	722

9.6	PCMAX,c	723

9.6.1	Report Mapping	723

9.6.2 	Estimation Period	723

9.6.3	Reporting Delay	723

9.7	IEEE802.11 Measurements	723

9.7.1	WLAN RSSI	723

9.7.2	WLAN RSSI Measurement Report Mapping	723

9.8	MBSFN Measurements	724

9.8.1	Introduction	724

9.8.2	MBSFN RSRP	724

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	724

9.8.2.2	MBSFN RSRP measurement report mapping	725

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	725

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	726

9.8.3	MBSFN RSRQ	726

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	726

9.8.3.2	MBSFN RSRQ measurement report mapping	727

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	727

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	728

9.8.4	MCH BLER	728

9.8.4.1	Measurement report mapping for MCH BLER	728

9.8.4.2	Measurement report mapping for MCH Block Number	729

9.9	ProSe Measurements	730

9.9.1	Introduction	730

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	730

9.9.2.1	Absolute S-RSRP Accuracy	730

9.9.2.2	Relative Accuracy of S-RSRP	731

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	732

9.9.3.1	Absolute SD-RSRP Accuracy	732

9.9.3.2	Relative Accuracy of SD-RSRP	732

9.10	V2X Measurements	733

9.10.1	Introduction	733

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	733

9.10.2.1	Absolute S-RSRP Accuracy	733

9.10.2.2	Relative Accuracy of S-RSRP	734

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	734

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	734

9.10.4	S-RSSI Measurement Accuracy Requirements	735

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	735

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	735

9.11	NR Measurements	736

9.11.1	NR SS-RSRP Measurements	736

9.11.2	NR SS-RSRQ Measurements	736

9.11.3	NR SS-SINR Measurements	737

10	Measurements Performance Requirements for E-UTRAN	737

10.1	Received Interference Power	737

10.1.1	Absolute accuracy requirement	737

10.1.2	Relative accuracy requirement	738

10.1.3	Received Interference Power measurement report mapping	738

10.2	Angle of Arrival (AOA)	738

10.2.1	Range/mapping	738

10.3	Timing Advance (TADV)	738

10.3.1	Report mapping	738

11	ProSe Requirements in Any Cell Selection state	739

11.1	Introduction	739

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	739

11.2.1	Introduction	739

11.2.2	ProSe UE transmission timing	739

11.3	Initiation/Cease of SLSS Transmissions	740

11.3.1	Introduction	740

11.3.2	Requirements	740

11.4	Measurements for ProSe in Any Cell Selection State	740

11.4.1	Introduction	740

11.4.2	Requirements	740

11.4.2.1	E-UTRA FDD	740

11.4.2.2	E-UTRA TDD	741

11.5	Selection / Reselection of ProSe Synchronization Reference	741

11.5.1	Introduction	741

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	741

11.5.2.1	Introduction	741

11.5.2.2	Requirements	741

11.6	Void	742

11.7	Selection / Reselection of ProSe relay UE	742

11.7.1	Introduction	742

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	742

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	742

12.1	Introduction	742

12.2	Transmit Timing	743

12.2.1	GNSS as timing reference	743

12.3	Interruption	743

12.4	Reliability of GNSS signal	743

13	V2X Requirements	743

13.1	Introduction	743

13.2	UE Transmit Timing	744

13.2.1	Introduction	744

13.2.2	GNSS as synchronization reference source	744

13.2.3	Serving cell/PCell as synchronization reference source	744

13.2.4	SyncRef UE as synchronization reference source	744

13.3	Initiation/Cease of SLSS Transmissions	744

13.3.1	Introduction	744

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	744

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	745

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	745

13.4	Selection / Reselection of V2X Synchronization Reference Source	746

13.5	Autonomous Resource Selection/Reselection measurements	747

13.5.1	Introduction	747

13.5.2	PSSCH-RSRP measurements	747

13.5.3	S-RSSI measurements	747

13.6	Congestion Control measurements	747

13.7	Interruption	747

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	747

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	747

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	748

13.8	Reliability of GNSS signal	748

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	748

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	749

Annex A (normative):	Test Cases	750

A.1	Purpose of annex	750

A.2	Requirement classification for statistical testing	750

A.2.1	Types of requirements in TS 36.133	750

A.2.1.1	Time and delay requirements on UE higher layer actions	750

A.2.1.2	Measurements of power levels, relative powers and time	750

A.2.1.3	Implementation requirements	751

A.2.1.4	Physical layer timing requirements	751

A.3	RRM test configurations	752

A.3.1	Reference Measurement Channels	752

A.3.1.1	PDSCH	752

A.3.1.1.1	FDD	752

A.3.1.1.2	TDD	756

A.3.1.1.3	FDD for UE category 0	759

A.3.1.1.4	HD-FDD for UE category 0	760

A.3.1.1.5	TDD for UE category 0	761

A.3.1.1.6	Frame Structure 3	762

A.3.1.2	PCFICH/PDCCH/PHICH	763

A.3.1.2.1	FDD	763

A.3.1.2.2	TDD	763

A.3.1.2.3	HD-FDD for UE category 0	764

A.3.1.2.4	FS 3	764

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	764

A.3.1.3.1	FDD in CEModeA	765

A.3.1.3.2	HD-FDD in CEModeA	765

A.3.1.3.3	TDD in CEModeA	766

A.3.1.3.4	FDD in CEModeB	766

A.3.1.3.5	HD-FDD in CEModeB	767

A.3.1.3.6	TDD in CEModeB	767

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	768

A.3.1.4.1	FDD in CEModeA	768

A.3.1.4.2	HD-FDD in CEModeA	769

A.3.1.4.3	TDD in CEModeA	770

A.3.1.4.4	FDD in CEModeB	771

A.3.1.4.5	 HD-FDD in CEModeB	772

A.3.1.4.6	TDD in CEModeB	772

A.3.1.5	NPDSCH Reference Channel for UE category NB1	773

A.3.1.5.1	HD-FDD in-band operation	773

A.3.1.5.2	Void	774

A.3.1.5.3	HD-FDD standalone operation	774

A.3.1.5.4	Void	775

A.3.1.5.5	HD-FDD guard band operation	775

A.3.1.5.6	Void	776

A.3.1.5.7	TDD in-band operation	776

A.3.1.5.8	TDD standalone operation	776

A.3.1.5.9	TDD guard band operation	777

A.3.1.6	NPDCCH Reference Channel for UE category NB1	777

A.3.1.6.1	In-band operation	777

A.3.1.6.2	Void	778

A.3.1.6.3	Standalone operation	778

A.3.1.6.4	Void	778

A.3.1.6.5	Guard band operation	778

A.3.1.6.6	Void	779

A.3.2	OFDMA Channel Noise Generator (OCNG)	779

A.3.2.1	OCNG Patterns for FDD	779

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	780

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	780

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	781

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	781

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	782

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	783

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	783

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	783

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	784

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	785

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	785

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	786

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	786

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	787

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	787

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	788

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	788

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	789

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	789

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	790

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	790

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	791

A.3.2.2	OCNG Patterns for TDD	791

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	792

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	792

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	793

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	793

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	794

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	795

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	796

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	797

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	797

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	798

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	798

A.3.2.3	OCNG Patterns for Narrowband IoT	799

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	800

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	801

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	801

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	802

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	803

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	804

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	806

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	806

A.3.2.4	OCNG Patterns for V2X sidelink	807

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	807

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	808

A.3.3	Reference DRX Configurations	808

A.3.4	ABS Transmission Configurations	809

A.3.4.1	Non-MBSFN ABS Transmission Configurations	809

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	809

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	809

A.3.4.2	MBSFN ABS Transmission Configurations	810

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	810

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	811

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	811

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	811

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	812

A.3.6.1	Introduction	812

A.3.7	Test Cases with Different Channel Bandwidths	812

A.3.7.1	Introduction	812

A.3.7.2	Principle of testing	812

A.3.8	Antenna Configuration	812

A.3.8.1	Antenna connection for 4 Rx capable UEs	812

A.3.8.1.1 Introduction	812

A.3.8.1.2 Principle of testing	812

A.3.8.1.2.1 Single carrier tests	812

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	813

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	814

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	814

A.3.8.2	Antenna connection for 8 Rx capable UEs	814

A.3.8.2.1	Introduction	814

A.3.8.2.2	Principle of testing	814

A.3.8.2.2.1	Single carrier tests	814

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	815

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	815

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	815

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	815

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	815

A.3.9.1	Introduction	815

A.3.9.2	Principle of testing	815

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	816

A.3.10.1	Introduction	816

A.3.10.2	Principle of testing	816

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	816

A.3.11.1	Introduction	816

A.3.11.2	Principle of Testing	816

A.3.12	Proximity-based Services	816

A.3.12.1	Introduction	816

A.3.12.2	Reference DRX configurations for ProSe tests	816

A.3.12.3	Test Cases with Different Channel Bandwidths	817

A.3.12.3.1	Introduction	817

A.3.12.3.2	Principle of testing	817

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	817

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	823

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	825

A.3.12.6.1	FDD	825

A.3.12.7	Reference measurement channels for ProSe Direct Communication	825

A.3.12.7.1	FDD	825

A.3.12.8	ProSe Receive Traffic Generator	826

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	826

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	826

A.3.13	Time Offset between Cells	827

A.3.13.1	Introduction	827

A.3.13.2	Definition	827

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	827

A.3.14.1	Introduction	827

A.3.14.2	Principle of testing	827

A.3.15	Dual connectivity test cases with different combination of duplex mode	827

A.3.15.1	Introduction	827

A.3.15.2	Principle of testing	827

A.3.16	Reference PRACH Configurations	828

A.3.17	Listen before talk model	828

A.3.17.1	Introduction	828

A.3.17.2	Definition	828

A.3.18	Reference NPRACH Configurations	829

A.3.19	Dual connectivity test cases with different bandwidth combinations	830

A.3.19.1	Introduction	830

A.3.19.2	Principle of testing	830

A.3.20	Category M1 UE Test Cases	831

A.3.20.1	Introduction	831

A.3.20.2	Principle of Cat-M1 UE Testing	831

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	832

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	833

A.3.21.1	Introduction	833

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	833

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	834

A.3.22	Category 1bis UE Test Cases	835

A.3.22.1	Introduction	835

A.3.22.2	Principle of Category 1bis UE Testing	835

A.3.23	Category NB2 UE Test Cases	838

A.3.23.1	Introduction	838

A.3.23.2	Principle of Category NB2 UE Testing	838

A.3.24	V2X sidelink communication	840

A.3.24.1	Introduction	840

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	841

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	844

A.3.25	Category M2 UE Test Cases	845

A.3.25.1	Introduction	845

A.3.25.2	Principle of Cat-M2 UE Testing	845

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	846

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	847

A.3.26.1	Introduction	847

A.3.26.2	Principle of testing	847

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	847

A.3.27.1	Introduction	847

A.3.27.2	Principle of INACTIVE cell re-selection Testing	847

A.4	E-UTRAN RRC_IDLE state	847

A.4.2	Cell Re-Selection	847

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	847

A.4.2.1.1	Test Purpose and Environment	847

A.4.2.1.2	Test Requirements	849

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	850

A.4.2.2.1	Test Purpose and Environment	850

A.4.2.2.2	Test Requirements	851

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	852

A.4.2.3.1	Test Purpose and Environment	852

A.4.2.3.2	Test Requirements	853

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	854

A.4.2.4.1	Test Purpose and Environment	854

A.4.2.4.2	Test Requirements	855

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	856

A.4.2.5.1	Test Purpose and Environment	856

A.4.2.5.2	Test Requirements	857

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	858

A.4.2.6.1	Test Purpose and Environment	858

A.4.2.6.2	Test Requirements	859

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	860

A.4.2.7.1	Test Purpose and Environment	860

A.4.2.7.2	Test Requirements	861

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	862

A.4.2.8.1	Test Purpose and Environment	862

A.4.2.8.2	Test Requirements	863

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	864

A.4.2.9.1	Test Purpose and Environment	864

A.4.2.9.2	Test Requirements	864

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	864

A.4.2.10.1	Test Purpose and Environment	864

A.4.2.10.2	Test Requirements	868

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	868

A.4.2.11.1	Test Purpose and Environment	868

A.4.2.11.2	Test Requirements	872

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	872

A.4.2.12.1	Test Purpose and Environment	872

A.4.2.12.2	Test Requirements	874

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	875

A.4.2.13.1	Test Purpose and Environment	875

A.4.2.13.2	Test Requirements	876

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	877

A.4.2.14.1	Test Purpose and Environment	877

A.4.2.14.2	Test Requirements	878

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	879

A.4.2.15.1	Test Purpose and Environment	879

A.4.2.15.2	Test Requirements	880

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	881

A.4.2.16.1	Test Purpose and Environment	881

A.4.2.16.2	Test Requirements	882

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	883

A.4.2.17.1	Test Purpose and Environment	883

A.4.2.17.2	Test Requirements	884

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	885

A.4.2.18.1	Test Purpose and Environment	885

A.4.2.18.2	Test Requirements	887

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	888

A.4.2.19.1	Test Purpose and Environment	888

A.4.2.19.2	Test Requirements	890

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	891

A.4.2.20.1	Test Purpose and Environment	891

A.4.2.20.2	Test Requirements	892

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	893

A.4.2.21.1	Test Purpose and Environment	893

A.4.2.21.2	Test Requirements	894

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	895

A.4.2.22.1	Test Purpose and Environment	895

A.4.2.22.2	Test Requirements	896

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	897

A.4.2.23.1	Test Purpose and Environment	897

A.4.2.23.2	Test Requirements	898

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	899

A.4.2.24.1	Test Purpose and Environment	899

A.4.2.24.2	Test Requirements	902

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	903

A.4.2.25.1	Test Purpose and Environment	903

A.4.2.25.2	Test Requirements	904

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	905

A.4.2.26.1	Test Purpose and Environment	905

A.4.2.26.2	Test Requirements	906

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	907

A.4.2.27.1	Test Purpose and Environment	907

A.4.2.27.2	Test Requirements	908

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	909

A.4.2.28.1	Test Purpose and Environment	909

A.4.2.28.2	Test Requirements	910

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	911

A.4.2.29.1	Test Purpose and Environment	911

A.4.2.29.2	Test Requirements	912

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	913

A.4.2.30.1	Test Purpose and Environment	913

A.4.2.30.2	Test Requirements	914

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	915

A.4.2.31.1	Test Purpose and Environment	915

A.4.2.31.2	Test Requirements	916

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	917

A.4.2.32.1	Test Purpose and Environment	917

A.4.2.32.2	Test Requirements	918

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	919

A.4.2.33.1	Test Purpose and Environment	919

A.4.2.33.2	Test Requirements	920

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	921

A.4.2.34.1	Test Purpose and Environment	921

A.4.2.34.2	Test Requirements	922

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	923

A.4.2.35.1	Test Purpose and Environment	923

A.4.2.35.2	Test Requirements	925

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	926

A.4.2.36.1	Test Purpose and Environment	926

A.4.2.36.2	Test Requirements	928

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	929

A.4.2.37.1	Test Purpose and Environment	929

A.4.2.37.2	Test Requirements	931

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	932

A.4.2.38.1	Test Purpose and Environment	932

A.4.2.38.2	Test Requirements	934

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	935

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	935

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	935

A.4.3.1.1.1	Test Purpose and Environment	935

A.4.3.1.1.2	Test Requirements	937

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	937

A.4.3.1.2.1	Test Purpose and Environment	937

A.4.3.1.2.2	Test Requirements	940

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	940

A.4.3.1.3.1	Test Purpose and Environment	940

A.4.3.1.3.2	Test Requirements	943

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	943

A.4.3.1.4.1	Test Purpose and Environment	943

A.4.3.1.4.2	Test Requirements	944

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	944

A.4.3.1.5.1	Test Purpose and Environment	944

A.4.3.1.5.2	Test Requirements	947

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	948

A.4.3.2.1	Test Purpose and Environment	948

A.4.3.2.1.1	Void	948

A.4.3.2.1.2	1.28Mcps TDD option	948

A.4.3.2.1.3	Void	950

A.4.3.2.2	Test Requirements	950

A.4.3.2.2.1	1.28Mcps TDD option	950

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	950

A.4.3.2A.1	Test Purpose and Environment	950

A.4.3.2A.2	Test Requirements	954

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	954

A.4.3.3.1	Test Purpose and Environment	954

A.4.3.3.2	Test Requirements	957

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	957

A.4.3.3A.1	Test Purpose and Environment	957

A.4.3.3A.2	Test Requirements	962

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	963

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	963

A.4.3.4.1.1	Test Purpose and Environment	963

A.4.3.4.1.2	Test Requirements	965

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	965

A.4.3.4.2.1	Test Purpose and Environment	965

A.4.3.4.2.2	Test Requirements	967

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	967

A.4.3.4.3.1	Test Purpose and Environment	967

A.4.3.4.3.2	Test Requirements	970

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	970

A.4.3.4.4.1	Test Purpose and Environment	970

A.4.3.4.4.2	Test Requirements	974

A.4.4	E-UTRAN to GSM Cell Re-Selection	974

A.4.4.1	E-UTRAN FDD – GSM:	974

A.4.4.1.1	Test Purpose and Environment	974

A.4.4.1.2	Test Requirements	976

A.4.4.2	E-UTRAN TDD – GSM:	976

A.4.4.2.1	Test Purpose and Environment	976

A.4.4.2.2	Test Requirements	978

A.4.5	E-UTRAN to HRPD Cell Re-Selection	979

A.4.5.1	E-UTRAN FDD – HRPD	979

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	979

A.4.5.1.1.1	Test Purpose and Environment	979

A.4.5.1.1.2	Test Requirements	981

A.4.5.2	E-UTRAN TDD – HRPD	981

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	981

A.4.5.2.1.1	Test Purpose and Environment	981

A.4.5.2.1.2	Test Requirements	984

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	984

A.4.6.1	E-UTRAN FDD – cdma2000 1X	984

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	984

A.4.6.1.1.1	Test Purpose and Environment	984

A.4.6.1.1.2	Test Requirements	987

A.4.6.2	E-UTRAN TDD – cdma2000 1X	987

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	987

A.4.6.2.1.1	Test Purpose and Environment	987

A.4.6.2.1.2	Test Requirements	990

A.4.7	Idle State Positioning Measurement for UE category NB1	990

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	990

A.4.7.1.1	Test Purpose and Environment	990

A.4.7.1.2	Test Requirements	994

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	995

A.4.7.2.1	Test Purpose and Environment	995

A.4.7.4.2	Test Requirements	998

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	999

A.4.7.3.1	Test Purpose and Environment	999

A.4.7.3.2	Test Requirements	1002

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1003

A.4.7.4.1	Test Purpose and Environment	1003

A.4.7.4.2	Test Requirements	1006

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1007

A.5.1	E-UTRAN Handover	1007

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1007

A.5.1.1.1	Test Purpose and Environment	1007

A.5.1.1.2	Test Requirements	1009

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1009

A.5.1.2.1	Test Purpose and Environment	1009

A.5.1.2.2	Test Requirements	1011

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1011

A.5.1.3.1	Test Purpose and Environment	1011

A.5.1.3.2	Test Requirements	1013

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1013

A.5.1.4.1	Test Purpose and Environment	1013

A.5.1.4.2	Test Requirements	1015

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1015

A.5.1.5.1	Test Purpose and Environment	1015

A.5.1.5.2	Test Requirements	1017

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1017

A.5.1.6.1	Test Purpose and Environment	1017

A.5.1.6.2	Test Requirements	1019

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1019

A.5.1.7.1	Test Purpose and Environment	1019

A.5.1.7.2	Test Requirements	1022

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1022

A.5.1.8.1	Test Purpose and Environment	1022

A.5.1.8.2	Test Requirements	1025

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1025

A.5.1.9.1	Test Purpose and Environment	1025

A.5.1.9.2	Test Requirements	1026

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1026

A.5.1.10.1	Test Purpose and Environment	1026

A.5.1.10.2	Test Requirements	1028

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1028

A.5.1.11.1	Test Purpose and Environment	1028

A.5.1.11.2	Test Requirements	1030

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1030

A.5.1.12.1	Test Purpose and Environment	1030

A.5.1.12.2	Test Requirements	1032

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1032

A.5.1.13.1	Test Purpose and Environment	1032

A.5.1.13.2	Test Requirements	1034

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1035

A.5.1.14.1	Test Purpose and Environment	1035

A.5.1.14.2	Test Requirements	1036

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1037

A.5.1.15.1	Test Purpose and Environment	1037

A.5.1.15.2	Test Requirements	1038

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1039

A.5.1.16.1	Test Purpose and Environment	1039

A.5.1.16.2	Test Requirements	1040

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1041

A.5.1.17.1	Test Purpose and Environment	1041

A.5.1.17.2	Test Requirements	1042

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1043

A.5.1.18.1	Test Purpose and Environment	1043

A.5.1.18.2	Test Requirements	1044

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1045

A.5.1.19.1	Test Purpose and Environment	1045

A.5.1.19.2	Test Requirements	1046

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1047

A.5.1.20.1	Test Purpose and Environment	1047

A.5.1.20.2	Test Requirements	1048

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1049

A.5.1.21.1	Test Purpose and Environment	1049

A.5.1.21.2	Test Requirements	1050

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1050

A.5.1.22.1	Test Purpose and Environment	1050

A.5.1.22.2	Test Requirements	1052

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1052

A.5.1.23.1	Test Purpose and Environment	1052

A.5.1.23.2	Test Requirements	1054

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1054

A.5.1.24.1	Test Purpose and Environment	1054

A.5.1.24.2	Test Requirements	1056

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1056

A.5.1.25.1	Test Purpose and Environment	1056

A.5.1.25.2	Test Requirements	1058

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1059

A.5.1.26.1	Test Purpose and Environment	1059

A.5.1.26.2	Test Requirements	1060

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1061

A.5.1.27.1	Test Purpose and Environment	1061

A.5.1.27.2	Test Requirements	1062

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1063

A.5.1.28.1	Test Purpose and Environment	1063

A.5.1.28.2	Test Requirements	1064

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1065

A.5.1.29.1	Test Purpose and Environment	1065

A.5.1.29.2	Test Requirements	1066

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1067

A.5.1.30.1	Test Purpose and Environment	1067

A.5.1.30.2	Test Requirements	1068

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1069

A.5.1.31.1	Test Purpose and Environment	1069

A.5.1.31.2	Test Requirements	1070

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1071

A.5.1.32.1	Test Purpose and Environment	1071

A.5.1.32.2	Test Requirements	1072

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1073

A.5.1.33.1	Test Purpose and Environment	1073

A.5.1.13.2	Test Requirements	1074

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1075

A.5.1.34.1	Test Purpose and Environment	1075

A.5.1.34.2	Test Requirements	1076

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1077

A.5.1.35.1	Test Purpose and Environment	1077

A.5.1.35.2	Test Requirements	1078

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1079

A.5.1.36.1	Test Purpose and Environment	1079

A.5.1.36.2	Test Requirements	1080

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1081

A.5.1.37.1	Test Purpose and Environment	1081

A.5.1.37.2	Test Requirements	1082

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1083

A.5.1.38.1	Test Purpose and Environment	1083

A.5.1.38.2	Test Requirements	1084

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1085

A.5.1.39.1	Test Purpose and Environment	1085

A.5.1.39.2	Test Requirements	1086

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1087

A.5.1.40.1	Test Purpose and Environment	1087

A.5.1.40.2	Test Requirements	1088

A.5.2	E-UTRAN Handover to other RATs	1089

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1089

A.5.2.1.1	Test Purpose and Environment	1089

A.5.2.1.2	Test Requirements	1091

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1092

A.5.2.2.1	Test Purpose and Environment	1092

A.5.2.2.2	Test Requirements	1095

A.5.2.3 E-UTRAN FDD- GSM Handover	1095

A.5.2.3.1	Test Purpose and Environment	1095

A.5.2.3.2	Test Requirements	1096

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1097

A.5.2.4.1	Test Purpose and Environment	1097

A.5.2.4.1.1	Void	1097

A.5.2.4.1.2	1.28 Mcps TDD option	1097

A.5.2.4.1.3	Void	1099

A.5.2.4.2	Test Requirements	1099

A.5.2.4.2.1	Void	1099

A.5.2.4.2.2	1.28 Mcps TDD option	1099

A.5.2.4.2.3	Void	1099

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1099

A.5.2.5.1	Test Purpose and Environment	1099

A.5.2.5.1.3	Void	1102

A.5.2.5.2	Test Requirements	1102

A.5.2.5.2.1	Void	1102

A.5.2.5.2.2	1.28 Mcps TDD option	1102

A.5.2.5.2.3	Void	1102

A.5.2.6	E-UTRAN TDD - GSM Handover	1102

A.5.2.6.1	Test Purpose and Environment	1102

A.5.2.6.2	Test Requirements	1104

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1105

A.5.2.7.1	Test Purpose and Environment	1105

A.5.2.7.2	Test Requirements	1107

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1107

A.5.2.8.1	Test Purpose and Environment	1107

A.5.2.8.2	Test Requirements	1108

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1109

A.5.2.9.1	Test Purpose and Environment	1109

A.5.2.9.2	Test Requirements	1110

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1111

A.5.2.10.1	Test Purpose and Environment	1111

A.5.2.10.2	Test Requirements	1113

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1113

A.5.2.10A.1	Test Purpose and Environment	1113

A.5.2.10A.2	Test Requirements	1116

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1116

A.5.2.10B.1	Test Purpose and Environment	1116

A.5.2.10B.2	Test Requirements	1119

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1119

A.5.2.11.1	Test Purpose and Environment	1119

A.5.2.11.2	Test Requirements	1119

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1120

A.5.3.1	E-UTRAN FDD – HRPD Handover	1120

A.5.3.1.1	Test Purpose and Environment	1120

A.5.3.1.2	Test Requirements	1122

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1122

A.5.3.2.1	Test Purpose and Environment	1122

A.5.3.2.2	Test Requirements	1125

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1125

A.5.3.3.1	Test Purpose and Environment	1125

A.5.3.3.2	Test Requirements	1127

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1128

A.5.3.4.1	Test Purpose and Environment	1128

A.5.3.4.2	Test Requirements	1129

A.5.3.5	E-UTRAN TDD – HRPD Handover	1130

A.5.3.5.1	Test Purpose and Environment	1130

A.5.3.5.2	Test Requirements	1132

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1133

A.5.3.6.1	Test Purpose and Environment	1133

A.5.3.6.2	Test Requirements	1136

A.6	RRC Connection Control	1136

A.6.1	RRC Re-establishment	1136

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1136

A.6.1.1.1	Test Purpose and Environment	1136

A.6.1.1.2	Test Requirements	1137

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1138

A.6.1.2.1	Test Purpose and Environment	1138

A.6.1.2.2	Test Requirements	1139

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1140

A.6.1.3.1	Test Purpose and Environment	1140

A.6.1.3.2	Test Requirements	1141

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1142

A.6.1.4.1	Test Purpose and Environment	1142

A.6.1.4.2	Test Requirements	1143

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1144

A.6.1.5.1	Test Purpose and Environment	1144

A.6.1.5.2	Test Requirements	1144

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1144

A.6.1.6.1	Test Purpose and Environment	1144

A.6.1.6.2	Test Requirements	1146

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1147

A.6.1.7.1	Test Purpose and Environment	1147

A.6.1.7.2	Test Requirements	1148

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1149

A.6.1.8.1	Test Purpose and Environment	1149

A.6.1.8.2	Test Requirements	1150

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1151

A.6.1.9.1	Test Purpose and Environment	1151

A.6.1.9.2	Test Requirements	1152

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1153

A.6.1.10.1	Test Purpose and Environment	1153

A.6.1.10.2	Test Requirements	1154

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1155

A.6.1.11.1	Test Purpose and Environment	1155

A.6.1.11.2	Test Requirements	1156

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1157

A.6.1.12.1	Test Purpose and Environment	1157

A.6.1.12.2	Test Requirements	1158

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1159

A.6.1.13.1	Test Purpose and Environment	1159

A.6.1.13.2	Test Requirements	1160

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1161

A.6.1.14.1	Test Purpose and Environment	1161

A.6.1.14.2	Test Requirements	1162

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1163

A.6.1.15.1	Test Purpose and Environment	1163

A.6.1.15.2	Test Requirements	1165

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1166

A.6.1.16.1	Test Purpose and Environment	1166

A.6.1.16.2	Test Requirements	1168

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1169

A.6.1.17.1	Test Purpose and Environment	1169

A.6.1.17.2	Test Requirements	1170

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1171

A.6.1.18.1	Test Purpose and Environment	1171

A.6.1.18.2	Test Requirements	1172

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1173

A.6.1.19.1	Test Purpose and Environment	1173

A.6.1.19.2	Test Requirements	1174

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1175

A.6.1.20.1	Test Purpose and Environment	1175

A.6.1.20.2	Test Requirements	1176

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1177

A.6.1.21.1	Test Purpose and Environment	1177

A.6.1.21.2	Test Requirements	1178

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1179

A.6.1.22.1	Test Purpose and Environment	1179

A.6.1.22.2	Test Requirements	1180

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1181

A.6.1.23.1	Test Purpose and Environment	1181

A.6.1.23.2	Test Requirements	1183

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1184

A.6.1.24.1	Test Purpose and Environment	1184

A.6.1.24.2	Test Requirements	1186

A.6.2	Random Access	1187

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1187

A.6.2.1.1	Test Purpose and Environment	1187

A.6.2.1.2	Test Requirements	1188

A.6.2.1.2.1	Random Access Response Reception	1188

A.6.2.1.2.2	No Random Access Response Reception	1188

A.6.2.1.2.3	Receiving a NACK on msg3	1188

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1189

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1189

A.6.2.1.2.6	Contention Resolution Timer expiry	1189

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1189

A.6.2.2.1	Test Purpose and Environment	1189

A.6.2.2.2	Test Requirements	1190

A.6.2.2.2.1	Random Access Response Reception	1191

A.6.2.2.2.2	No Random Access Response Reception	1191

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1191

A.6.2.3.1	Test Purpose and Environment	1191

A.6.2.3.2	Test Requirements	1193

A.6.2.3.2.1	Random Access Response Reception	1193

A.6.2.3.2.2	No Random Access Response reception	1193

A.6.2.3.2.3	Receiving a NACK on msg3	1193

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1194

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1194

A.6.2.3.2.6	Contention Resolution Timer expiry	1194

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1194

A.6.2.4.1	Test Purpose and Environment	1194

A.6.2.4.2	Test Requirements	1196

A.6.2.4.2.1	Random Access Response Reception	1196

A.6.2.4.2.2	No Random Access Response Reception	1196

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1196

A.6.2.5.1	Test Purpose and Environment	1196

A.6.2.5.2	Test Requirements	1197

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1197

A.6.2.6.1	Test Purpose and Environment	1197

A.6.2.6.2	Test Requirements	1197

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1198

A.6.2.7.1	Test Purpose and Environment	1198

A.6.2.7.2	Test Requirements	1200

A.6.2.7.2.1	Random Access Response Reception	1200

A.6.2.7.2.2	No Random Access Response Reception	1200

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1200

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1201

A.6.2.8.1	Test Purpose and Environment	1201

A.6.2.8.2	Test Requirements	1203

A.6.2.8.2.1	Random Access Response Reception	1203

A.6.2.8.2.2	No Random Access Response Reception	1203

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1203

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1204

A.6.2.9.1	Test Purpose and Environment	1204

A.6.2.9.2	Test Requirements	1206

A.6.2.9.2.1	Random Access Response Reception	1206

A.6.2.9.2.2	No Random Access Response Reception	1207

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1207

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1208

A.6.2.10.1	Test Purpose and Environment	1208

A.6.2.10.2	Test Requirements	1210

A.6.2.10.2.1	Random Access Response Reception	1210

A.6.2.10.2.2	No Random Access Response Reception	1211

A.6.2.10.2.3	Receiving a NACK on msg3	1211

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1211

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1211

A.6.2.10.2.6	Contention Resolution Timer expiry	1211

A.6.2.10.2.7	PRACH Resource Selection	1212

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1212

A.6.2.11.1	Test Purpose and Environment	1212

A.6.2.11.2	Test Requirements	1214

A.6.2.11.2.1	Random Access Response Reception	1214

A.6.2.11.2.2	No Random Access Response Reception	1215

A.6.2.11.2.3	Receiving a NACK on msg3	1215

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1215

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1215

A.6.2.11.2.6	Contention Resolution Timer expiry	1215

A.6.2.11.2.7	PRACH Resource Selection	1216

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1216

A.6.2.12.1	Test Purpose and Environment	1216

A.6.2.12.2	Test Requirements	1218

A.6.2.12.2.1	Random Access Response Reception	1218

A.6.2.12.2.2	No Random Access Response Reception	1219

A.6.2.12.2.3	Receiving a NACK on msg3	1219

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1219

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1219

A.6.2.12.2.6	Contention Resolution Timer expiry	1219

A.6.2.12.2.7	PRACH Resource Selection	1220

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1220

A.6.2.13.1	Test Purpose and Environment	1220

A.6.2.13.2	Test Requirements	1222

A.6.2.13.2.1	Random Access Response Reception	1222

A.6.2.13.2.2	No Random Access Response Reception	1223

A.6.2.13.2.3	Receiving a NACK on msg3	1223

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1223

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1223

A.6.2.13.2.6	Contention Resolution Timer expiry	1223

A.6.2.13.2.7	PRACH Resource Selection	1224

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1224

A.6.2.14.1	Test Purpose and Environment	1224

A.6.2.14.2	Test Requirements	1226

A.6.2.14.2.1	Random Access Response Reception	1226

A.6.2.14.2.2	No Random Access Response Reception	1227

A.6.2.14.2.3	Receiving a NACK on msg3	1227

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1227

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1227

A.6.2.14.2.6	Contention Resolution Timer expiry	1227

A.6.2.14.2.7	PRACH Resource Selection	1228

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1228

A.6.2.15.1	Test Purpose and Environment	1228

A.6.2.15.2	Test Requirements	1230

A.6.2.15.2.1	Random Access Response Reception	1230

A.6.2.15.2.2	No Random Access Response Reception	1231

A.6.2.15.2.3	Receiving a NACK on msg3	1231

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1231

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1231

A.6.2.15.2.6	Contention Resolution Timer expiry	1231

A.6.2.15.2.7	PRACH Resource Selection	1232

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1232

A.6.2.16.1	Test Purpose and Environment	1232

A.6.2.16.2	Test Requirements	1235

A.6.2.16.2.1	Random Access Response Reception	1235

A.6.2.16.2.2	No Random Access Response Reception	1236

A.6.2.16.2.3	Receiving a NACK on msg3	1236

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1236

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1236

A.6.2.16.2.6	Contention Resolution Timer expiry	1236

A.6.2.16.2.7	NPRACH Resource Selection	1236

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1237

A.6.2.17.1	Test Purpose and Environment	1237

A.6.2.17.2	Test Requirements	1240

A.6.2.17.2.1	Random Access Response Reception	1240

A.6.2.17.2.2	No Random Access Response Reception	1241

A.6.2.17.2.3	Receiving a NACK on msg3	1241

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1241

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1241

A.6.2.17.2.6	Contention Resolution Timer expiry	1241

A.6.2.17.2.7	NPRACH Resource Selection	1241

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1242

A.6.2.18.1	Test Purpose and Environment	1242

A.6.2.18.2	Test Requirements	1245

A.6.2.18.2.1	Random Access Response Reception	1245

A.6.2.18.2.2	No Random Access Response Reception	1246

A.6.2.18.2.3	Receiving a NACK on msg3	1246

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1246

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1246

A.6.2.18.2.6	Contention Resolution Timer expiry	1246

A.6.2.18.2.7	NPRACH Resource Selection	1246

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1247

A.6.2.19.1	Test Purpose and Environment	1247

A.6.2.19.2	Test Requirements	1249

A.6.2.19.2.1	Random Access Response Reception	1249

A.6.2.19.2.2	No Random Access Response Reception	1250

A.6.2.19.2.3	Receiving a NACK on msg3	1250

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1250

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1250

A.6.2.19.2.6	Contention Resolution Timer expiry	1250

A.6.2.19.2.7	NPRACH Resource Selection	1250

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1251

A.6.2.20.1	Test Purpose and Environment	1251

A.6.2.20.2	Test Requirements	1253

A.6.2.20.2.1	Random Access Response Reception	1253

A.6.2.20.2.2	No Random Access Response Reception	1254

A.6.2.20.2.3	Receiving a NACK on msg3	1254

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1254

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1254

A.6.2.20.2.6	Contention Resolution Timer expiry	1254

A.6.2.20.2.7	NPRACH Resource Selection	1254

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1255

A.6.2.21.1	Test Purpose and Environment	1255

A.6.2.21.2	Test Requirements	1257

A.6.2.21.2.1	Random Access Response Reception	1257

A.6.2.21.2.2	No Random Access Response Reception	1258

A.6.2.21.2.3	Receiving a NACK on msg3	1258

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1258

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1258

A.6.2.21.2.6	Contention Resolution Timer expiry	1258

A.6.2.21.2.7	NPRACH Resource Selection	1258

A.6.3	RRC Connection Release with Redirection	1259

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1259

A.6.3.1.1	Test Purpose and Environment	1259

A.6.3.1.2	Test Requirements	1261

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1261

A.6.3.2.1	Test Purpose and Environment	1261

A.6.3.2.2	Test Requirements	1263

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1263

A.6.3.3.1	Test Purpose and Environment	1263

A.6.3.3.2	Test Requirements	1264

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1265

A.6.3.4.1	Test Purpose and Environment	1265

A.6.3.4.2	Test Requirements	1266

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1267

A.6.3.5.1	Test Purpose and Environment	1267

A.6.3.5.2	Test Requirements	1269

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1269

A.6.3.6.1	Test Purpose and Environment	1269

A. 6.3.6.2	Test Requirements	1272

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1272

A.6.3.7.1	Test Purpose and Environment	1272

A.6.3.7.2	Test Requirements	1275

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1275

A.6.3.8.1	Test Purpose and Environment	1275

A.6.3.8.2	Test Requirements	1278

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1278

A.6.3.9.1	Test Purpose and Environment	1278

A.6.3.9.2	Test Requirements	1280

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1280

A.6.3.10.1	Test Purpose and Environment	1280

A.6.3.10.2	Test Requirements	1282

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1282

A.6.3.11.1	Test Purpose and Environment	1282

A.6.3.11.2	Test Requirements	1284

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1284

A.6.3.12.1	Test Purpose and Environment	1284

A.6.3.12.2	Test Requirements	1287

A.7	Timing and Signalling Characteristics	1288

A.7.1	UE Transmit Timing	1288

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1288

A.7.1.1.1	Test Purpose and Environment	1288

A.7.1.1.2	Test Requirements	1289

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1290

A.7.1.2.1	Test Purpose and Environment	1290

A.7.1.2.2	Test Requirements	1292

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1293

A.7.1.3.1	Test Purpose and Environment	1293

A.7.1.3.2	Test Requirements	1295

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1296

A.7.1.4.1	Test Purpose and Environment	1296

A.7.1.4.2	Test Requirements	1297

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1298

A.7.1.4A.1	Test Purpose and Environment	1298

A.7.1.4A.2	Test Requirements	1298

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1298

A.7.1.5.1	Test Purpose and Environment	1298

A.7.1.5.2	Test Requirements	1299

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1299

A.7.1.6.1	Test Purpose and Environment	1299

A.7.1.6.2	Test Requirements	1301

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1302

A.7.1.7.1	Test Purpose and Environment	1302

A.7.1.7.2	Test Requirements	1303

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1304

A.7.1.7A.1	Test Purpose and Environment	1304

A.7.1.7A.2	Test Requirements	1304

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1304

A.7.1.7B.1	Test Purpose and Environment	1304

A.7.1.7B.2	Test Requirements	1304

A.7.1.8	Void	1305

A.7.1.8.1	Void	1305

A.7.1.8.2	Void	1305

A.7.1.9	Void	1305

A.7.1.9.1	Void	1305

A.7.1.9.2	Void	1305

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1305

A.7.1.10.1	Test Purpose and Environment	1305

A.7.1.10.2	Test Requirements	1307

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1307

A.7.1.11.1	Test Purpose and Environment	1307

A.7.1.11.2	Test Requirements	1309

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1309

A.7.1.12.1	Test Purpose and Environment	1309

A.7.1.12.2	Test Requirements	1311

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1311

A.7.1.13.1	Test Purpose and Environment	1311

A.7.1.13.2	Test Requirements	1313

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1314

A.7.1.14.1	Test Purpose and Environment	1314

A.7.1.14.2	Test Requirements	1315

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1315

A.7.1.15.1	Test Purpose and Environment	1315

A.7.1.15.2	Test Requirements	1316

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1317

A.7.1.16.1	Test Purpose and Environment	1317

A.7.1.16.2	Test Requirements	1318

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1319

A.7.1.17.1	Test Purpose and Environment	1319

A.7.1.17.2	Test Requirements	1321

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1322

A.7.1.18.1	Test Purpose and Environment	1322

A.7.1.18.2	Test Requirements	1324

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1325

A.7.1.19.1	Test Purpose and Environment	1325

A.7.1.19.2	Test Requirements	1326

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1326

A.7.1.20.1	Test Purpose and Environment	1326

A.7.1.20.2	Test Requirements	1328

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1328

A.7.1.21.1	Test Purpose and Environment	1328

A.7.1.21.2	Test Requirements	1330

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1330

A.7.1.22.1	Test Purpose and Environment	1330

A.7.1.22.2	Test Requirements	1332

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1332

A.7.1.23.1	Test Purpose and Environment	1332

A.7.1.23.2	Test Requirements	1334

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1334

A.7.1.24.1	Test Purpose and Environment	1334

A.7.1.24.2	Test Requirements	1335

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1336

A.7.1.25.1	Test Purpose and Environment	1336

A.7.1.25.2	Test Requirements	1337

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1338

A.7.1.26.1	Test Purpose and Environment	1338

A.7.1.26.2	Test Requirements	1339

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1340

A.7.1.27.1	Test Purpose and Environment	1340

A.7.1.27.2	Test Requirements	1342

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1342

A.7.1.28.1	Test Purpose and Environment	1342

A.7.1.28.2	Test Requirements	1344

A.7.2	UE Timing Advance	1345

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1345

A.7.2.1.1	Test Purpose and Environment	1345

A.7.2.1.2	Test Requirements	1346

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1347

A.7.2.2.1	Test Purpose and Environment	1347

A.7.2.2.2	Test Requirements	1349

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1349

A.7.2.3.1	Test Purpose and Environment	1349

A.7.2.3.2	Test Requirements	1349

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1349

A.7.2.4.1	Test Purpose and Environment	1349

A.7.2.4.2	Test Requirements	1352

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1352

A.7.2.5.1	Test Purpose and Environment	1352

A.7.2.5.2	Test Requirements	1354

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1354

A.7.2.5A.1	Test Purpose and Environment	1354

A.7.2.5A.2	Test Requirements	1355

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1355

A.7.2.5B.1	Test Purpose and Environment	1355

A.7.2.5B.2	Test Requirements	1355

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1355

A.7.2.6.1	Test Purpose and Environment	1355

A.7.2.6.2	Test Requirements	1358

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1358

A.7.2.7.1	Test Purpose and Environment	1358

A.7.2.7.2	Test Requirements	1360

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1360

A.7.2.8.1	Test Purpose and Environment	1360

A.7.2.8.2	Test Requirements	1362

A.7.2.9.2	Test Requirements	1363

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1364

A.7.2.10.1	Test Purpose and Environment	1364

A.7.2.10.2	Test Requirements	1365

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1365

A.7.2.11.1	Test Purpose and Environment	1365

A.7.2.11.2	Test Requirements	1367

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1367

A.7.2.12.1	Test Purpose and Environment	1367

A.7.2.12.2	Test Requirements	1369

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1369

A.7.2.13.1	Test Purpose and Environment	1369

A.7.2.13.2	Test Requirements	1371

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1371

A.7.2.14.1	Test Purpose and Environment	1371

A.7.2.14.2	Test Requirements	1373

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1373

A.7.2.15.1	Test Purpose and Environment	1373

A.7.2.15.2	Test Requirements	1375

A.7.3	Radio Link Monitoring	1375

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1376

A.7.3.1.1	Test Purpose and Environment	1376

A.7.3.1.2	Test Requirements	1380

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1380

A.7.3.2.1	Test Purpose and Environment	1380

A.7.3.2.2	Test Requirements	1383

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1383

A.7.3.3.1	Test Purpose and Environment	1383

A.7.3.3.2	Test Requirements	1387

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1387

A.7.3.4.1	Test Purpose and Environment	1387

A.7.3.4.2	Test Requirements	1390

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1390

A.7.3.5.1	Test Purpose and Environment	1390

A.7.3.5.2	Test Requirements	1394

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1395

A.7.3.6.1	Test Purpose and Environment	1395

A.7.3.6.2	Test Requirements	1398

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1398

A.7.3.7.1	Test Purpose and Environment	1398

A.7.3.7.2	Test Requirements	1401

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1401

A.7.3.8.1	Test Purpose and Environment	1401

A.7.3.8.2	Test Requirements	1404

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1404

A.7.3.9.2	Test Requirements	1407

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1407

A.7.3.10.1	Test Purpose and Environment	1407

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1410

A.7.3.11.1	Test Purpose and Environment	1410

A.7.3.11.2	Test Requirements	1414

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1414

A.7.3.12.1	Test Purpose and Environment	1414

A.7.3.12.2	Test Requirements	1418

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1418

A.7.3.13.1	Test Purpose and Environment	1418

A.7.3.13.2	Test Requirements	1421

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1421

A.7.3.14.1	Test Purpose and Environment	1421

A.7.3.14.2	Test Requirements	1424

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1424

A.7.3.15.1	Test Purpose and Environment	1424

A.7.3.15.2	Test Requirements	1427

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1427

A.7.3.16.1	Test Purpose and Environment	1427

A.7.3.16.2	Test Requirements	1431

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1431

A.7.3.17.1	Test Purpose and Environment	1431

A.7.3.17.2	Test Requirements	1434

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1434

A.7.3.18.1	Test Purpose and Environment	1434

A.7.3.18.2	Test Requirements	1438

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1438

A.7.3.19.1	Test Purpose and Environment	1438

A.7.3.19.2	Test Requirements	1442

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1442

A.7.3.20.1	Test Purpose and Environment	1442

A.7.3.20.2	Test Requirements	1446

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1446

A.7.3.21.1	Test Purpose and Environment	1446

A.7.3.21.2	Test Requirements	1450

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1450

A.7.3.22.1	Test Purpose and Environment	1450

A.7.3.22.2	Test Requirements	1454

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1454

A.7.3.23.1	Test Purpose and Environment	1454

A.7.3.23.2	Test Requirements	1455

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1455

A.7.3.24.1	Test Purpose and Environment	1455

A.7.3.24.2	Test Requirements	1456

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1456

A.7.3.25.1	Test Purpose and Environment	1456

A.7.3.25.2	Test Requirements	1457

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1457

A.7.3.26.1	Test Purpose and Environment	1457

A.7.3.26.2	Test Requirements	1460

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1460

A.7.3.27.1	Test Purpose and Environment	1460

A.7.3.27.2	Test Requirements	1463

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1463

A.7.3.28.1	Test Purpose and Environment	1463

A.7.3.28.2	Test Requirements	1466

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1466

A.7.3.29.1	Test Purpose and Environment	1466

A.7.3.29.2	Test Requirements	1469

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1469

A.7.3.30.1	Test Purpose and Environment	1469

A.7.3.30.2	Test Requirements	1472

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1472

A.7.3.31.1	Test Purpose and Environment	1472

A.7.3.31.2	Test Requirements	1475

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1475

A.7.3.32.1	Test Purpose and Environment	1475

A.7.3.32.2	Test Requirements	1478

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1478

A.7.3.33.1	Test Purpose and Environment	1478

A.7.3.33.2	Test Requirements	1481

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1481

A.7.3.34.1	Test Purpose and Environment	1481

A.7.3.34.2	Test Requirements	1484

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1484

A.7.3.35.1	Test Purpose and Environment	1484

A.7.3.35.2	Test Requirements	1487

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1487

A.7.3.36.1	Test Purpose and Environment	1487

A.7.3.36.2	Test Requirements	1490

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1490

A.7.3.37.1	Test Purpose and Environment	1490

A.7.3.37.2	Test Requirements	1493

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1493

A.7.3.38.1	Test Purpose and Environment	1493

A.7.3.38.2	Test Requirements	1496

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1496

A.7.3.39.1	Test Purpose and Environment	1496

A.7.3.39.2	Test Requirements	1499

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1500

A.7.3.40.1	Test Purpose and Environment	1500

A.7.3.40.2	Test Requirements	1502

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1503

A.7.3.41.1	Test Purpose and Environment	1503

A.7.3.41.2	Test Requirements	1506

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1506

A.7.3.42.1	Test Purpose and Environment	1506

A.7.3.42.2	Test Requirements	1509

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1509

A.7.3.43.1	Test Purpose and Environment	1509

A.7.3.43.2	Test Requirements	1512

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1512

A.7.3.44.1	Test Purpose and Environment	1512

A.7.3.44.2	Test Requirements	1515

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1516

A.7.3.45.1	Test Purpose and Environment	1516

A.7.3.45.2	Test Requirements	1518

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1519

A.7.3.46.1	Test Purpose and Environment	1519

A.7.3.46.2	Test Requirements	1522

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1522

A.7.3.47.1	Test Purpose and Environment	1522

A.7.3.47.2	Test Requirements	1525

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1525

A.7.3.48.1	Test Purpose and Environment	1525

A.7.3.48.2	Test Requirements	1528

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1528

A.7.3.49.1	Test Purpose and Environment	1528

A.7.3.49.2	Test Requirements	1531

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1531

A.7.3.50.1	Test Purpose and Environment	1531

A.7.3.50.2	Test Requirements	1534

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1534

A.7.3.51.1	Test Purpose and Environment	1534

A.7.3.51.2	Test Requirements	1537

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1537

A.7.3.52.1	Test Purpose and Environment	1537

A.7.3.52.2	Test Requirements	1540

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1540

A.7.3.53.1	Test Purpose and Environment	1540

A.7.3.53.2	Test Requirements	1543

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1543

A.7.3.54.1	Test Purpose and Environment	1543

A.7.3.54.2	Test Requirements	1546

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1546

A.7.3.55.1	Test Purpose and Environment	1546

A.7.3.55.2	Test Requirements	1549

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1549

A.7.3.56.1	Test Purpose and Environment	1549

A.7.3.56.2	Test Requirements	1552

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1552

A.7.3.57.1	Test Purpose and Environment	1552

A.7.3.57.2	Test Requirements	1555

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1555

A.7.3.58.1	Test Purpose and Environment	1555

A.7.3.58.2	Test Requirements	1558

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1558

A.7.3.59.1	Test Purpose and Environment	1558

A.7.3.59.2	Test Requirements	1561

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1561

A.7.3.60.1	Test Purpose and Environment	1561

A.7.3.60.2	Test Requirements	1565

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1565

A.7.3.61.1	Test Purpose and Environment	1565

A.7.3.61.2	Test Requirements	1569

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1569

A.7.3.62.1	Test Purpose and Environment	1569

A.7.3.62.2	Test Requirements	1573

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1573

A.7.3.63.1	Test Purpose and Environment	1573

A.7.3.63.2	Test Requirements	1577

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1577

A.7.3.64.1	Test Purpose and Environment	1577

A.7.3.64.2	Test Requirements	1580

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1581

A.7.3.65.1	Test Purpose and Environment	1581

A.7.3.65.2	Test Requirements	1584

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1585

A.7.3.66.1	Test Purpose and Environment	1585

A.7.3.66.2	Test Requirements	1587

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1588

A.7.3.67.1	Test Purpose and Environment	1588

A.7.3.67.2	Test Requirements	1591

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1592

A.7.3.68.1	Test Purpose and Environment	1592

A.7.3.68.2	Test Requirements	1594

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1594

A.7.3.69.1	Test Purpose and Environment	1594

A.7.3.69.2	Test Requirements	1596

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1596

A.7.3.70.1	Test Purpose and Environment	1596

A.7.3.70.2	Test Requirements	1598

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1598

A.7.3.71.1	Test Purpose and Environment	1598

A.7.3.71.2	Test Requirements	1601

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1601

A.7.3.72.1	Test Purpose and Environment	1601

A.7.3.72.2	Test Requirements	1603

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1603

A.7.3.73.1	Test Purpose and Environment	1603

A.7.3.73.2	Test Requirements	1605

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1605

A.7.3.74.1	Test Purpose and Environment	1605

A.7.3.74.2	Test Requirements	1608

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1608

A.7.3.75.1	Test Purpose and Environment	1608

A.7.3.75.2	Test Requirements	1610

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1611

A.7.3.76.1	Test Purpose and Environment	1611

A.7.3.76.2	Test Requirements	1614

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1614

A.7.3.77.1	Test Purpose and Environment	1614

A.7.3.77.2	Test Requirements	1617

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1617

A.7.3.78.1	Test Purpose and Environment	1617

A.7.3.78.2	Test Requirements	1620

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1620

A.7.3.79.1	Test Purpose and Environment	1620

A.7.3.79.2	Test Requirements	1623

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1623

A.7.3.80.1	Test Purpose and Environment	1623

A.7.3.80.2	Test Requirements	1626

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1626

A.7.3.81.1	Test Purpose and Environment	1626

A.7.3.81.2	Test Requirements	1629

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1629

A.7.3.82.1	Test Purpose and Environment	1629

A.7.3.82.2	Test Requirements	1632

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1632

A.7.3.83.1	Test Purpose and Environment	1632

A.7.3.83.2	Test Requirements	1633

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1634

A.7.3.84.1	Test Purpose and Environment	1634

A.7.3.84.2	Test Requirements	1636

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1636

A.7.3.85.1	Test Purpose and Environment	1636

A.7.3.85.2	Test Requirements	1637

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1638

A.7.3.86.1	Test Purpose and Environment	1638

A.7.3.86.2	Test Requirements	1640

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1640

A.7.3.87.1	Test Purpose and Environment	1640

A.7.3.87.2	Test Requirements	1642

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1643

A.7.3.88.1	Test Purpose and Environment	1643

A.7.3.88.2	Test Requirements	1647

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1647

A.7.3.89.1	Test Purpose and Environment	1647

A.7.3.89.2	Test Requirements	1651

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1651

A.7.3.90.1	Test Purpose and Environment	1651

A.7.3.90.2	Test Requirements	1655

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1655

A.7.3.91.1	Test Purpose and Environment	1655

A.7.3.91.2	Test Requirements	1659

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1659

A.7.3.92.1	Test Purpose and Environment	1659

A.7.3.92.2	Test Requirements	1662

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1663

A.7.3.93.1	Test Purpose and Environment	1663

A.7.3.93.2	Test Requirements	1666

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1667

A.7.3.94.1	Test Purpose and Environment	1667

A.7.3.94.2	Test Requirements	1669

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1670

A.7.3.95.1	Test Purpose and Environment	1670

A.7.3.95.2	Test Requirements	1673

A.7.4	Interruption for Dual Connectivity	1674

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1674

A.7.4.1.1	Test Purpose and Environment	1674

A.7.4.1.2	Test Requirements	1676

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1676

A.7.4.2.1	Test Purpose and Environment	1676

A.7.4.2.2	Test Requirements	1678

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1678

A.7.4.3.1	Test Purpose and Environment	1678

A.7.4.3.2	Test Requirements	1680

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1680

A.7.4.4.1	Test Purpose and Environment	1680

A.7.4.4.2	Test Requirements	1683

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1683

A.7.4.5.1	Test Purpose and Environment	1683

A.7.4.5.2	Test Requirements	1685

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1685

A.7.4.6.1	Test Purpose and Environment	1685

A.7.4.6.2	Test Requirements	1687

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1687

A.7.4.7.1	Test Purpose and Environment	1687

A.7.4.7.2	Test Requirements	1690

A.7.5	Proximity-based Services	1690

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1690

A.7.5.1.1	Test Purpose and Environment	1690

A.7.5.1.2	Test Requirements	1691

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1692

A.7.5.2.1	Test Purpose and Environment	1692

A.7.5.1.2	Test Requirements	1692

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1693

A.7.5.3.1	Test Purpose and Environment	1693

A.7.5.3.2	Test Requirements	1694

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1695

A.7.5.4.1	Test Purpose and Environment	1695

A.7.5.4.2	Test Requirements	1696

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1697

A.7.5.5.1	Test Purpose and Environment	1697

A.7.5.5.2	Test Requirements	1699

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1700

A.7.5.6.1	Test Purpose and Environment	1700

A.7.5.6.2	Test Requirements	1701

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1702

A.7.5.7.1	Test Purpose and Environment	1702

A.7.5.7.2	Test Requirements	1704

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1704

A.7.5.8.1	Test Purpose and Environment	1704

A.7.5.8.2	Test Requirements	1706

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1707

A.7.5.9.1	Test Purpose and Environment	1707

A.7.5.9.2	Test Requirements	1709

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1710

A.7.5.10.1	Test Purpose and Environment	1710

A.7.5.10.2	Test Requirements	1712

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1713

A.7.5.11.1	Test Purpose and Environment	1713

A.7.5.11.2	Test Requirements	1715

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1715

A.7.5.12.1	Test Purpose and Environment	1715

A.7.5.12.2	Test Requirements	1719

A.7.6	Interruption for carrier aggregation	1720

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1720

A.7.6.1.1	Test Purpose and Environment	1720

A.7.6.1.2	Test Requirements	1722

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1722

A.7.6.2.1	Test Purpose and Environment	1722

A.7.6.2.2	Test Requirements	1725

A.8	UE Measurements Procedures	1725

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1725

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1725

A.8.1.1.1	Test Purpose and Environment	1725

A.8.1.1.2	Test Requirements	1727

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1727

A.8.1.2.1	Test Purpose and Environment	1727

A.8.1.2.2	Test Requirements	1729

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1729

A.8.1.3.1	Test Purpose and Environment	1729

A.8.1.3.2	Test Requirements	1732

A.8.1.4	Void	1732

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1732

A.8.1.5.1	Test Purpose and Environment	1732

A.8.1.5.2	Test Requirements	1734

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1734

A.8.1.6.1	Test Purpose and Environment	1734

A.8.1.6.2	Test Requirements	1736

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1736

A.8.1.7.1	Test Purpose and Environment	1736

A.8.1.7.2	Test Requirements	1738

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1739

A.8.1.8.1	Test Purpose and Environment	1739

A.8.1.8.2	Test Requirements	1741

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1742

A.8.1.9.1	Test Purpose and Environment	1742

A.8.1.9.2	Test Requirements	1742

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1742

A.8.1.10.1	Test Purpose and Environment	1742

A.8.1.10.2	Test Requirements	1743

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1743

A.8.1.11.1	Test Purpose and Environment	1743

A.8.1.11.2	Test Requirements	1745

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1746

A.8.1.12.1	Test Purpose and Environment	1746

A.8.1.12.2	Test Requirements	1747

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1748

A.8.1.13.1	Test Purpose and Environment	1748

A.8.1.13.2	Test Requirements	1750

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1750

A.8.1.14.1	Test Purpose and Environment	1750

A.8.1.14.2	Test Requirements	1752

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1753

A.8.1.15.1	Test Purpose and Environment	1753

A.8.1.15.2	Test Requirements	1754

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1755

A.8.1.16.1	Test Purpose and Environment	1755

A.8.1.16.2	Test Requirements	1757

A.8.1.17	Void	1757

A.8.1.18	Void	1757

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1757

A.8.1.19.1	Test Purpose and Environment	1757

A.8.1.19.2	Test Requirements	1759

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1760

A.8.1.20.1	Test Purpose and Environment	1760

A.8.1.20.2	Test Requirements	1762

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1762

A.8.1.21.1	Test Purpose and Environment	1762

A.8.1.21.2	Test Requirements	1764

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1765

A.8.1.22.1	Test Purpose and Environment	1765

A.8.1.22.2	Test Requirements	1767

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1767

A.8.1.23.1	Test Purpose and Environment	1767

A.8.1.23.2	Test Requirements	1768

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1769

A.8.1.24.1	Test Purpose and Environment	1769

A.8.1.24.2	Test Requirements	1771

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1771

A.8.1.25.1	Test Purpose and Environment	1771

A.8.1.25.2	Test Requirements	1773

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1773

A.8.1.26.1	Test Purpose and Environment	1773

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1776

A.8.1.27.1	Test Purpose and Environment	1776

A.8.1.27.2	Test Requirements	1778

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1778

A.8.1.28.1	Test Purpose and Environment	1778

A.8.1.28.2	Test Requirements	1780

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1780

A.8.1.29.1	Test Purpose and Environment	1780

A.8.1.29.2	Test Requirements	1782

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1783

A.8.1.30.1	Test Purpose and Environment	1783

A.8.1.30.2	Test Requirements	1785

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1785

A.8.1.31.1	Test Purpose and Environment	1785

A.8.1.31.2	Test Requirements	1787

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1788

A.8.1.32.1	Test Purpose and Environment	1788

A.8.1.32.2	Test Requirements	1789

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1790

A.8.1.33.1	Test Purpose and Environment	1790

A.8.1.33.2	Test Requirements	1791

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1792

A.8.1.34.1	Test Purpose and Environment	1792

A.8.1.34.2	Test Requirements	1793

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1794

A.8.1.35.1	Test Purpose and Environment	1794

A.8.1.35.2	Test Requirements	1795

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1796

A.8.1.36.1	Test Purpose and Environment	1796

A.8.1.36.2	Test Requirements	1797

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1798

A.8.1.37.1	Test Purpose and Environment	1798

A.8.1.37.2	Test Requirements	1800

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1800

A.8.1.38.1	Test Purpose and Environment	1800

A.8.1.38.2	Test Requirements	1801

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1802

A.8.1.39.1	Test Purpose and Environment	1802

A.8.1.39.2	Test Requirements	1804

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1804

A.8.1.40.1	Test Purpose and Environment	1804

A.8.1.40.2	Test Requirements	1806

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1806

A.8.1.41.1	Test Purpose and Environment	1806

A.8.1.41.2	Test Requirement	1808

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1808

A.8.1.42.1	Test Purpose and Environment	1808

A.8.1.42.2	Test Requirement	1809

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1810

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1810

A.8.2.1.1	Test Purpose and Environment	1810

A.8.2.1.2	Test Requirements	1811

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1811

A.8.2.2.1	Test Purpose and Environment	1811

A.8.2.2.2	Test Requirements	1814

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1814

A.8.2.3.1	Test Purpose and Environment	1814

A.8.2.3.2	Test Requirements	1816

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1817

A.8.2.4.1	Test Purpose and Environment	1817

A.8.2.4.2	Test Requirements	1819

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1819

A.8.2.5.1	Test Purpose and Environment	1819

A.8.2.5.2	Test Requirements	1821

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1822

A.8.2.6.1	Test Purpose and Environment	1822

A.8.2.6.2	Test Requirements	1824

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1825

A.8.2.7.1	Test Purpose and Environment	1825

A.8.2.7.2	Test Requirements	1826

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1827

A.8.2.8.1	Test Purpose and Environment	1827

A.8.2.8.2	Test Requirements	1829

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1829

A.8.2.9.1	Test Purpose and Environment	1829

A.8.2.9.2	Test Requirements	1831

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1832

A.8.2.10.1	Test Purpose and Environment	1832

A.8.2.10.2	Test Requirements	1834

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1834

A.8.2.11.1	Test Purpose and Environment	1834

A.8.2.11.2	Test Requirements	1836

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1836

A.8.2.12.1	Test Purpose and Environment	1836

A.8.2.12.2	Test Requirements	1838

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1839

A.8.2.13.1	Test Purpose and Environment	1839

A.8.2.13.2	Test Requirements	1841

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1841

A.8.2.14.1	Test Purpose and Environment	1841

A.8.2.14.2	Test Requirement	1843

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1844

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1844

A.8.3.1.1	Test Purpose and Environment	1844

A.8.3.1.2	Test Requirements	1845

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1845

A.8.3.2.1	Test Purpose and Environment	1845

A.8.3.2.2	Test Requirements	1848

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1848

A.8.3.3.1	Test Purpose and Environment	1848

A.8.3.3.2	Test Requirements	1851

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1851

A.8.3.4.1	Test Purpose and Environment	1851

A.8.3.4.2	Test Requirements	1853

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1853

A.8.3.5.2	Test Requirements	1855

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1855

A.8.3.6.1	Test Purpose and Environment	1855

A.8.3.6.2	Test Requirements	1857

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1858

A.8.3.7.1	Test Purpose and Environment	1858

A.8.3.7.2	Test Requirements	1860

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1861

A.8.3.8.1	Test Purpose and Environment	1861

A.8.3.8.2	Test Requirements	1864

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1864

A.8.3.9.1	Test Purpose and Environment	1864

A.8.3.9.2	Test Requirements	1867

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1868

A.8.3.10.1	Test Purpose and Environment	1868

A.8.3.10.2	Test Requirements	1869

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1869

A.8.3.11.1	Test Purpose and Environment	1869

A.8.3.11.2	Test Requirement	1871

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1872

A.8.3.12.1	Test Purpose and Environment	1872

A.8.3.12.2	Test Requirement	1873

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1874

A.8.3.13.1	Test Purpose and Environment	1874

A.8.3.13.2	Test Requirement	1875

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1876

A.8.3.14.1	Test Purpose and Environment	1876

A.8.3.14.2	Test Requirement	1877

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1878

A.8.3.15.1	Test Purpose and Environment	1878

A.8.3.15.2	Test Requirement	1879

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1880

A.8.3.16.1	Test Purpose and Environment	1880

A.8.3.16.2	Test Requirement	1882

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1882

A.8.3.17.1	Test Purpose and Environment	1882

A.8.3.17.2	Test Requirement	1885

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1885

A.8.3.18.1	Test Purpose and Environment	1885

A.8.3.18.2	Test Requirement	1888

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1888

A.8.3.19.1	Test Purpose and Environment	1888

A.8.3.19.2	Test Requirement	1891

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1891

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1891

A.8.4.1.1	Test Purpose and Environment	1891

A.8.4.1.2	Test Requirements	1893

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1893

A.8.4.2.1	Test Purpose and Environment	1893

A.8.4.2.2	Test Requirements	1896

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1896

A.8.4.3.1	Test Purpose and Environment	1896

A.8.4.3.2	Test Requirements	1898

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1899

A.8.4.4.1	Test Purpose and Environment	1899

A.8.4.4.2	Test Requirements	1901

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1902

A.8.4.5.1	Test Purpose and Environment	1902

A.8.4.5.2	Test Requirements	1904

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1904

A.8.4.6.1	Test Purpose and Environment	1904

A.8.4.6.2	Test Requirements	1905

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1905

A.8.4.7.1	Test Purpose and Environment	1905

A.8.4.7.2	Test Requirements	1909

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1909

A.8.4.8.1	Test Purpose and Environment	1909

A.8.4.8.2	Test Requirements	1912

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1912

A.8.4.9.1	Test Purpose and Environment	1912

A.8.4.9.2	Test Requirements	1916

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1916

A.8.4.10.1	Test Purpose and Environment	1916

A.8.4.10.2	Test Requirements	1918

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1918

A.8.4.11.1	Test Purpose and Environment	1918

A.8.4.11.2	Test Requirement	1920

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1920

A.8.4.12.1	Test Purpose and Environment	1920

A.8.4.12.2	Test Requirement	1922

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1923

A.8.4.13.1	Test Purpose and Environment	1923

A.8.4.13.2	Test Requirement	1924

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1925

A.8.4.14.1	Test Purpose and Environment	1925

A.8.4.14.2	Test Requirement	1927

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1927

A.8.4.15.1	Test Purpose and Environment	1927

A.8.4.15.2	Test Requirement	1930

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1930

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1930

A.8.5.1.1	Test Purpose and Environment	1930

A.8.5.1.2	Test Requirements	1932

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1932

A.8.5.2.1	Test Purpose and Environment	1932

A.8.5.2.2	Test Requirements	1934

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1934

A.8.5.3.1	Test Purpose and Environment	1934

A.8.5.3.2	Test Requirements	1937

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1937

A.8.5.4.1	Test Purpose and Environment	1937

A.8.5.4.2	Test Requirements	1939

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1939

A.8.5.5.1	Test Purpose and Environment	1939

A.8.5.5.2	Test Requirements	1942

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1942

A.8.5.6.1	Test Purpose and Environment	1942

A.8.5.6.2	Test Requirements	1943

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1944

A.8.5.7.1	Test Purpose and Environment	1944

A.8.5.7.2	Test Requirements	1944

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1944

A.8.5.8.1	Test Purpose and Environment	1944

A.8.5.8.2	Test Requirements	1947

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1947

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1947

A.8.6.1.1	Test Purpose and Environment	1947

A.8.6.1.2	Test Requirements	1949

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1949

A.8.6.2.1	Test Purpose and Environment	1949

A.8.6.2.2	Test Requirements	1952

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1952

A.8.6.3.1	Test Purpose and Environment	1952

A.8.6.3.2	Test Requirements	1955

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1955

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1955

A.8.7.1.1	Test Purpose and Environment	1955

A.8.7.1.1.1	Void	1955

A.8.7.1.1.2	1.28 Mcps TDD option	1955

A.8.7.1.1.3	Void	1957

A.8.7.1.2	Test Requirements	1957

A.8.7.1.2.1	Void	1957

A.8.7.1.2.2	1.28 Mcps TDD option	1957

A.8.7.1.2.3	Void	1957

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1957

A.8.7.2.1	Test Purpose and Environment	1957

A.8.7.2.2	Test Requirements	1960

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1961

A.8.7.3.1	Test Purpose and Environment	1961

A.8.7.3.2	Test Parameters	1961

A.8.7.3.3	Test Requirements	1962

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1963

A.8.7.4.1	Test Purpose and Environment	1963

A.8.7.4.2	Test Requirements	1965

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1965

A.8.7.5.1	Test Purpose and Environment	1965

A.8.7.5.2	Test Requirements	1967

A.8.7A	TBD	1967

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1967

A.8.7A.1.1	Test Purpose and Environment	1967

A.8.7A.1.2	Test Requirements	1970

A.8.8	E-UTRAN FDD – GSM Measurements	1970

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1970

A.8.8.1.1	Test Purpose and Environment	1970

A.8.8.1.2	Test Requirements	1972

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1972

A.8.8.2.1	Test Purpose and Environment	1972

A.8.8.2.2	Test Requirements	1974

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1975

A.8.8.3.1	Test Purpose and Environment	1975

A.8.8.3.2	Test Requirements	1976

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1977

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1977

A.8.9.1.1	Test Purpose and Environment	1977

A.8.9.1.2	Test Requirements	1978

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1979

A.8.9.2.1	Test Purpose and Environment	1979

A.8.9.2.2	Test Requirements	1981

A.8.10	E-UTRAN TDD – GSM Measurements	1981

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1981

A.8.10.1.1	Test Purpose and Environment	1981

A.8.10.1.2	Test Requirements	1982

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1983

A.8.10.2.1	Test Purpose and Environment	1983

A.8.10.2.2	Test Requirements	1985

A.8.11	Monitoring of Multiple Layers	1985

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1985

A.8.11.1.1	Test Purpose and Environment	1985

A.8.11.1.2	Test Requirements	1987

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1988

A.8.11.2.1	Test Purpose and Environment	1988

A.8.11.2.2	Test Requirements	1989

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1990

A.8.11.3.1	Test Purpose and Environment	1990

A.8.11.3.2	Test Requirements	1992

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1992

A.8.11.4.1	Test Purpose and Environment	1992

A.8.11.4.2	Test Requirements	1995

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1995

A.8.11.5.1	Test Purpose and Environment	1995

A.8.11.5.2	Test Requirements	1997

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1998

A.8.11.6.1	Test Purpose and Environment	1998

A.8.11.6.2	Test Requirements	2000

A.8.12	RSTD Intra-frequency Measurements	2001

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2001

A.8.12.1.1	Test Purpose and Environment	2001

A.8.12.1.2	Test Requirements	2004

A.8.12.1.2A	Test Requirements for UE Category 1bis	2005

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2005

A.8.12.2.1	Test Purpose and Environment	2005

A.8.12.2.2	Test Requirements	2009

A.8.12.2.2A	Test Requirements for UE Category 1bis	2010

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2010

A.8.12.3.1	Test Purpose and Environment	2010

A.8.12.3.2	Test Requirements	2014

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2015

A.8.12.4.1	Test Purpose and Environment	2015

A.8.12.4.2	Test Requirements	2018

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2019

A.8.12.5.1	Test Purpose and Environment	2019

A.8.12.5.2	Test Requirements	2022

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2023

A.8.12.6.1	Test Purpose and Environment	2023

A.8.12.6.2	Test Requirements	2026

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2027

A.8.12.7.1	Test Purpose and Environment	2027

A.8.12.7.2	Test Requirements	2030

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2031

A.8.12.8.1	Test Purpose and Environment	2031

A.8.12.8.2	Test Requirements	2034

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2035

A.8.12.9.1	Test Purpose and Environment	2035

A.8.12.9.2	Test Requirements	2038

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2039

A.8.12.10.1	Test Purpose and Environment	2039

A.8.12.10.2	Test Requirements	2042

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2043

A.8.12.11.1	Test Purpose and Environment	2043

A.8.12.11.2	Test Requirements	2047

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2048

A.8.12.12.1	Test Purpose and Environment	2048

A.8.12.12.2	Test Requirements	2051

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2052

A.8.12.13.1	Test Purpose and Environment	2052

A.8.12.13.2	Test Requirements	2055

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2056

A.8.12.14.1	Test Purpose and Environment	2056

A.8.12.14.2	Test Requirements	2060

A.8.13	RSTD Inter-frequency Measurements	2061

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2061

A.8.13.1.1	Test Purpose and Environment	2061

A.8.13.1.2	Test Requirements	2064

A.8.13.1.2A	Test Requirements for UE Category 1bis	2065

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2065

A.8.13.2.1	Test Purpose and Environment	2065

A.8.13.2.2	Test Requirements	2069

A.8.13.2.2A	Test Requirements for UE Category 1bis	2070

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2070

A.8.13.3.1	Test Purpose and Environment	2070

A.8.13.3.2	Test Requirements	2074

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2075

A.8.13.4.1	Test Purpose and Environment	2075

A.8.13.4.2	Test Requirements	2078

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2079

A.8.13.5.1	Test Purpose and Environment	2079

A.8.13.5.2	Test Requirements	2083

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2084

A.8.13.6.1	Test Purpose and Environment	2084

A.8.13.6.2	Test Requirements	2087

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2088

A.8.13.7.1	Test Purpose and Environment	2088

A.8.13.7.2	Test Requirements	2091

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2092

A.8.13.8.1	Test Purpose and Environment	2092

A.8.13.8.2	Test Requirements	2096

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2097

A.8.13.9.1	Test Purpose and Environment	2097

A.8.13.9.2	Test Requirements	2101

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2102

A.8.13.10.1	Test Purpose and Environment	2102

A.8.13.10.2	Test Requirements	2106

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2107

A.8.13.11.1	Test Purpose and Environment	2107

A.8.13.11.2	Test Requirements	2111

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2112

A.8.13.12.1	Test Purpose and Environment	2112

A.8.13.12.2	Test Requirements	2116

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2117

A.8.13.13.1	Test Purpose and Environment	2117

A.8.13.13.2	Test Requirements	2121

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2122

A.8.13.14.1	Test Purpose and Environment	2122

A.8.13.14.2	Test Requirements	2126

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2127

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2127

A.8.14.1.1	Test Purpose and Environment	2127

A.8.14.1.2	Test Requirements	2129

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2129

A.8.14.2.1	Test Purpose and Environment	2129

A.8.14.2.2	Test Requirements	2132

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2132

A.8.14.3.1	Test Purpose and Environment	2132

A.8.14.3.2	Test Requirements	2134

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2135

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2135

A.8.15.1.1	Test Purpose and Environment	2135

A.8.15.1.2	Test Requirements	2136

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2136

A.8.15.2.1	Test Purpose and Environment	2136

A.8.15.2.2	Test Requirements	2139

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2139

A.8.15.3.1	Test Purpose and Environment	2139

A.8.15.3.2	Test Requirements	2141

A.8.16	E-UTRAN Carrier Aggregation Measurements	2142

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2142

A.8.16.1.1	Test Purpose and Environment	2142

A.8.16.1.2	Test Requirements	2144

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2144

A.8.16.2.1	Test Purpose and Environment	2144

A.8.16.2.2	Test Requirements	2146

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2146

A.8.16.3.1	Test Purpose and Environment	2146

A.8.16.3.2	Test Requirements	2148

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2149

A.8.16.3A.1	Test Purpose and Environment	2149

A.8.16.3A.2	Test Requirements	2150

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2151

A.8.16.4.1	Test Purpose and Environment	2151

A.8.16.4.2	Test Requirements	2152

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2153

A.8.16.4A.1	Test Purpose and Environment	2153

A.8.16.4A.2	Test Requirements	2155

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2156

A.8.16.5.1	Test Purpose and Environment	2156

A.8.16.5.2	Test Requirements	2156

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2156

A.8.16.6.1	Test Purpose and Environment	2156

A.8.16.6.2	Test Requirements	2157

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2157

A.8.16.7.1	 Test Purpose and Environment	2157

A.8.16.7.2	 Test Requirements	2158

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2158

A.8.16.8.1	 Test Purpose and Environment	2158

A.8.16.8.2	 Test Requirements	2159

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2159

A.8.16.9.1	Test Purpose and Environment	2159

A.8.16.9.2	Test Requirements	2160

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2160

A.8.16.10.1	Test Purpose and Environment	2160

A.8.16.10.2	Test Requirements	2161

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2161

A.8.16.11.1	 Test Purpose and Environment	2161

A.8.16.11.2	 Test Requirements	2162

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2162

A.8.16.12.1	 Test Purpose and Environment	2162

A.8.16.12.2	 Test Requirements	2162

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2162

A.8.16.13.1	Test Purpose and Environment	2162

A.8.16.13.2	Test Requirements	2163

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2163

A.8.16.14.1	Test Purpose and Environment	2163

A.8.16.14.2	Test Requirements	2164

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2164

A.8.16.15.1	 Test Purpose and Environment	2164

A.8.16.7.2	 Test Requirements	2164

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2164

A.8.16.16.1	 Test Purpose and Environment	2164

A.8.16.16.2	 Test Requirements	2165

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2165

A.8.16.17.1	Test Purpose and Environment	2165

A.8.16.17.2	Test Requirements	2167

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2168

A.8.16.17A.1	Test Purpose and Environment	2168

A.8.16.17A.2	Test Requirements	2168

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2168

A.8.16.17B.1	Test Purpose and Environment	2168

A.8.16.17B.2	Test Requirements	2169

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2169

A.8.16.17C.1	Test Purpose and Environment	2169

A.8.16.17C.2	Test Requirements	2169

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2169

A.8.16.18.1	Test Purpose and Environment	2169

A.8.16.18.2	Test Requirements	2171

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2172

A.8.16.18A.1	Test Purpose and Environment	2172

A.8.16.18A.2	Test Requirements	2172

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2172

A.8.16.18B.1	Test Purpose and Environment	2172

A.8.16.18B.2	Test Requirements	2173

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2173

A.8.16.18C.1	Test Purpose and Environment	2173

A.8.16.18C.2	Test Requirements	2173

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2173

A.8.16.18D.1	Test Purpose and Environment	2173

A.8.16.18D.2	Test Requirements	2174

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2174

A.8.16.19.1	Test Purpose and Environment	2174

A.8.16.19.2	Test Requirements	2176

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2176

A.8.16.19A.1	Test Purpose and Environment	2176

A.8.16.19A.2	Test Requirements	2177

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2177

A.8.16.19B.1	Test Purpose and Environment	2177

A.8.16.19B.2	Test Requirements	2177

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2177

A.8.16.19C.1	Test Purpose and Environment	2177

A.8.16.19C.2	Test Requirements	2178

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2178

A.8.16.20.1	Test Purpose and Environment	2178

A.8.16.20.2	Test Requirements	2180

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2181

A.8.16.20A.1	Test Purpose and Environment	2181

A.8.16.20A.2	Test Requirements	2181

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2181

A.8.16.20B.1	Test Purpose and Environment	2181

A.8.16.20B.2	Test Requirements	2182

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2182

A.8.16.20C.1	Test Purpose and Environment	2182

A.8.16.20C.2	Test Requirements	2182

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2182

A.8.16.20D.1	Test Purpose and Environment	2182

A.8.16.20D.2	Test Requirements	2183

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2183

A.8.16.21.1	Test Purpose and Environment	2183

A.8.16.21.2	Test Requirements	2185

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2185

A.8.16.22.1	Test Purpose and Environment	2185

A.8.16.22.2	Test Requirements	2187

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2187

A.8.16.23.1	Test Purpose and Environment	2187

A.8.16.23.2	Test Requirements	2190

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2190

A.8.16.24.1	Test Purpose and Environment	2190

A.8.16.24.2	Test Requirements	2193

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2193

A.8.16.25.1	Test Purpose and Environment	2193

A.8.16.25.2	Test Requirements	2195

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2195

A.8.16.26.1	Test Purpose and Environment	2195

A.8.16.26.2	Test Requirements	2198

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2198

A.8.16.27.1	Test Purpose and Environment	2198

A.8.16.27.2	Test Requirements	2202

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2202

A.8.16.28.1	Test Purpose and Environment	2202

A.8.16.28.2	Test Requirements	2206

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2206

A.8.16.29.1	Test Purpose and Environment	2206

A.8.16.29.2	Test Requirements	2210

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2210

A.8.16.30.1	Test Purpose and Environment	2210

A.8.16.30.2	Test Requirements	2214

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2214

A.8.16.31.1	Test Purpose and Environment	2214

A.8.16.31.2	Test Requirements	2218

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2218

A.8.16.32.1	Test Purpose and Environment	2218

A.8.16.32.2	Test Requirements	2222

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2222

A.8.16.33.1	Test Purpose and Environment	2222

A.8.16.33.2	Test Requirements	2226

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2226

A.8.16.34.1	Test Purpose and Environment	2226

A.8.16.34.2	Test Requirements	2230

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2230

A.8.16.35.1	Test Purpose and Environment	2230

A.8.16.35.2	Test Requirements	2232

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2233

A.8.16.36.1	Test Purpose and Environment	2233

A.8.16.36.2	Test Requirements	2235

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2236

A.8.16.37.1	Test Purpose and Environment	2236

A.8.16.37.2	Test Requirements	2238

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2239

A.8.16.38.1	Test Purpose and Environment	2239

A.8.16.38.2	Test Requirements	2241

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2242

A.8.16.39.1	Test Purpose and Environment	2242

A.8.16.39.2	Test Requirements	2245

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2245

A.8.16.40.1	Test Purpose and Environment	2245

A.8.16.40.2	Test Requirements	2248

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2248

A.8.16.41.1	Test Purpose and Environment	2248

A.8.16.41.2	Test Requirements	2250

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2251

A.8.16.42.1	Test Purpose and Environment	2251

A.8.16.42.2	Test Requirements	2253

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2254

A.8.16.43.1	Test Purpose and Environment	2254

A.8.16.43.2	Test Requirements	2257

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2257

A.8.16.44.1	Test Purpose and Environment	2257

A.8.16.44.2	Test Requirements	2260

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2260

A.8.16.45.1	Test Purpose and Environment	2260

A.8.16.45.2	Test Requirements	2263

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2263

A.8.16.46.1	Test Purpose and Environment	2263

A.8.16.46.2	Test Requirements	2265

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2266

A.8.16.47.1	Test Purpose and Environment	2266

A.8.16.47.2	Test Requirements	2268

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2269

A.8.16.48.1	Test Purpose and Environment	2269

A.8.16.48.2	Test Requirements	2271

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2272

A.8.16.49.1	Test Purpose and Environment	2272

A.8.16.49.2	Test Requirements	2275

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2275

A.8.16.50.1	Test Purpose and Environment	2275

A.8.16.50.2	Test Requirements	2277

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2278

A.8.16.51.1	Test Purpose and Environment	2278

A.8.16.51.2	Test Requirements	2282

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2282

A.8.16.52.1	Test Purpose and Environment	2282

A.8.16.52.2	Test Requirements	2286

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2286

A.8.16.53.1	Test Purpose and Environment	2286

A.8.16.53.2	Test Requirements	2290

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2290

A.8.16.54.1	Test Purpose and Environment	2290

A.8.16.54.2	Test Requirements	2294

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2294

A.8.16.55.1	Test Purpose and Environment	2294

A.8.16.55.2	Test Requirements	2299

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2299

A.8.16.56.1	Test Purpose and Environment	2299

A.8.16.56.2	Test Requirements	2304

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2304

A.8.16.57.1	Test Purpose and Environment	2304

A.8.16.57.2	Test Requirements	2306

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2307

A.8.16.58.1	Test Purpose and Environment	2307

A.8.16.58.2	Test Requirements	2309

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2310

A.8.16.59.1	Test Purpose and Environment	2310

A.8.16.59.2	Test Requirements	2313

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2314

A.8.16.60.1	Test Purpose and Environment	2314

A.8.16.60.2	Test Requirements	2317

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2318

A.8.16.61.1	Test Purpose and Environment	2318

A.8.16.61.2	Test Requirements	2321

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2321

A.8.16.62.1	Test Purpose and Environment	2321

A.8.16.62.2	Test Requirements	2323

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2324

A.8.16.63.1	Test Purpose and Environment	2324

A.8.16.63.2	Test Requirements	2327

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2328

A.8.16.64.1	Test Purpose and Environment	2328

A.8.16.64.2	Test Requirements	2332

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2332

A.8.16.65.1	Test Purpose and Environment	2332

A.8.16.65.2	Test Requirements	2337

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2337

A.8.16.66.1	Test Purpose and Environment	2337

A.8.16.66.2	Test Requirements	2342

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2342

A.8.16.67.1	Test Purpose and Environment	2342

A.8.16.67.2	Test Requirements	2347

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2347

A.8.16.68.1	Test Purpose and Environment	2347

A.8.16.68.2	Test Requirements	2352

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2352

A.8.16.69.1	Test Purpose and Environment	2352

A.8.16.69.2	Test Requirements	2355

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2355

A.8.16.70.1	Test Purpose and Environment	2355

A.8.16.70.2	Test Requirements	2358

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2358

A.8.16.71.1	Test Purpose and Environment	2358

A.8.16.71.2	Test Requirements	2362

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2362

A.8.16.72.1	Test Purpose and Environment	2362

A.8.16.72.2	Test Requirements	2366

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2366

A.8.16.73.1	Test Purpose and Environment	2366

A.8.16.73.2	Test Requirements	2371

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2371

A.8.16.74.1	Test Purpose and Environment	2371

A.8.16.74.2	Test Requirements	2376

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2376

A.8.16.75.1	Test Purpose and Environment	2376

A.8.16.75.2	Test Requirements	2381

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2381

A.8.16.76.1	Test Purpose and Environment	2381

A.8.16.76.2	Test Requirements	2386

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2386

A.8.16.77.1	Test Purpose and Environment	2386

A.8.16.77.2	Test Requirements	2389

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2389

A.8.16.78.1	Test Purpose and Environment	2389

A.8.16.78.2	Test Requirements	2392

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2393

A.8.16.79.1	Test Purpose and Environment	2393

A.8.16.79.2	Test Requirements	2398

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2398

A.8.16.80.1	Test Purpose and Environment	2398

A.8.16.80.2	Test Requirements	2403

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2403

A.8.16.81.1	Test Purpose and Environment	2403

A.8.16.81.2	Test Requirements	2408

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2408

A.8.16.82.1	Test Purpose and Environment	2408

A.8.16.82.2	Test Requirements	2413

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2413

A.8.16.83.1	Test Purpose and Environment	2413

A.8.16.83.2	Test Requirements	2418

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2418

A.8.16.84.1	Test Purpose and Environment	2418

A.8.16.84.2	Test Requirements	2424

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2424

A.8.16.85.1	Test Purpose and Environment	2424

A.8.16.85.2	Test Requirements	2427

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2427

A.8.16.86.1	Test Purpose and Environment	2427

A.8.16.86.2	Test Requirements	2430

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2430

A.8.16.87.1	Test Purpose and Environment	2430

A.8.16.87.2	Test Requirements	2435

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2435

A.8.16.88.1	Test Purpose and Environment	2435

A.8.16.88.2	Test Requirements	2443

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2443

A.8.16.89.1	Test Purpose and Environment	2443

A.8.16.89.2	Test Requirements	2448

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2448

A.8.16.90.1	Test Purpose and Environment	2448

A.8.16.90.2	Test Requirements	2453

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2453

A.8.16.91.1	Test Purpose and Environment	2453

A.8.16.91.2	Test Requirements	2459

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2459

A.8.16.92.1	Test Purpose and Environment	2459

A.8.16.92.2	Test Requirements	2467

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2467

A.8.16.93.1	Test Purpose and Environment	2467

A.8.16.93.2	Test Requirements	2472

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2472

A.8.16.94.1	Test Purpose and Environment	2472

A.8.16.94.2	Test Requirements	2477

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2477

A.8.16.95.1	Test Purpose and Environment	2477

A.8.16.95.2	Test Requirements	2485

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2485

A.8.16.96.1	Test Purpose and Environment	2485

A.8.16.96.2	Test Requirements	2492

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2492

A.8.16.97.1	Test Purpose and Environment	2492

A.8.16.97.2	Test Requirements	2497

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2497

A.8.16.98.1	Test Purpose and Environment	2497

A.8.16.98.2	Test Requirements	2503

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2503

A.8.16.99.1	Test Purpose and Environment	2503

A.8.16.99.2	Test Requirements	2510

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2510

A.8.16.100.1	Test Purpose and Environment	2510

A.8.16.100.2	Test Requirements	2518

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2518

A.8.16.101.1	Test Purpose and Environment	2518

A.8.16.101.2	Test Requirements	2525

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2525

A.8.16.102.1	Test Purpose and Environment	2525

A.8.16.102.2	Test Requirements	2533

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2533

A.8.16.103.1	Test Purpose and Environment	2533

A.8.16.103.2	Test Requirements	2537

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2538

A.8.16.104.1	Test Purpose and Environment	2538

A.8.16.104.2	Test Requirements	2541

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2542

A.8.16.105.1	Test Purpose and Environment	2542

A.8.16.105.2	Test Requirements	2546

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2547

A.8.16.106.1	Test Purpose and Environment	2547

A.8.16.106.2	Test Requirements	2550

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2550

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2550

A.8.17.1.1	Test Purpose and Environment	2550

A.8.17.1.2	Test Requirements	2556

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2556

A.8.17.2.1	Test Purpose and Environment	2556

A.8.17.2.2	Test Requirements	2562

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2562

A.8.17.3.1	Test Purpose and Environment	2562

A.8.17.3.2	Test Requirements	2563

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2563

A.8.17.4.1	Test Purpose and Environment	2563

A.8.17.4.2	Test Requirements	2564

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2564

A.8.17.5.1	Test Purpose and Environment	2564

A.8.17.5.2	Test Requirements	2565

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2565

A.8.17.6.1	Test Purpose and Environment	2565

A.8.17.6.2	Test Requirements	2566

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2567

A.8.17.7.1	Test Purpose and Environment	2567

A.8.17.7.2	Test Requirements	2567

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2568

A.8.17.8.1	Test Purpose and Environment	2568

A.8.17.8.2	Test Requirements	2568

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2569

A.8.17.9.1	Test Purpose and Environment	2569

A.8.17.9.2	Test Requirements	2570

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2570

A.8.17.10.1	Test Purpose and Environment	2570

A.8.17.10.2	Test Requirements	2575

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2576

A.8.17.11.1	Test Purpose and Environment	2576

A.8.17.11.2	Test Requirements	2582

A.8.18	E-UTRAN TDD – HRPD Measurements	2583

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2583

A.8.18.1.1	Test Purpose and Environment	2583

A.8.18.1.2	Test Requirements	2585

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2585

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2585

A.8.19.1.1	Test Purpose and Environment	2585

A.8.19.1.2	Test Requirements	2586

A.8.20	Inter-frequency/RAT Measurements in CA mode	2587

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2587

A.8.20.1.1	Test Purpose and Environment	2587

A.8.20.1.2	Test Requirements	2588

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2588

A.8.20.2.1	Test Purpose and Environment	2589

A.8.20.2.2	Test Requirements	2590

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2591

A.8.20.2A.1	Test Purpose and Environment	2591

A.8.20.2A.2	Test Requirements	2591

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2591

A.8.20.2B.1	Test Purpose and Environment	2591

A.8.20.2B.2	Test Requirements	2594

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2594

A.8.20.3.1	Test Purpose and Environment	2594

A.8.20.3.2	Test Requirements	2596

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2596

A.8.20.4.1	Test Purpose and Environment	2596

A.8.20.4.1.1	1.28 Mcps TDD option	2596

A.8.20.4.2	Test Requirements	2598

A.8.20.4.2.1	1.28 Mcps TDD option	2598

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2598

A.8.20.4A.1	Test Purpose and Environment	2598

A.8.20.4A.1.1	1.28 Mcps TDD option	2598

A.8.20.4A.2	Test Requirements	2598

A.8.20.4A.2.1	1.28 Mcps TDD option	2598

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2599

A.8.20.4B.1	Test Purpose and Environment	2599

A.8.20.4B.1.1	1.28 Mcps TDD option	2599

A.8.20.4B.2	Test Requirements	2601

A.8.20.4B.2.1	1.28 Mcps TDD option	2601

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2601

A.8.21.1	Test Purpose and Environment	2601

A.8.21.2 Test Requirements	2604

A.8.22	E-UTRAN Discovery Signal Measurements	2604

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2604

A.8.22.1.1	Test Purpose and Environment	2604

A.8.22.1.2	Test Requirements	2607

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2607

A.8.22.2.1	Test Purpose and Environment	2607

A.8.22.2.2	Test Requirements	2610

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2610

A.8.22.3.1	Test Purpose and Environment	2610

A.8.22.3.2	Test Requirements	2613

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2613

A.8.22.4.1	Test Purpose and Environment	2613

A.8.22.4.2	Test Requirements	2616

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2616

A.8.22.5.1	Test Purpose and Environment	2616

A.8.22.5.2	Test Requirements	2619

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2619

A.8.22.6.1	Test Purpose and Environment	2619

A.8.22.6.2	Test Requirements	2622

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2622

A.8.22.7.1	Test Purpose and Environment	2622

A.8.22.7.2	Test Requirements	2625

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2625

A.8.22.8.1	Test Purpose and Environment	2625

A.8.22.8.2	Test Requirements	2628

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2628

A.8.22.9.1	Test Purpose and Environment	2628

A.8.22.9.2	Test Requirements	2630

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2631

A.8.22.10.1	Test Purpose and Environment	2631

A.8.22.10.2	Test Requirements	2633

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2634

A.8.22.11.1	Test Purpose and Environment	2634

A.8.22.11.2	Test Requirements	2636

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2636

A.8.22.12.1	Test Purpose and Environment	2636

A.8.22.12.2	Test Requirements	2639

A.8.23	E-UTRAN Dual Connectivity Measurements	2639

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2639

A.8.23.1.1	Test Purpose and Environment	2639

A.8.23.1.2	Test Requirements	2642

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2642

A.8.23.2.1	Test Purpose and Environment	2642

A.8.23.2.2	Test Requirements	2645

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2645

A.8.23.3.1	Test Purpose and Environment	2645

A.8.23.3.2	Test Requirements	2648

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2648

A.8.23.4.1	Test Purpose and Environment	2648

A.8.23.4.2	Test Requirements	2651

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2651

A.8.23.5.1	Test Purpose and Environment	2651

A.8.23.5.2	Test Requirements	2654

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2654

A.8.23.6.1	Test Purpose and Environment	2654

A.8.23.6.2	Test Requirements	2657

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2657

A.8.23.7.1	Test Purpose and Environment	2657

A.8.23.7.2	Test Requirements	2659

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2660

A.8.23.8.1	Test Purpose and Environment	2660

A.8.23.8.2	Test Requirements	2662

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2663

A.8.23.9.1	Test Purpose and Environment	2663

A.8.23.9.2	Test Requirements	2666

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2666

A.8.23.10.1	Test Purpose and Environment	2666

A.8.23.10.2	Test Requirements	2669

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2669

A.8.23.11.1	Test Purpose and Environment	2669

A.8.23.11.2	Test Requirements	2672

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2672

A.8.23.12.1	Test Purpose and Environment	2672

A.8.23.12.2	Test Requirements	2675

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2675

A.8.23.13.1	Test Purpose and Environment	2675

A.8.23.13.2	Test Requirements	2678

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2678

A.8.23.14.1	Test Purpose and Environment	2678

A.8.23.14.2	Test Requirements	2681

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2681

A.8.23.15.1	Test Purpose and Environment	2681

A.8.23.15.2	Test Requirements	2684

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2684

A.8.23.16.1	Test Purpose and Environment	2684

A.8.23.16.2	Test Requirements	2685

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2686

A.8.23.17.1	Test Purpose and Environment	2686

A.8.23.17.2	Test Requirements	2688

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2688

A.8.23.18.1	Test Purpose and Environment	2688

A.8.23.18.2	Test Requirements	2689

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2690

A.8.23.19.1	Test Purpose and Environment	2690

A.8.23.19.2	Test Requirements	2691

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2692

A.8.23.20.1	Test Purpose and Environment	2692

A.8.23.20.2	Test Requirements	2693

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2694

A.8.23.21.1	Test Purpose and Environment	2694

A.8.23.21.2	Test Requirements	2695

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2696

A.8.23.22.1	Test Purpose and Environment	2696

A.8.23.22.2	Test Requirements	2697

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2698

A.8.23.23.1	Test Purpose and Environment	2698

A.8.23.23.2	Test Requirements	2699

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2700

A.8.23.24.1	Test Purpose and Environment	2700

A.8.23.24.2	Test Requirements	2702

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2703

A.8.23.25.1	Test Purpose and Environment	2703

A.8.23.25.2	Test Requirements	2705

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2706

A.8.23.26.1	Test Purpose and Environment	2706

A.8.23.26.2	Test Requirements	2708

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2709

A.8.23.27.1	Test Purpose and Environment	2709

A.8.23.27.2	Test Requirements	2712

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2712

A.8.23.28.1	Test Purpose and Environment	2712

A.8.23.28.2	Test Requirements	2715

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2715

A.8.23.29.1	Test Purpose and Environment	2715

A.8.23.29.2	Test Requirements	2718

A.8.24	Proximity-based Services	2718

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2718

A.8.24.1.1	Test Purpose and Environment	2718

A.8.24.1.2	Test Requirements	2719

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2720

A.8.24.2.1	Test Purpose and Environment	2720

A.8.24.2.2	Test Requirements	2721

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2721

A.8.24.3.1	Test Purpose and Environment	2722

A.8.24.3.2	Test Requirements	2723

A.8.25	E-UTRAN-WLAN Measurements	2724

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2724

A.8.25.1.1	Test Purpose and Environment	2724

A.8.25.1.2	Test Requirements	2726

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2726

A.8.25.2.1	Test Purpose and Environment	2726

A.8.25.2.2	Test Requirements	2728

A.8.26 Frame Structure 3 (FS3)	2728

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2728

A.8.26.1.1	Test Purpose and Environment	2728

A.8.26.1.2	Test Requirements	2730

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2731

A.8.26.2.1	Test Purpose and Environment	2731

A.8.26.2.2	Test Requirements	2733

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2734

A.8.26.3.1	Test Purpose and Environment	2734

A.8.26.3.2	Test Requirements	2737

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2737

A.8.26.3A.1	Test Purpose and Environment	2737

A.8.26.3A.2	Test Requirements	2740

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2740

A.8.26.4.1	Test Purpose and Environment	2740

A.8.26.4.2	Test Requirements	2743

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2743

A.8.26.4A.1	Test Purpose and Environment	2743

A.8.26.4A.2	Test Requirements	2746

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2746

A.8.26.5.1	Test Purpose and Environment	2746

A.8.26.5.2	Test Requirements	2749

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2749

A.8.26.5A.1	Test Purpose and Environment	2749

A.8.26.5A.2	Test Requirements	2752

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2752

A.8.26.6.1	Test Purpose and Environment	2752

A.8.26.6.2	Test Requirements	2755

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2755

A.8.26.6A.1	Test Purpose and Environment	2755

A.8.26.6A.2	Test Requirements	2758

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2758

A.8.26.7.1	Test Purpose and Environment	2758

A.8.26.7.2	Test Requirements	2761

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2761

A.8.26.8.1	Test Purpose and Environment	2761

A.8.26.8.2	Test Requirements	2764

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2764

A.8.26.9.1	Test Purpose and Environment	2764

A.8.26.9.2	Test Requirements	2767

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2767

A.8.26.10.1	Test Purpose and Environment	2767

A.8.26.10.2	Test Requirements	2769

A.9	Measurement Performance Requirements	2771

A.9.1	RSRP	2771

A.9.1.1	FDD Intra frequency case	2771

A.9.1.1.1	Test Purpose and Environment	2771

A.9.1.1.2	Test parameters	2771

A.9.1.1.3	Test Requirements	2774

A.9.1.2	TDD Intra frequency case	2774

A.9.1.2.1	Test Purpose and Environment	2774

A.9.1.2.2	Test parameters	2774

A.9.1.2.3	Test Requirements	2775

A.9.1.3	FDD—FDD Inter frequency case	2776

A.9.1.3.1	Test Purpose and Environment	2776

A.9.1.3.2	Test parameters	2776

A.9.1.3.3	Test Requirements	2779

A.9.1.4	TDD—TDD Inter frequency case	2779

A.9.1.4.1	Test Purpose and Environment	2779

A.9.1.4.2	Test parameters	2779

A.9.1.4.3	Test Requirements	2781

A.9.1.5	FDD—TDD Inter frequency case	2782

A.9.1.5.1	Test Purpose and Environment	2782

A.9.1.5.2	Test parameters	2782

A.9.1.5.3	Test Requirements	2783

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2784

A.9.1.6.1	Test Purpose and Environment	2784

A.9.1.6.2	Test parameters	2784

A.9.1.6.3	Test Requirements	2787

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2787

A.9.1.7.1	Test Purpose and Environment	2787

A.9.1.7.2	Test parameters	2787

A.9.1.7.3	Test Requirements	2790

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2790

A.9.1.8.1	Test Purpose and Environment	2790

A.9.1.8.2	Test parameters	2790

A.9.1.8.3	Test Requirements	2794

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2794

A.9.1.9.1	Test Purpose and Environment	2794

A.9.1.9.2	Test parameters	2794

A.9.1.9.3	Test Requirements	2796

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2797

A.9.1.10.1	Test Purpose and Environment	2797

A.9.1.10.2	Test parameters	2797

A.9.1.10.3	Test Requirements	2800

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2800

A.9.1.11.1	Test Purpose and Environment	2800

A.9.1.11.2	Test parameters	2800

A.9.1.11.3	Test Requirements	2804

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2804

A.9.1.12.1	Test Purpose and Environment	2804

A.9.1.12.2	Test parameters	2804

A.9.1.12.3	Test Requirements	2805

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2805

A.9.1.13.1	Test Purpose and Environment	2805

A.9.1.13.2	Test parameters	2805

A.9.1.13.3	Test Requirements	2806

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2806

A.9.1.14.1	Test Purpose and Environment	2806

A.9.1.14.2	Test parameters	2806

A.9.1.14.3	Test Requirements	2810

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2810

A.9.1.15.1	Test Purpose and Environment	2810

A.9.1.15.2	Test parameters	2810

A.9.1.15.3	Test Requirements	2812

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2813

A.9.1.16.1	Test Purpose and Environment	2813

A.9.1.16.2	Test parameters	2813

A.9.1.16.3	Test Requirements	2814

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2814

A.9.1.17.1	Test Purpose and Environment	2814

A.9.1.17.2	Test parameters	2815

A.9.1.17.3	Test Requirements	2816

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2816

A.9.1.18.1	Test Purpose and Environment	2816

A.9.1.18.2	Test parameters	2816

A.9.1.18.3	Test Requirements	2817

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2818

A.9.1.19.1	Test Purpose and Environment	2818

A.9.1.19.2	Test parameters	2818

A.9.1.19.3	Test Requirements	2818

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2818

A.9.1.20.1	Test Purpose and Environment	2818

A.9.1.20.2	Test parameters	2818

A.9.1.20.3	Test Requirements	2820

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2820

A.9.1.21.1	Test Purpose and Environment	2820

A.9.1.21.2	Test parameters	2820

A.9.1.21.3	Test Requirements	2820

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2820

A.9.1.22.1	Test Purpose and Environment	2820

A.9.1.22.2	Test parameters	2821

A.9.1.22.3	Test Requirements	2825

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2825

A.9.1.23.1	Test Purpose and Environment	2825

A.9.1.23.2	Test parameters	2825

A.9.1.23.3	Test Requirements	2829

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2829

A.9.1.24.1	Test Purpose and Environment	2829

A.9.1.24.2	Test parameters	2829

A.9.1.24.3	Test Requirements	2830

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2830

A.9.1.25.1	Test Purpose and Environment	2830

A.9.1.25.2	Test parameters	2830

A.9.1.25.3	Test Requirements	2833

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2833

A.9.1.26.1	Test Purpose and Environment	2833

A.9.1.26.2	Test parameters	2833

A.9.1.26.3	Test Requirements	2836

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2836

A.9.1.27.1	Test Purpose and Environment	2836

A.9.1.27.2	Test parameters	2836

A.9.1.27.3	Test Requirements	2839

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2839

A.9.1.28.1	Test Purpose and Environment	2839

A.9.1.28.2	Test parameters	2839

A.9.1.28.3	Test Requirements	2842

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2842

A.9.1.29.1	Test Purpose and Environment	2842

A.9.1.29.2	Test parameters	2842

A.9.1.29.3	Test Requirements	2845

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2845

A.9.1.30.1	Test Purpose and Environment	2845

A.9.1.30.2	Test parameters	2846

A.9.1.30.3	Test Requirements	2849

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2849

A.9.1.31.1	Test Purpose and Environment	2849

A.9.1.31.2	Test parameters	2849

A.9.1.31.3	Test Requirements	2852

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2853

A.9.1.32.1	Test Purpose and Environment	2853

A.9.1.32.2	Test parameters	2853

A.9.1.32.3	Test Requirements	2856

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2856

A.9.1.33.1	Test Purpose and Environment	2856

A.9.1.33.2	Test parameters	2856

A.9.1.33.3	Test Requirements	2859

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2859

A.9.1.34.1	Test Purpose and Environment	2859

A.9.1.34.2	Test parameters	2859

A.9.1.34.3	Test Requirements	2862

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2862

A.9.1.35.1	Test Purpose and Environment	2862

A.9.1.35.2	Test parameters	2862

A.9.1.35.3	Test Requirements	2865

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2866

A.9.1.36.1	Test Purpose and Environment	2866

A.9.1.36.2	Test parameters	2866

A.9.1.36.3	Test Requirements	2869

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2869

A.9.1.37.1	Test Purpose and Environment	2869

A.9.1.37.2	Test parameters	2869

A.9.1.37.3	Test Requirements	2872

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2873

A.9.1.38.1	Test Purpose and Environment	2873

A.9.1.38.2	Test parameters	2873

A.9.1.38.3	Test Requirements	2876

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2877

A.9.1.39.1	Test Purpose and Environment	2877

A.9.1.39.2	Test parameters	2877

A.9.1.39.3	Test Requirements	2883

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2883

A.9.1.40.1	Test Purpose and Environment	2883

A.9.1.40.2	Test parameters	2884

A.9.1.40.3	Test Requirements	2890

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2890

A.9.1.41.1	Test Purpose and Environment	2890

A.9.1.41.2	Test parameters	2890

A.9.1.41.3	Test Requirements	2893

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2893

A.9.1.42.1	Test Purpose and Environment	2893

A.9.1.42.2	Test parameters	2893

A.9.1.42.3	Test Requirements	2896

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2896

A.9.1.43.1	Test Purpose and Environment	2896

A.9.1.43.2	Test parameters	2896

A.9.1.43.3	Test Requirements	2897

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2898

A.9.1.44.1	Test Purpose and Environment	2898

A.9.1.44.2	Test parameters	2898

A.9.1.44.3	Test Requirements	2903

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2903

A.9.1.45.1	Test Purpose and Environment	2903

A.9.1.45.2	Test parameters	2903

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2482

A.9.1.54.1	Test Purpose and Environment	2482

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.2	RSRQ	2582

A.9.2.1	FDD Intra frequency case	2582

A.9.2.1.1	Test Purpose and Environment	2582

A.9.2.1.2	Test parameters	2582

A.9.2.1.3	Test Requirements	2585

A.9.2.2	TDD Intra frequency case	2585

A.9.2.2.1	Test Purpose and Environment	2585

A.9.2.2.2	Test parameters	2585

A.9.2.2.3	Test Requirements	2587

A.9.2.3	FDD—FDD Inter frequency case	2587

A.9.2.3.1	Test Purpose and Environment	2587

A.9.2.3.2	Test parameters	2587

A.9.2.3.3	Test Requirements	2590

A.9.2.4	TDD—TDD Inter frequency case	2590

A.9.2.4.1	Test Purpose and Environment	2590

A.9.2.4.2	Test parameters	2590

A.9.2.4.3	Test Requirements	2592

A.9.2.4A	FDD—TDD Inter frequency case	2593

A.9.2.4A.1	Test Purpose and Environment	2593

A.9.2.4A.2	Test parameters	2593

A.9.2.4A.3	Test Requirements	2595

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2595

A.9.2.5.1	Test Purpose and Environment	2595

A.9.2.5.2	Test parameters	2596

A.9.2.5.3	Test Requirements	2599

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2599

A.9.2.6.1	Test Purpose and Environment	2599

A.9.2.6.2	Test parameters	2600

A.9.2.6.3	Test Requirements	2603

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2603

A.9.2.7.1	Test Purpose and Environment	2603

A.9.2.7.2	Test parameters	2603

A.9.2.7.3	Test Requirements	2607

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2607

A.9.2.8.1	Test Purpose and Environment	2607

A.9.2.8.2	Test parameters	2607

A.9.2.8.3	Test Requirements	2611

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2611

A.9.2.9.1	Test Purpose and Environment	2611

A.9.2.9.2	Test parameters	2611

A.9.2.9.3	Test Requirements	2615

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2615

A.9.2.10.1	Test Purpose and Environment	2615

A.9.2.10.2	Test parameters	2615

A.9.2.10.3	Test Requirements	2619

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2619

A.9.2.11.1	Test Purpose and Environment	2619

A.9.2.11.2	Test parameters	2619

A.9.2.11.3	Test Requirements	2620

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2620

A.9.2.12.1	Test Purpose and Environment	2620

A.9.2.12.2	Test parameters	2621

A.9.2.12.3	Test Requirements	2621

A.9.2.13	Void	2622

A.9.2.13.1	Void	2622

A.9.2.13.2	Void	2622

A.9.2.13.3	Void	2622

A.9.2.14	Void	2622

A.9.2.14.1	Void	2622

A.9.2.14.2	Void	2622

A.9.2.14.3	Void	2622

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2622

A.9.2.15.1	Test Purpose and Environment	2622

A.9.2.15.2	Test parameters	2622

A.9.2.15.3	Test Requirements	2626

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2626

A.9.2.16.1	Test Purpose and Environment	2626

A.9.2.16.2	Test parameters	2626

A.9.2.16.3	Test Requirements	2631

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2631

A.9.2.17.1	Test Purpose and Environment	2631

A.9.2.17.2	Test parameters	2631

A.9.2.17.3	Test Requirements	2633

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2633

A.9.2.18.1	Test Purpose and Environment	2633

A.9.2.18.2	Test parameters	2633

A.9.2.18.3	Test Requirements	2636

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2636

A.9.2.19.1	Test Purpose and Environment	2636

A.9.2.19.2	Test parameters	2636

A.9.2.19.3	Test Requirements	2638

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2638

A.9.2.20.1	Test Purpose and Environment	2638

A.9.2.20.2	Test parameters	2638

A.9.2.20.3	Test Requirements	2641

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2641

A.9.2.21.1	Test Purpose and Environment	2641

A.9.2.21.2	Test parameters	2641

A.9.2.21.3	Test Requirements	2644

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2644

A.9.2.22.1	Test Purpose and Environment	2645

A.9.2.22.2	Test parameters	2645

A.9.2.22.3	Test Requirements	2645

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2645

A.9.2.23.1	Test Purpose and Environment	2645

A.9.2.23.2	Test parameters	2645

A.9.2.23.3	Test Requirements	2648

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2648

A.9.2.24.1	Test Purpose and Environment	2648

A.9.2.24.2	Test parameters	2648

A.9.2.24.3	Test Requirements	2649

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2649

A.9.2.25.1	Test Purpose and Environment	2649

A.9.2.25.2	Test parameters	2649

A.9.2.25.3	Test Requirements	2654

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2654

A.9.2.26.1	Test Purpose and Environment	2654

A.9.2.26.2	Test parameters	2654

A.9.2.26.3	Test Requirements	2658

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2658

A.9.2.27.1	Test Purpose and Environment	2658

A.9.2.27.2	Test parameters	2658

A.9.2.27.3	Test Requirements	2659

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2659

A.9.2.28.1	Test Purpose and Environment	2659

A.9.2.28.2	Test parameters	2659

A.9.2.28.3	Test Requirements	2662

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2662

A.9.2.29.1	Test Purpose and Environment	2662

A.9.2.29.2	Test parameters	2662

A.9.2.29.3	Test Requirements	2665

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2665

A.9.2.30.1	Test Purpose and Environment	2665

A.9.2.30.2	Test parameters	2665

A.9.2.30.3	Test Requirements	2668

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2668

A.9.2.31.1	Test Purpose and Environment	2668

A.9.2.31.2	Test parameters	2668

A.9.2.31.3	Test Requirements	2671

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2671

A.9.2.32.1	Test Purpose and Environment	2671

A.9.2.32.2	Test parameters	2671

A.9.2.32.3	Test Requirements	2674

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2674

A.9.2.33.1	Test Purpose and Environment	2674

A.9.2.33.2	Test parameters	2674

A.9.2.33.3	Test Requirements	2678

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2678

A.9.2.34.1	Test Purpose and Environment	2678

A.9.2.34.2	Test parameters	2678

A.9.2.34.3	Test Requirements	2681

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2681

A.9.2.35.1	Test Purpose and Environment	2681

A.9.2.35.2	Test parameters	2681

A.9.2.35.3	Test Requirements	2684

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2684

A.9.2.36.1	Test Purpose and Environment	2684

A.9.2.36.2	Test parameters	2684

A.9.2.36.3	Test Requirements	2685

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2686

A.9.2.37.1	Test Purpose and Environment	2686

A.9.2.37.2	Test parameters	2686

A.9.2.37.3	Test Requirements	2687

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2688

A.9.2.38.1	Test Purpose and Environment	2688

A.9.2.38.2	Test parameters	2688

A.9.2.38.3	Test Requirements	2692

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2692

A.9.2.39.1	Test Purpose and Environment	2692

A.9.2.39.2	Test parameters	2692

A.9.2.39.3	Test Requirements	2696

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2696

A.9.2.40.1	Test Purpose and Environment	2696

A.9.2.40.2	Test parameters	2696

A.9.2.40.3	Test Requirements	2699

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2700

A.9.2.41.1	Test Purpose and Environment	2700

A.9.2.41.2	Test parameters	2700

A.9.2.41.3	Test Requirements	2703

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2703

A.9.2.42.1	Test Purpose and Environment	2703

A.9.2.42.2	Test parameters	2703

A.9.2.42.3	Test Requirements	2706

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2706

A.9.2.43.1	Test Purpose and Environment	2706

A.9.2.43.2	Test parameters	2706

A.9.2.43.3	Test Requirements	2709

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2709

A.9.2.44.1	Test Purpose and Environment	2709

A.9.2.44.2	Test parameters	2709

A.9.2.44.3	Test Requirements	2711

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2711

A.9.2.45.1	Test Purpose and Environment	2711

A.9.2.45.2	Test parameters	2711

A.9.2.45.3	Test Requirements	2715

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2716

A.9.2.46.1	Test Purpose and Environment	2716

A.9.2.46.2	Test parameters	2716

A.9.2.46.3	Test Requirements	2720

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2721

A.9.2.47.1	Test Purpose and Environment	2721

A.9.2.47.2	Test parameters	2721

A.9.2.47.3	Test Requirements	2725

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2725

A.9.2.48.1	Test Purpose and Environment	2725

A.9.2.48.2	Test parameters	2726

A.9.2.48.3	Test Requirements	2730

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2731

A.9.2.49.1	Test Purpose and Environment	2731

A.9.2.49.2	Test parameters	2731

A.9.2.49.3	Test Requirements	2737

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2738

A.9.2.50.1	Test Purpose and Environment	2738

A.9.2.50.2	Test parameters	2738

A.9.2.50.3	Test Requirements	2744

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2744

A.9.2.51.1	Test Purpose and Environment	2744

A.9.2.51.2	Test parameters	2745

A.9.2.51.3	Test Requirements	2749

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2749

A.9.2.52.1	Test Purpose and Environment	2749

A.9.2.52.2	Test parameters	2749

A.9.2.52.3	Test Requirements	2752

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2753

A.9.2.53.1	Test Purpose and Environment	2753

A.9.2.53.2	Test parameters	2753

A.9.2.53.3	Test Requirements	2759

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2760

A.9.2.54.1	Test Purpose and Environment	2760

A.9.2.54.2	Test parameters	2760

A.9.2.54.3	Test Requirements	2766

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2766

A.9.2.55.1	Test Purpose and Environment	2766

A.9.2.55.2	Test parameters	2767

A.9.2.55.3	Test Requirements	2772

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2772

A.9.2.56.1	Test Purpose and Environment	2772

A.9.2.56.2	Test parameters	2772

A.9.2.56.3	Test Requirements	2777

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2777

A.9.2.57.1	Test Purpose and Environment	2777

A.9.2.57.2	Test parameters	2778

A.9.2.57.3	Test Requirements	2782

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2783

A.9.2.58.1	Test Purpose and Environment	2783

A.9.2.58.2	Test parameters	2783

A.9.2.58.3	Test Requirements	2787

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2788

A.9.2.59.1	Test Purpose and Environment	2788

A.9.2.59.2	Test parameters	2788

A.9.2.59.3	Test Requirements	2793

A.9.3	UTRAN FDD CPICH RSCP	2794

A.9.3.1	E-UTRAN FDD	2794

A.9.3.1.1	Test Purpose and Environment	2794

A.9.3.1.2	Parameters	2794

A.9.3.1.3	Test Requirements	2796

A.9.3.2 E-UTRAN TDD	2796

A.9.3.2.1	Test Purpose and Environment	2796

A.9.3.2.2	Parameters	2796

A.9.3.2.3	Test Requirements	2799

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2799

A.9.3.3.1	Test Purpose and Environment	2799

A.9.3.3.2	Parameters	2799

A.9.3.3.3	Test Requirements	2800

A.9.4	UTRAN FDD CPICH Ec/No	2801

A.9.4.1	E-UTRAN FDD	2801

A.9.4.1.1	Test Purpose and Environment	2801

A.9.4.1.2	Parameters	2801

A.9.4.1.3	Test Requirements	2803

A.9.4.2	E-UTRAN TDD	2804

A.9.4.2.1	Test Purpose and Environment	2804

A.9.4.2.2	Parameters	2804

A.9.4.2.3	Test Requirements	2806

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2807

A.9.4.3.1	Test Purpose and Environment	2807

A.9.4.3.2	Parameters	2807

A.9.4.3.3	Test Requirements	2808

A.9.5	UTRAN TDD measurement	2808

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2808

A.9.5.1.1	Test Purpose and Environment	2808

A.9.5.1.2	Test parameters	2808

A.9.5.1.3	Test Requirements	2810

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2810

A.9.5.2.1	Test Purpose and Environment	2810

A.9.5.2.2	Test parameters	2810

A.9.5.2.3	Test Requirements	2812

A.9.6	GSM Carrier RSSI	2813

A.9.6.1	E-UTRAN FDD	2813

A.9.6.1.1	Test Purpose and Environment	2813

A.9.6.1.2	Test Requirements	2814

A.9.6.2	E-UTRAN TDD	2814

A.9.6.2.1	Test Purpose and Environment	2814

A.9.6.2.2	Test Requirements	2816

A.9.7	UE Rx – Tx Time Difference	2817

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2817

A.9.7.1.1	Test Purpose and Environment	2817

A.9.7.1.2	Test parameters	2817

A.9.7.1.3	Test Requirements	2818

A.9.7.2	E-UTRA TDD	2818

A.9.7.2.1	Test Purpose and Environment	2818

A.9.7.2.2	Test parameters	2818

A.9.7.2.3	Test Requirements	2820

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2820

A.9.7.3.1	Test Purpose and Environment	2820

A.9.7.3.2	Test parameters	2820

A.9.7.3.3	Test Requirements	2823

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2823

A.9.7.4.1	Test Purpose and Environment	2823

A.9.7.4.2	Test Parameters	2823

A.9.7.4.3	Test Requirements	2826

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2826

A.9.7.5.1	Test Purpose and Environment	2826

A.9.7.5.2	Test parameters	2826

A.9.7.5.3	Test Requirements	2829

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2829

A.9.7.6.1	Test Purpose and Environment	2829

A.9.7.6.2	Test Parameters	2829

A.9.7.6.3	Test Requirements	2832

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2832

A.9.7.7.1	Test Purpose and Environment	2832

A.9.7.7.2	Test parameters	2832

A.9.7.7.3	Test Requirements	2833

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2834

A.9.7.8.1	Test Purpose and Environment	2834

A.9.7.8.2	Test parameters	2834

A.9.7.8.3	Test Requirements	2835

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2835

A.9.7.9.1	Test Purpose and Environment	2835

A.9.7.9.2	Test parameters	2835

A.9.7.9.3	Test Requirements	2836

A.9.8	RSTD	2837

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2837

A.9.8.1.1	Test Purpose and Environment	2837

A.9.8.1.2	Test Requirements	2839

A.9.8.1.2A	Test Requirements for UE Category 1bis	2839

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2840

A.9.8.2.1	Test Purpose and Environment	2840

A.9.8.2.2	Test Requirements	2843

A.9.8.2.2A	Test Requirements for UE Category 1bis	2843

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2844

A.9.8.3.1	Test Purpose and Environment	2844

A.9.8.3.2	Test Requirements	2846

A.9.8.3.2A	Test Requirements for UE Category 1bis	2846

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2847

A.9.8.4.1	Test Purpose and Environment	2847

A.9.8.4.2	Test Requirements	2849

A.9.8.4.2A	Test Requirements for UE Category 1bis	2849

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2850

A.9.8.5.1	Test Purpose and Environment	2850

A.9.8.5.2	Test Requirements	2852

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2852

A.9.8.6.1	Test Purpose and Environment	2852

A.9.8.6.2	Test Requirements	2855

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2855

A.9.8.7.1	Test Purpose and Environment	2855

A.9.8.7.2	Test Requirements	2856

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2856

A.9.8.8.1	Test Purpose and Environment	2856

A.9.8.8.2	Test Requirements	2857

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2857

A.9.8.9.1	Test Purpose and Environment	2857

A.9.8.9.2	Test Requirements	2858

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2858

A.9.8.10.1	Test Purpose and Environment	2858

A.9.8.10.2	Test Requirements	2859

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2859

A.9.8.11.1	Test Purpose and Environment	2859

A.9.8.11.2	Test Requirements	2860

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2860

A.9.8.12.1	Test Purpose and Environment	2860

A.9.8.12.2	Test Requirements	2861

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2861

A.9.8.13.1	Test Purpose and Environment	2861

A.9.8.13.2	Test Requirements	2862

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2862

A.9.8.14.1	Test Purpose and Environment	2862

A.9.8.14.2	Test Requirements	2869

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2869

A.9.8.15.1	Test Purpose and Environment	2869

A.9.8.15.2	Test Requirements	2875

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2875

A.9.8.16.1	Test Purpose and Environment	2875

A.9.8.16.2	Test Requirements	2878

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2878

A.9.8.17.1	Test Purpose and Environment	2878

A.9.8.17.2	Test Requirements	2881

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2881

A.9.8.18.1	Test Purpose and Environment	2881

A.9.8.18.2	Test Requirements	2884

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2884

A.9.8.19.1	Test Purpose and Environment	2884

A.9.8.19.2	Test Requirements	2887

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2887

A.9.8.20.1	Test Purpose and Environment	2887

A.9.8.20.2	Test Requirements	2890

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2890

A.9.8.21.1	Test Purpose and Environment	2890

A.9.8.21.2	Test Requirements	2894

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2895

A.9.8.22.1	Test Purpose and Environment	2895

A.9.8.22.2	Test Requirements	2898

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2899

A.9.8.23.1	Test Purpose and Environment	2899

A.9.8.23.2	Test Requirements	2902

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2902

A.9.8.24.1	Test Purpose and Environment	2902

A.9.8.24.2	Test Requirements	2906

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2906

A.9.8.25.1	Test Purpose and Environment	2906

A.9.8.25.2	Test Requirements	2910

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2911

A.9.8.26.1	Test Purpose and Environment	2911

A.9.8.26.2	Test Requirements	2913

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2913

A.9.8.27.1	Test Purpose and Environment	2913

A.9.8.27.2	Test Requirements	2916

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2916

A.9.8.28.1	Test Purpose and Environment	2916

A.9.8.28.2	Test Requirements	2919

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2919

A.9.8.29.1	Test Purpose and Environment	2919

A.9.8.29.2	Test Requirements	2923

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2924

A.9.8.30.1	Test Purpose and Environment	2924

A.9.8.30.2	Test Requirements	2927

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2928

A.9.8.31.1	Test Purpose and Environment	2928

A.9.8.31.2	Test Requirements	2932

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2932

A.9.8.32.1	Test Purpose and Environment	2932

A.9.8.32.2	Test Requirements	2935

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2935

A.9.8.33.1	Test Purpose and Environment	2935

A.9.8.33.2	Test Requirements	2938

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2938

A.9.8.34.1	Test Purpose and Environment	2938

A.9.8.34.2	Test Requirements	2941

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2941

A.9.8.35.1	Test Purpose and Environment	2941

A.9.8.35.2	Test Requirements	2944

A.9.9	RSRP and RSRQ on the serving cell	2944

A.9.9.1	FDD Intra frequency serving cell case	2944

A.9.9.1.1	Test Purpose and Environment	2944

A.9.9.1.2	Test parameters	2944

A.9.9.1.3	Test Requirements	2947

A.9.9.2	TDD Intra frequency serving cell case	2947

A.9.9.2.1	Test Purpose and Environment	2947

A.9.9.2.2	Test parameters	2947

A.9.9.2.3	Test Requirements	2949

A.9.10	SSTD	2949

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2949

A.9.10.1.1	Test Purpose and Environment	2949

A.9.10.1.2	Test parameters	2949

A.9.10.1.3	Test Requirements	2951

A.9.10.2	Void	2951

A.9.10.3	Void	2951

A.9.10.4	Void	2951

A.9.11 RSSI	2951

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2951

A.9.11.1.1	Test Purpose and Environment	2951

A.9.11.1.2	Test parameters	2951

A.9.11.1.3	Test Requirements	2954

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2954

A.9.11.2.1	Test Purpose and Environment	2954

A.9.11.2.2	Test parameters	2954

A.9.12	Channel occupancy	2957

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2957

A.9.12.1.1	Test Purpose and Environment	2957

A.9.12.1.2	Test parameters	2957

A.9.12.1.3	Test Requirements	2960

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2960

A.9.12.2.1	Test Purpose and Environment	2960

A.9.12.2.2	Test parameters	2960

A.9.12.2.3	Test Requirements	2963

A.9.13	RS-SINR	2963

A.9.13.1	FDD Intra-Frequency Case	2963

A.9.13.1.1	Test Purpose and Environment	2963

A.9.13.1.2	Test parameters	2963

A.9.13.1.3	Test Requirements	2966

A.9.13.2	TDD Intra-Frequency Case	2966

A.9.13.2.1	Test Purpose and Environment	2966

A.9.13.2.2	Test parameters	2966

A.9.13.2.3	Test Requirements	2969

A.9.13.3	FDD—FDD Inter frequency case	2969

A.9.13.3.1	Test Purpose and Environment	2969

A.9.13.3.2	Test parameters	2969

A.9.13.3.3	Test Requirements	2972

A.9.13.4	TDD—TDD Inter frequency case	2972

A.9.13.4.1	Test Purpose and Environment	2972

A.9.13.4.2	Test parameters	2972

A.9.13.4.3	Test Requirements	2976

A.9.13.5	FDD—TDD Inter frequency case	2977

A.9.13.5.1	Test Purpose and Environment	2977

A.9.13.5.2	Test parameters	2977

A.9.13.5.3	Test Requirements	2983

A.9.13.6	TDD—FDD Inter frequency case	2983

A.9.13.6.1	Test Purpose and Environment	2983

A.9.13.6.2	Test parameters	2983

A.9.13.6.3	Test Requirements	2987

A.9.14	Channel quality reporting accuracy	2987

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2987

A.9.14.1.1	Test Purpose and Environment	2987

A.9.14.1.2	Test parameters	2988

A.9.14.1.3	Test Requirements	2988

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2988

A.9.14.2.1	Test Purpose and Environment	2988

A.9.14.2.2	Test parameters	2989

A.9.14.2.3	Test Requirements	2989

A.10	Proximity-based Services in Any Cell Selection State	2990

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2990

A.10.1.1	Test Purpose and Environment	2990

A.10.1.2	Test Requirements	2991

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2991

A.10.2.1	Test Purpose and Environment	2991

A.10.2.2	Test Requirements	2992

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2993

A.10.3.1	Test Purpose and Environment	2993

A.10.3.2	Test Requirements	2995

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2996

A.10.4.1	Test Purpose and Environment	2996

A.10.4.2	Test Requirements	2998

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2998

A.11.1	V2V UE Transmission Timing Accuracy Test	2998

A.11.1.1	Test Purpose and Environment	2998

A.11.1.2	Test requirements	2999

A.11.2	Interruptions due to V2V sidelink communication	2999

A.11.2.1	Test Purpose and Environment	2999

A.11.2.2	Test Requirements	3001

A.12	3001

A.12.1	V2X UE Transmission Timing Accuracy Test	3001

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3001

A.12.1.1.1	Test Purpose and Environment	3001

A.12.1.1.2	Test requirements	3002

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3003

A.12.1.2.1	Test Purpose and Environment	3003

A.12.1.2.2	Test Requirements	3003

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3004

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3004

A.12.2.1.1	Test Purpose and Environment	3004

A.12.2.1.2	Test Requirements	3005

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3006

A.12.2.2.1	Test Purpose and Environment	3006

A.12.2.2.2	Test Requirements	3007

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3007

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3007

A.12.3.1.1	Test Purpose and Environment	3007

A.12.3.1.2	Test Requirements	3010

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3011

A.12.3.2.1	Test Purpose and Environment	3011

A.12.3.1.2	Test Requirements	3013

A.12.4	Congestion Control Measurement Test for V2X UE	3013

A.12.4.1	Test Purpose and Environment	3013

A.12.4.2	Test Requirements	3015

A.12.5	Interruptions due to V2X Sidelink Communication	3015

A.12.5.1	Test Purpose and Environment	3015

A.12.5.1	Test Requirements	3017

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3017

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3017

A.12.6.1.1	Test Purpose and Environment	3017

A.12.6.1.2	Test Requirements	3019

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3019

A.12.6.2.1	Test Purpose and Environment	3019

A.12.6.1.2	Test Requirements	3021

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3021

A.12.7.1	Test Purpose and Environment	3021

A.12.7.2	Test Requirements	3024

A.12.8	Interruptions due to V2X Carrier Aggregation	3025

A.12.8.1	Interruptions on a FDD PCell	3025

A.12.8.1.1	Test Purpose and Environment	3025

A.12.8.1.2	Test Requirements	3026

A.12.8.2	Interruptions on a TDD PCell	3026

A.12.8.2.1	Test Purpose and Environment	3026

A.12.8.2.2	Test Requirements	3028

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	3029

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3029

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3029

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3029

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3029

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3031

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3032

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3032

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3032

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3032

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3034

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3035

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3035

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3035

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3036

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3036

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3037

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3037

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3037

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3038

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3038

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3039

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3039

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3039

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3039

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3040

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3040

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3040

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3041

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3041

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3041

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3042

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3042

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3043

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3044

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3045

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3046

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3047

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3047

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3047

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3047

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3048

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3048

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3050

B.3	Conditions for measurements performance requirements for UE	3050

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3050

B.3.2	Void	3050

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3050

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3050

B.3.5	Conditions for UE Rx – Tx time difference	3050

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3051

B.3.7	Conditions for inter-frequency RSTD measurements	3051

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3051

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3051

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3051

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3052

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3052

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3052

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3052

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3052

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3052

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3053

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3053

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3053

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3054

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3054

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3054

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3054

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3054

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3054

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3054

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3054

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3054

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3055

B.3.21.1	Conditions for RSRP measurements	3055

B.3.21.2	Conditions for RSRQ measurements	3055

B.3.21.3	Conditions for CSI-RSRP measurements	3055

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3055

B.3.22.1	Conditions for RSRP measurements	3055

B.3.22.2	Void	3056

B.3.22.3	Conditions for CSI-RSRP measurements	3056

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3056

B.3.23.1	Conditions for RSRP measurements	3056

B.3.23.2	Conditions for RSRQ measurements	3056

B.3.23.3	Conditions for CSI-RSRP measurements	3056

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3057

B.3.24.1	Conditions for RSRP measurements	3057

B.3.24.2	Conditions for RSRQ measurements	3057

B.3.24.3	Conditions for CSI-RSRP measurements	3057

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3057

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3057

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3057

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3058

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3058

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3058

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3058

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3058

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3059

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3059

B.4	RRM Requirements Exceptions	3059

B.4.1	General	3059

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3059

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3059

B.4.3.1	Inter-band carrier aggregation	3059

B.4.3.2	Intra-band non-contiguous carrier aggregation	3059

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3060

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3060

B.5.1	Conditions for S-RSRP Accuracy Requirements	3060

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3060

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3061

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3061

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3062

B.6	Conditions for V2X	3062

B.6.1	Test parameters for GNSS signals	3062

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3062

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3062

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3063

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3063

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3063

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3063

Annex C (informative):	Change history:	3065