3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200016

Online Meeting, 17 January – 25 January 2022

# 3GPP TSG-RAN WG2 Meeting # 116-E 	R2-2111465

Electronic Meeting, November, 2021

Title:	Reply LS on signalling for intra-band CA with UL-MIMO

Response to:	LS on signalling for intra-band CA with UL-MIMO (R4-2114754)

Release:	Rel-17

Work Item:	NR_RF_FR1_enh

Source:	TSG RAN WG2

To:	TSG RAN WG4

Cc:

Contact Person:

Name: Qianxi Lu

E-mail Address: qianxi.lu@OPPO.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN4 for the LS R4-2114754, RAN2 has discussed the questions in the LS, and the answers are provided as follows.

For the first question

- Whether current signalling can support UE’s different CA bandwidth class capabilities or different frequency separation class capabilities for UL CA with and without UL MIMO for the same band?

For example:

  - when maxNumberMIMO-LayersCB-PUSCH = 1, the UE can support CA_ n77C;

  - when maxNumberMIMO-LayersCB-PUSCH = 2, the UE can support CA_ n77B;

Answer: Yes, the current signalling support it via reporting in two different band-combination entries (e.g., maxNumberMIMO-LayersCB-PUSCH = 1 and CA_ n77C in one band-combination entry, and maxNumberMIMO-LayersCB-PUSCH = 2 and CA_ n77B in another band-combination entry).

For the second question

- Is UE allowed to indicate the capabilities separately in two different band combination entries?

  - If yes, whether gNB take account on the different capabilities across different Band combination entries?

If the answer to the above questions is no, RAN4 respectfully ask RAN2 to consider designing new signalling to differentiate such different UE capabilities in UL CA with and without UL MIMO.

Answer: As usual, the network takes UE capability into account and it is up to network implementation on exact reconfiguration. It is possible that the network may configure the UE according to a band combination entry, and it may later reconfigure the UE according to another band combination entry.

2. Actions:

To RAN4:

ACTION: RAN2 respectfully asks RAN4 to take the answer above into account.

3. Date of Next TSG WG RAN2 Meetings:

TSG-RAN WG2 Meeting #116bis-e	17 - 25 Jan 2022		E-meeting

TSG-RAN WG2 Meeting #117bis-e	21 Feb - 03 Mar 2022	E-meeting