3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200102

Electronic Meeting, January 17-25, 2022

Agenda item:	6.9.4.5

Source: 	CATT

Title: 	Discussion on RLM/BFD requirement for FR2 HST

Document for:	Discussion

Introduction

There were extensive discussions for FR2 HST during the past RAN4 meetings and a WF on RRM requirements for NR FR2 HST was approved [1] in RAN4#101-e meeting.

In this contribution, we provide our consideration of open issues for RLM/BFD requirements and give our proposal.

Discussion

In RAN4#101-e meeting, two options are listed for RLM/BFD in WF [1] as below:

For RLM/BFD requirements - The existing 1280ms duration for known condition is applied for FR2 HST scenario - RAN4 will further study the RLM/BFD requirements for DRX <=80ms for FR2 HST scenarios - Option 1: following RX beam factor for set-1 and set-2 - Option 2: The existing RLM/BFD requirements for DRX <=80ms is applied

In current RLM requirements, evaluation periods are specified with N=8 for SSB based RLM:

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-2 for FR2 with scaling factor N=8.

Table 8.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P N) TSSB) | Max(100, Ceil(5 P N) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 P N) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 P N) TDRX | Ceil(5 P N) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

For SSB-based BFD requirements, it is the similar and shown as:

The value of TEvaluate_BFD_SSB is defined in Table 8.5.2.2-2 for FR2 with scaling factor N=8

Table 8.5.2.2-2: Evaluation period TEvaluate_BFD_SSB for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 P N) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

The evaluation period in FR2 is scaled by UE RX beam number. In last meeting, it is agreed to have only two sets for FR2 HST. It is included in WF [2]:

GtW Agreements: - Define only two sets of enhanced RRM requirements in terms of number of RX beams (i.e. RX beam sweeping scaling factor) per UE - Set 1: 2 RX beams - Set 2: 6 RX beams - Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements - Note: the applicability of Set 1/2 requirements to the FR2 HST scenarios will be captured in the TR

HST scenarios are different from regular deployments. We discussed in many meetings about the scenarios and conclude to Scenario-A and Scenario-B and in those scenarios, the number of RX beam sweeping scaling factors can be reduced to 2 and 6 for Set 1 and Set 2. It is unnecessary to keep N=8 as non-HST scenarios to extend the evaluation period.

Proposal 1: For RLM/BFD requirements for DRX <=80ms for FR2 HST scenarios, use agreed RX beam factor for set-1 and set-2.

Conclusion

This document presented our consideration of open issues for RLM/BFD requirements for FR2 HST. The following proposals are concluded as below:

Proposal 1: For RLM/BFD requirements for DRX <=80ms for FR2 HST scenarios, use agreed RX beam factor for set-1 and set-2.

Reference

[1] R4-2120416, WF on FR2 HST RRM requirements (part 2), Samsung

[2] R4-2120292, WF on FR2 HST RRM (part 1), Nokia, Nokia Shanghai Bell