3GPP TSG-RAN WG4 Meeting #101-bis-e                                                                                 R4-2200136

Electronic Meeting, January 17 - 25, 2022

Source: 	CATT

Title: 	Discussion on the BS TX RF requirements for 52.6-71GHz

Agenda item:	6.16.4.1

Document for:	Approval

- Introduction

In RAN4#101-e, a WF [1] on BS RF TX requirements for 52.6 – 71 GHz was approved. TAE, ΔfOBUE, and step frequencies for spurious emissions need to be further discussed

In this contribution, we provide our views on the remaining issues in the WF.

- Discussion

TAE

As per the WF [1], the remaining issues concerning TAE are shown as below:

For MIMO case: “Scale the value under 120kHz for larger SCS cases” as starting point. For CA cases: FFS.

Figure 2-1: The impact of TAE on throughput for 480kHz SCS(left) and 960 kHz SCS(right),

The simulation results with different TAEs for two layers transmission without HARQ are shown in Figure 2-1.  And the link level simulation assumptions for TAE are shown in Table A5-1 in annex. From Figure 2-1, it is observed that,

  - For 10ns TAE for 960kHz SCS, the throughput can reach 70% of maximum throughput.

  - For 20ns, 35ns and 65ns TAE for 960kHz SCS, the throughput is difficult to reach 70% of maximum throughput. Especially for 35ns and 65ns TAE, throughput @ 35dB SNR is close to 0.

  - For 10ns, 35ns, and 65ns TAE for 480kHz SCS, the throughput can reach 70% of maximum throughput. For 70% metric, the SNR of 65ns is about 5dB higher than that of 0ns, and the SNR of 35ns is about 2.5dB higher than that of 0ns.

From the above simulation results, the 20ns, 35ns and 65ns TAEs cause obvious performance degradation for DL two layers transmission for 960kHz SCS. And 35ns and 65ns TAE cause slightly performance degradation for DL two layers transmission for 480kHz SCS compared to 960kHz.

Proposal 1: For MIMO TAE，to specify [10ns] for 906kHz SCS TAE and a value in range of [10ns-35ns] for 480kHz SCS TAE.

OBUE

As per the WF [1], the remaining issues concerning OBUE are shown as below:

[Re-use FR2-1 OBUE], deltafOBUE needs more discussion but tentatively in range [3 – 4 GHz] Make decision on deltafOBUE on Jan 2022 RAN4 meeting.

In ETSI EN 302 567 V2.2.0[4] and Draft ETSI EN 303 722 V0.0.11 (2021-06)[5], the boundary between the spurious domain and the out-of-band domain for 60GHz is defined as min(2,5 × nominal channel BW, 1,5 × nominal channel BW + 500 MHz), so the ΔfOBUE for 52.6GHz can be defined as 1.5*BWcontiguous + 500 MHz = 3500MHz. Therefore, we propose to specify ΔfOBUE = 3500 MHz.

Proposal 2: To specifyΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz.

Spurious emissions

As per the WF [1], the remaining issues concerning spurious emissions are shown as below:

For licensed operation and unlicensed operation, RAN4 should re-use the FR2 approach and use FR2 spurious emission requirements for NR in 52.6 to 71 GHz and make adaptations with respect Fstep,X taking to account larger carrier bandwidths.

Figure 2-3: FR2 approach spurious emission mask for frequency >=18GHz[6].

For option 1, the FR2 approach for frequency >=18GHz is referenced to Figure 7 in ERC Recommendation 74-01[6], the spurious emission mask is shown in Figure 2-3.

According to Table 9 and Figure 7 in Annex 2 of ERC Recommendation 74-01, the step frequencies can be calculated by the following equation.

WB = FDL,high-FDL,low

Fstep,1 = max(18, FDL,low-max(10,4*WB))

Fstep,2 = FDL,low-max(2,WB)

Fstep,3 = FDL,low-ΔfOBUE

Fstep,4 =FDL,high+ΔfOBUE

Fstep,5 =FDL,high+max(2,WB)

Fstep,6 =FDL,high+max(10,4*WB)

Using above equations for step frequencies, the step frequencies for 66-71GHz, and 57-71GHz with proposedΔfOBUE = 3500 MHz are show in Table 2-3.

Table 2-3: Step frequencies for 66-71GHz, and 57-71GHz with proposedΔfOBUE = 3500 MHz

| Operating band | Fstep,1 | Fstep,2 | Fstep,3 | FDL,low | FDL,high | fOBUE | WB | Fstep,4 | Fstep,5 | Fstep,6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) |
| n257 | 18 | 23.5 | 25 | 26.5 | 29.5 | 1.5 | 3 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 24.25 | 27.5 | 1.5 | 3.25 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 39.5 | 43.5 | 1.5 | 4 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 37 | 40 | 1.5 | 3 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26 | 27.5 | 28.35 | 1.5 | 0.85 | 29.85 | 30.35 | 38.35 |
| nxxx (lic) | 46 | 61 | 62.5 | 66 | 71 | 3.5 | 5 | 74.5 | 76 | 91 |
| n263 | 18 | 43 | 53.5 | 57 | 71 | 3.5 | 14 | 74.5 | 85 | 127 |

Proposal 3: To specify the following Fstep,X for spurious emissions.

| Operating band | Fstep,1 | Fstep,2 | Fstep,3 | Fstep,4 | Fstep,5 | Fstep,6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |

- Conclusion

This contribution provides analysis on BS TX RF requirements for 52.6-71GHz. The following proposals and observations are concluded as follows:

Proposal 1: For MIMO TAE，to specify [10ns] for 906kHz SCS TAE and a value in range of [10ns-35ns] for 480kHz SCS TAE.

Proposal 2: To specifyΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz.

Proposal 3: To specify the following Fstep,X for spurious emissions.

| Operating band | Fstep,1 | Fstep,2 | Fstep,3 | Fstep,4 | Fstep,5 | Fstep,6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |

- References

- R4-2120677, WF on BS RF Tx requirements, Nokia, RAN4#101-e

- Recommendation ITU-R M.2003-2, “Multiple Gigabit Wireless Systems in frequencies around 60 GHz”, ITU-R

- ERC Recommendation 70-03, “Relating to the use of Short Range Devices (SRD)”, CEPT.

- Final draft ETSI EN 302 567 V2.2.1 (2021-05),Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum,ETSI

- Draft ETSI EN 303 722 V0.0.11 (2021-06),Wideband Data Transmission System (WDTS) for Fixed Network Radio Equipment operating in the 57 GHz to 71 GHz band; Harmonised Standard for access to radio spectrum, ETSI

- ERC Recommendation 74-01, unwanted emissions in the spurious domain, ECC

- Annex

The link level simulation assumptions for TAE are shown in Table A5-1.

Table A5-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 60GHz |
| CBW | 400MHz |
| SCS | 480kHz, 960kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, Maximum Doppler frequency: 5Hz |
| MCS | MCS 16 in 64QAM table of TS 38.214 |
| Pre-coding | Fixed PMI |
| Symbol type | CP-OFDM |
| HARQ | NO |
| RANK | 2 |
| BS antenna configuration | 2 |
| UE antenna configuration | 2 |
| Antenna correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 0, Duration 14 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | NO |
| PN model | NO |
| PN compensation | NO |