3GPP TSG-RAN WG4 Meeting # 101-bis-e      		R4-2200799

Electronic Meeting, January 17-25, 2022

Agenda Item:	6.12.2.1.2

Source: 	CMCC

Title: 	Discussion on CQI requirements for inter-cell interference MMSE-IRC

Document for:	Discussion

- Introduction

In RAN4#101-e meeting, CQI requirements for inter-cell interference MMSE-IRC receiver were discussed, a way forward was agreed in [1]. In this contribution, we provide our views on remaining issues.

- Discussion

  - Interference Model

NZP CSI-RS on target cell

Agreement in RAN4#101-e meeting:

- Overlaps with NZP CSI-RS from interference

In last meeting, we had agreed that NZP CSI-RS on serving cell overlaps with NZP CSI-RS on interference cell. However, some companies concern that this agreement will be invalid when interference cell is 1Tx and serving cell is 2Tx, because the port number of NZP CSI-RS between serving cell and interference cell is mismatched, we provide an example in Figure 1 and Figure 2.

| Figure 1. NZP CSI-RS on serving cell | Figure 2. NZP CSI-RS on interference cell |
| --- | --- |

However, in this test assumption working phase, 2Tx shouldn’t be precluded due to this agreement. If companies have strong concern about this mismatch issue, we can revisit the NZP CSI-RS conclusion.

We propose a modification for this agreement:

- If the Tx antenna of serving cell is 2Tx

  - One port of NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference, the other port of NZP CSI-RS on serving cell overlaps with PDSCH/OCNG from interference.

- If the Tx antenna of serving cell is 1Tx

  - NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference.

Proposal 1: 2Tx shouldn’t be precluded due to the agreement about NZP CSI-RS relation between target and interference cells.

Proposal 2: If modification for NZP CSI-RS agreement is needed, we propose to revisit as follows:

- If the Tx antenna of serving cell is 2Tx

  - One port of NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference, the other port of NZP CSI-RS on serving cell overlaps with PDSCH/OCNG from interference.

- If the Tx antenna of serving cell is 1Tx

  - NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference.

- Conclusion

In this contribution, considering of different Tx assumption for serving and interference cell, we provide our views on NZP CSI-RS configuration. Our proposals are:

Proposal 1: 2Tx shouldn’t be precluded due to the agreement about NZP CSI-RS relation between target and interference cells.

Proposal 2: If modification for NZP CSI-RS agreement is needed, we propose to revisit as follows:

- If the Tx antenna of serving cell is 2Tx

  - One port of NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference, the other port of NZP CSI-RS on serving cell overlaps with PDSCH/OCNG from interference.

- If the Tx antenna of serving cell is 1Tx

  - NZP CSI-RS on serving cell overlaps with NZP CSI-RS from interference.

- Reference

[1] R4-2120708, WF on CSI requirements for inter-cell interference MMSE-IRC, Ericsson.