3GPP RAN WG4 Meeting #101bis		R4-2200910

Electronic meeting, 17 – 26th January 2022

Agenda item:	5.1.2

Source:	Apple

Title:	On channel rasters for the 6GHz unlicensed band

WI/SI:	NR_6GHz_unlic_EU

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which was approved recently in different regulatory regions. Following the publication of the 6GHz regulatory decision for the EU/CEPT countries is November 2020, 3GPP RAN#90 meeting decided to resume the corresponding WI [2] that aims at adding 3GPP support for the 6GHz band in that regulatory region. As per 3GPP RAN plenary guidance made during the RAN#93 meeting, RAN WG4 was tasked to define a new band for the unlicensed operation in the lower 6GHz band. In addition to that, RAN#94 meeting has also clarified that a dedicated lower 6GHz unlicensed band should be defined as 5925-6425MHz.

It is worth noting that even though the RAN plenary recommended to define the new lower unlicensed band as 5925-6425MHz, the latest version of the endorsed CR has the table with channel ratsers does not include the raster for the first 20MHz channel in 5925-5945MHz. In this discussion paper we present our further technical considerations on this matter and suggest enabling first 20MHz.

# 2	Channel raster for the 6GHz unlicensed band

As already mentioned in the Introduction part, while the 3GPP TSG RAN recommended to define the new lower unlicensed band as 5925-6425MHz, the latest version of the draft CR for the lower unlicensed band has the list of channel raster points that start at 5945MHz. For instance, Nref 797000 (see the highlighted text below) corresponds to 5955MHz center frequency of the 20MHz channel at 5945-5965MHz; and similarly, Nref 797668 corresponds to the 5965MHz center frequency of the 40MHz channel at 5945-5985MHz. While this is aligned with the available channels in the EU/CEPT region, it will effectively prohibit using first 20MHz in other countries/regions, such as UK, Morocco, UAE, etc [3]. In fact, it is only EU/CEPT where this unlicensed band is limited to 5945-6425MHz, while in other countries it starts at 5925MHz. Furthermore, even Region 3 countries, such as Australia and New Zealand, define the starting point of this band at 5925MHz.

| n10217 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD13 |
| --- | --- | --- | --- |

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668 |

Thus, our view is that 3GPP can consider extending the list of the channel raster points so that NR-U can also utilise first 20MHz chunk of spectrum in 5925-5945MHz range (allowing not only 20MHz channel, but also 40 and 80MHz channels). And it bears mentioning that it will not conflict with the fact this channel cannot be used in EU/CEPT because the latter region will anyway have a dedicated NS flag explicitly limited to 5945-6425MHz in TS 38.101-1. In other words, the EU/CEPT NS flag will not be broadcast in 5925-5945MHz range.

Fnally, the same issue exists also for band n96 which also starts 5925MHz, but first channel raster point of which does not allow using first 20MHz. Thus, the same change can be done for both band n96 and n102.

Proposal 1:	Consider extending the list of channel raster points so that first 20MHz, i.e. 5925-5945MHz, can be used by NR-U.

Proposal 2:	The proposed change can be applied to both n102 and n96.

Proposal 3:	To avoid delaying Rel-17 specification work, the corresponding changes (to both band n96 and n102) can be considered in Rel-18.

# 3	Conclusions

In this contribution we have presented summary of several Region 1 countries that have opened the lower 6GHz band for the unlicensed operation.

Proposal 1:	Consider extending the list of channel raster points so that first 20MHz, i.e. 5925-5945MHz, can be used by NR-U.

Proposal 2:	The proposed change can be applied to both n102 and n96.

Proposal 3:	To avoid delaying Rel-17 specification work, the corresponding changes (to both band n96 and n102) can be considered in Rel-18.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP‐202592, "WID on introduction of lower 6GHz NR unlicensed operation for Europe", Nokia, Nokia Shanghai Bell

- R4-2200429, "Overview of the Region 1 countries implementing lower 6GHz unlicensed band", Apple