3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201552

Electronic Meeting, 17 January – 25 January 2022

Source:	Ericsson

Title:	TP for TR 38.717-03-01 to include CA_n41-n70-n78

Agenda Item:	5.9.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-01 to include CA_n41A-n70A-n78A.

# 2. Text Proposal

# ---Start of changes---

## 6.X	CA_n41-n70-n78

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41-n70-n78 | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n70A-n78A | - | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n70 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.X.3	Co-existence studies

For 3DL/1UL NR CA, only single uplink operation needs to be considered. For single uplink operation of this combination, only harmonic issue and harmonic mixing issue need to be considered.

Table 6.x.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. As can be seen there are 2nd harmonic issues from UL n70 affecting DL n78., which are addressed in lower order combinations.

Table 6.x.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n70 | 1695 | 1710 | 1995 | 2020 | 3390 | 3420 | 5085 | 5130 | 6780 | 6840 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

Table 6.x.3-2 gives harmonic mixing issue for the 3DL bands CA with 1 UL. No issues can be seen.

Table 6.x.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n70 | 1695 | 1710 | 1995 | 2020 | 3990 | 4040 | 5985 | 6060 | 7980 | 8080 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

    - 6.X.4	∆TIB and ∆RIB values

For three simultaneous DLs and one UL of Band n41, n70 and n78, the TIB,c and RIB,c  values are shown in table 6.1.x.4-1 and  table 6.1.x.4-2, respectively. Values are derived from CA_n3-n7-n78.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n70-n78 | n41 | 0.6 |
|  | n70 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n70-n78 | n41 | 0.2 |
|  | n70 | 0.2 |
|  | n78 | 0.5 |

### 6.X.5	MSD

The 2nd harmonic from n70 UL into n78 DL is addressed in lower order combination.

---End of changes---

# Reference

- RP-213299 TR 38.717-03-01, Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL,v0.4.0